Supplementary Information

Physical properties of the Y-hole core measured for each test piece are listed. Orientations of an arbitrary reference at individual distance from hole collar may be different from one another, since the core is not oriented.

| Table A1. Physical property of Y-hole core (UCS, Young’s modulus, Poisson’s ratio, Vp, Density) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Distance from hole collar, m | Azimuth from an arbitrary reference | Uniaxial compression strength, MPa | Young's modulus, GPa | Poisson's ratio | Vp, km/s* | Density, kg/m3** |
| 34 | 0 | 149.1 | 75.31 | 0.18 | 5.913 | 2691 |
| 34 | 0 | 162.7 | 75.91 | 0.11 | 5.954 | 2683 |
| 34 | 0 | 222.8 | 79.38 | 0.12 | 5.876 | 2679 |
| 34 | 45 | 150.0 | 74.23 | 0.17 | 5.915 | 2693 |
| 34 | 45 | 158.6 | 73.73 | 0.15 | 5.838 | 2683 |
| 34 | 45 | 192.8 | 75.44 | 0.17 | 5.838 | 2675 |
| 34 | 90 | 187.8 | 75.44 | 0.13 | 5.874 | 2688 |
| 34 | 90 | 201.9 | 76.73 | 0.13 | 5.957 | 2680 |
| 34 | 90 | 202.1 | 76.71 | 0.13 | 5.878 | 2678 |
| 34 | 135 | 177.3 | 73.26 | 0.15 | 5.838 | 2681 |
| 34 | 135 | 200.3 | 75.63 | 0.14 | 5.838 | 2679 |
| 34 | 135 | 193.4 | 71.62 | 0.085 | 5.836 | 2678 |
| 64 | 0 | 318.0 | 97.02 | 0.22 | 6.607 | 2992 |
| 64 | 0 | 186.7 | 87.62 | 0.18 | 6.200 | 2984 |
| 64 | 0 | 304.3 | 87.09 | 0.20 | 6.329 | 2990 |
| 64 | 45 | 290.6 | 102.80 | 0.25 | 6.512 | 2990 |
| 64 | 45 | 228.6 | 71.36 | 0.17 | 6.032 | 2988 |
| 64 | 45 | 320.6 | 71.85 | 0.16 | 6.074 | 2988 |
| 64 | 90 | 342.1 | 89.61 | 0.25 | 6.377 | 2977 |
| 64 | 90 | 319.1 | 87.35 | 0.18 | 6.241 | 2995 |
| 64 | 90 | 317.4 | 60.49 | 0.095 | 5.876 | 2995 |
| 64 | 135 | 272.1 | 80.30 | 0.16 | 6.198 | 2981 |
| 64 | 135 | 341.5 | 75.76 | 0.15 | 6.156 | 2990 |
| 64 | 135 | 337.5 | 74.74 | 0.16 | 5.993 | 3002 |
| 82 | 0 | 333.6 | 86.72 | 0.21 | 6.159 | 2991 |
| 82 | 0 | 352.6 | 87.75 | 0.20 | 6.284 | 3005 |
| 82 | 0 | 349.4 | 89.62 | 0.22 | 6.286 | 3006 |
| 82 | 45 | 341.9 | 76.71 | 0.17 | 5.997 | 2998 |
| 82 | 45 | 345.5 | 73.20 | 0.11 | 5.915 | 3001 |
| 82 | 45 | 337.4 | 81.51 | 0.18 | 5.995 | 2980 |
| 82 | 90 | 352.9 | 71.51 | 0.14 | 5.799 | 2999 |
| 82 | 90 | 353.8 | 59.13 | 0.12 | 5.876 | 3007 |
| 82 | 90 | 349.9 | 72.18 | 0.15 | 5.878 | 3005 |
| 82 | 135 | 343.9 | 84.53 | 0.17 | 6.032 | 3005 |
| 82 | 135 | 328.1 | 84.69 | 0.14 | 5.992 | 3003 |
| 82 | 135 | 338.7 | 90.81 | 0.20 | 6.115 | 3003 |
| *: P-wave speed was measured by a pulse transmission method. **: Density was calculated from mass and volume of test piece. |  |  |  |  |  |  |

| Table A2. Physical property of Y-hole core (UTS, Vp, Density) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Distance from hole collar, m | Azimuth from an arbitrary reference*** | Uniaxial tensile strength, MPa | Vp, km/s | Density, kg/m3 |
| 51 | along core axis | 18.42 |  | 2709 |
| 51 | along core axis | 18.61 |  | 2663 |
| 51 | 0 | 21.64 | 6.025 | 2664 |
| 51 | 0 | 20.18 | 6.024 | 2681 |
| 51 | 45 | 21.20 | 5.869 | 2680 |
| 51 | 45 | 18.09 | 5.740 | 2675 |
| 51 | 90 | 19.78 | 6.007 | 2670 |
| 51 | 90 | 14.13 | 5.973 | 2681 |
| 51 | 135 | 21.58 | 6.095 | 2690 |
| 51 | 135 | 22.87 | 5.911 | 2663 |
| 65 | along core axis | 21.79 |  | 3004 |
| 65 | along core axis | 24.74 |  | 3010 |
| 65 | along core axis | 19.88 |  | 3003 |
| 65 | 0 | 24.60 | 6.449 | 3014 |
| 65 | 0 | 18.99 | 6.447 | 2999 |
| 65 | 0 | 18.55 | 6.420 | 3012 |
| 65 | 45 | 20.22 | 6.503 | 2993 |
| 65 | 45 | 21.71 | 6.374 | 2980 |
| 65 | 45 | 21.81 | 6.675 | 3009 |
| 65 | 90 | 19.67 | 6.569 | 3017 |
| 65 | 90 | 19.79 | 6.549 | 3014 |
| 65 | 90 | 22.95 | 6.567 | 3020 |
| 65 | 135 | 21.77 | 6.386 | 3012 |
| 65 | 135 | 18.94 | 6.555 | 3006 |
| 86 | along core axis | 19.07 |  | 3002 |
| 86 | along core axis | 19.57 |  | 2999 |
| 86 | 0 | 14.41 | 6.401 | 3006 |
| 86 | 45 | 25.35 | 6.193 | 2980 |
| 86 | 90 | 21.63 | 6.688 | 3012 |
| 86 | 135 | 23.67 | 6.612 | 3012 |
| ***: Normal direction of a disk for Brazilian test. |  |  |  |  |