Electronic Supplementary Material 1:

Tables showing the analytical conditions for all analysed minerals on the electron microprobe.

Feldspar; 20 kV, 20 nA, 10 µm beam diameter

| Element | Line | Analysing crystal | Peak/background time (s) |
| --- | --- | --- | --- |
| Na2O | Kα | TAP | 30/15 |
| SiO2 | Kα | TAP | 16/8 |
| Al2O3 | Kα | TAP | 16/8 |
| SrO | Lα | PET | 30/15 |
| BaO | Lα | PET | 30/15 |
| PbO | Lα | LIF | 30/15 |
| FeO | Kα | LIF | 30/15 |
| CaO | Kα | PETJ | 16/8 |
| K2O | Kα | PETJ | 16/8 |

Garnet and cordierite; 15 kV, 20 nA, focussed beam

| Element | Line | Analysing crystal | Peak/background time (s) |
| --- | --- | --- | --- |
| Na2O | Kα | TAP | 30/15 |
| CaO | Kα | PET | 20/10 |
| K2O | Kα | PET | 16/8 |
| TiO2 | Kα | PET | 20/10 |
| Cr2O3 | Kα | PET | 16/8 |
| FeO | Kα | LIF | 10/5 |
| MnO | Kα | PET | 20/10 |
| MgO, SiO2, Al2O3 | Analysed with EDS only |  |  |

Gahnite; 20 kV, 20 nA, focussed beam

| Element | Line | Analysing crystal | Peak/background time (s) |
| --- | --- | --- | --- |
| NiO | Kα | LIF | 30/15 |
| Cr2O3 | Kα | PET | 30/15 |
| V2O3 | Kα | LIF | 30/15 |
| ZnO | Kα | LIF | 16/8 |
| Al2O3 | Kα | PET | 16/8 |
| MnO | Kα | LIF | 30/15 |
| SiO2 | Kα | TAP | 30/15 |
| MgO | Kα | TAP | 30/15 |
| TiO2 | Kα | PETJ | 30/15 |
| FeO | Kα | LIF | 16/8 |

Pyrrhotite, pyrite, chalcopyrite, alloys; 25 kV, 20 nA, focussed beam

| Element | Line | Analysing crystal | Peak/background time (s) |
| --- | --- | --- | --- |
| Se | Lα | TAP | 30/15 |
| S | Kα | PET | 16/8 |
| Fe | Kα | LIF | 16/8 |
| Sb | Lα | PET | 30/15 |
| Te | Lα | PET | 30/15 |
| Mn | Kα | PET | 30/15 |
| Ag | Lα | PET | 20/10 |
| Bi | Lα | LIF | 20/10 |
| Hg | Lα | LIF | 20/10 |
| Au | Lα | LIF | 30/15 |
| Ni | Kα | LIF | 30/15 |
| Co | Kα | LIF | 30/15 |
| Zn | Kα | LIF | 30/15 |
| Cu | Lβ | LIF | 20/10 |
| As | Kα | LIF | 30/15 |

Sphalerite, galena; 20 kV, 20 nA, focussed beam

| Element | Line | Analysing crystal | Peak/background time (s) |
| --- | --- | --- | --- |
| AS | Lα | TAP | 30/15 |
| S | Kα | PET | 16/8 |
| Fe | Kα | LIF | 16/8 |
| Pb | Mα | PET | 16/8 |
| Sb | Lα | PET | 30/15 |
| Tl | Mα | PET | 30/15 |
| Ag | Lα | PET | 30/15 |
| Cd | Lα | PET | 30/15 |
| Bi | Lα | LIF | 20/10 |
| Mn | Kα | LIF | 30/15 |
| Zn | Kα | LIF | 16/8 |
| Cu | Lα | LIF | 20/10 |