# Supplementary Information

Python Script for Thin Wall Deposition

import numpy as np

from stl import mesh

# Refined G/M/T Code Library for WAAM process with E1 and E2 for extruder selection

gcode_library = {

'G1': 'Linear move',

'M98': 'Call macro (e.g., weld torch on/off)',

'E1': 'Select extruder 1 (Wire Feeder 1)',

'E2': 'Select extruder 2 (Wire Feeder 2)',

'M0': 'End of program'

}

# Function to get the full G-code command from the library

def get_gcode_command(command, params=None):

if command in gcode_library:

gcode = command

if params:

gcode += ' ' + ' '.join([f'{k}{v}' for k, v in params.items()])

return gcode

else:

return f'; Unsupported command: {command}'  # Return comment for unsupported commands

# Function to calculate the bounding box (length, width, and height) from the STL file

def calculate_dimensions_from_stl(stl_file):

# Load the mesh

your_mesh = mesh.Mesh.from_file(stl_file)

# Extract the min and max X, Y, Z values to define the bounding box

min_x, max_x = np.min(your_mesh.x), np.max(your_mesh.x)

min_y, max_y = np.min(your_mesh.y), np.max(your_mesh.y)

min_z, max_z = np.min(your_mesh.z), np.max(your_mesh.z)

# Calculate the dimensions

length = max_x - min_x  # Length along the X-axis

width = max_y - min_y   # Width along the Y-axis

height = max_z - min_z  # Height along the Z-axis

return length, width, height

# Function to generate the G-code for WAAM process

def generate_gcode_waam(stl_file, output_gcode, extruder_choice, wire_feedrate, metal_diameter, feedrate):

"""

Generates G-code for WAAM process with torch moving along the X-axis for each layer.

Automatically extracts the length, width, and height from the STL file.

Parameters:

- stl_file: Path to the input STL file.

- output_gcode: Path to the output G-code file.

- extruder_choice: User's choice of extruder ('E1', 'E2', or 'both').

- wire_feedrate: The wire feed rate to use for each layer (mm/min).

- metal_diameter: Diameter of the liquid metal (controls Z height increments).

- feedrate: The torch travel speed (F parameter).

"""

# Extract the length, width, and height from the STL file

length, width, height = calculate_dimensions_from_stl(stl_file)

print(f"Extracted dimensions from STL: Length = {length} mm, Width = {width} mm, Height = {height} mm")

# Number of layers based on total height and metal diameter

num_layers = int(height / metal_diameter)

# Open the G-code file for writing

with open(output_gcode, 'w') as f:

# G-code header

f.write("G21 ; Set units to millimeters\n")  # G21 for millimeters

f.write("G90 ; Use absolute positioning\n")  # G90 for absolute positioning

# Initialize current Z height

current_z = 0.0

# Iterate over each layer

for layer in range(num_layers):

# Select extruder before starting layer

if extruder_choice == 'E1':

f.write(get_gcode_command('E1') + ' ; Select extruder 1 (Wire Feeder 1)\n')

elif extruder_choice == 'E2':

f.write(get_gcode_command('E2') + ' ; Select extruder 2 (Wire Feeder 2)\n')

elif extruder_choice == 'both':

f.write(get_gcode_command('E1') + ' ; Select extruder 1 (Wire Feeder 1)\n')

f.write(get_gcode_command('E2') + ' ; Select extruder 2 (Wire Feeder 2)\n')

# Move to start position (X0 Y0) without E and F

f.write(get_gcode_command('G1', {'X': 0, 'Y': 0, 'Z': current_z}) + ' ; Move to start position\n')

# Start of a new layer - Torch ON

f.write(get_gcode_command('M98', {'P': '"WELD TORCH ON"'}) + '\n')

# Move from X0 Y0 to X[length] Y0 with E and F for deposition

f.write(get_gcode_command('G1', {'X': length, 'Y': 0, 'Z': current_z, 'E': wire_feedrate, 'F': feedrate}) + '\n')

# End of the layer - Torch OFF

f.write(get_gcode_command('M98', {'P': '"WELD TORCH OFF"'}) + '\n')

# Increment the Z height for the next layer

current_z += metal_diameter

# End of the program with M0

f.write(get_gcode_command('M0') + ' ; End program\n')

print(f"G-code successfully generated in {output_gcode}")

# Example usage

stl_file = 'single_wall.stl'  # Replace with your STL file path

output_gcode = 'output_single_wall.gcode'  # Output G-code file name

# User inputs: Choose extruder, wire feedrate, metal diameter, and torch feed rate

extruder_choice = input("Select extruder configuration (E1, E2, or both): ").strip()

while extruder_choice not in ["E1", "E2", "both"]:

print("Invalid input. Please select E1, E2, or both.")

extruder_choice = input("Select extruder configuration (E1, E2, or both): ").strip()

try:

wire_feedrate = float(input("Enter wire feedrate (in mm/min): ").strip())

metal_diameter = float(input("Enter metal diameter (in mm): ").strip())

feedrate = float(input("Enter torch feedrate (F parameter, in mm/min): ").strip())

except ValueError:

print("Invalid input. Please enter valid numbers.")

exit(1)

# Generate the G-code

generate_gcode_waam(stl_file, output_gcode, extruder_choice, wire_feedrate, metal_diameter, feedrate)

print(f"G-code successfully generated in {output_gcode}")