Comprehensive in vitro analysis evaluating the variable drug-drug interaction risk of rifampicin compared to rifabutin

Julie Nilles1,2, Johanna Weiss1, Max Sauter1, Walter E. Haefeli1, Stephanie Ruez2, Dirk Theile1

1) Department of Clinical Pharmacology and Pharmacoepidemiology, Heidelberg University Hospital, Im Neuenheimer Feld 410, 69120 Heidelberg, Germany

2) Boehringer Ingelheim Pharma GmbH & Co. KG, Birkendorfer Str. 65, 88397 Biberach an der Riss, Germany.

Supplemental Material

Table S1: Validation results of the UPLC-MS/MS quantification method for rifampicin in LS180 cells.

| Rifampicin |  | LLOQ 0.100 ng/mL | Low QC 0.300 ng/mL | Mid QC 37.5 ng/mL | High QC 75.0 ng/mL |
| --- | --- | --- | --- | --- | --- |
| Within-batch |  |  |  |  |  |
| 1 | Mean [ng/mL] | 0.113 | 0.303 | 36.5 | 75.3 |
|  | Accuracy % | 113 | 101 | 97.3 | 101 |
|  | Precision % | 8.32 | 14.6 | 1.43 | 0.471 |
| 2 | Mean [ng/mL] | 0.105 | 0.298 | 37.9 | 78.7 |
|  | Accuracy % | 105 | 99.4 | 101 | 105 |
|  | Precision % | 4.76 | 3.58 | 1.09 | 1.96 |
| 3 | Mean [ng/mL] | 0.113 | 0.269 | 39.9 | 80.4 |
|  | Accuracy % | 113 | 89.7 | 106 | 107 |
|  | Precision % | 11.6 | 6.74 | 1.41 | 3.21 |
| Batch-to-batch |  |  |  |  |  |
|  | Mean [ng/mL] | 0.111 | 0.290 | 38.1 | 78.1 |
|  | Accuracy % | 111 | 96.7 | 102 | 104 |
|  | Precision % | 9.54 | 11.1 | 3.94 | 3.51 |

LLOQ: lower limit of quantification; QC: quality control.	Validation followed the applicable sections of the FDA and EMA recommendations. All experiments were performed with n = 6.

Table S2: Validation results of the UPLC-MS/MS quantification method for rifabutin in LS180 cells.

| Rifabutin |  | LLOQ 0.100 ng/mL | Low QC 0.300 ng/mL | Mid QC 37.5 ng/mL | High QC 75.0 ng/mL |
| --- | --- | --- | --- | --- | --- |
| Within-batch |  |  |  |  |  |
| 1 | Mean [ng/mL] | 0.115 | 0.310 | 35.7 | 73.4 |
|  | Accuracy [%] | 115 | 103 | 95.2 | 97.9 |
|  | Precision [%] | 13.0 | 3.72 | 1.19 | 0.632 |
| 2 | Mean [ng/mL] | 0.112 | 0.252 | 34.5 | 69.6 |
|  | Accuracy [%] | 112 | 83.9 | 92.0 | 92.8 |
|  | Precision [%] | 15.9 | 8.08 | 1.50 | 2.94 |
| 3 | Mean [ng/mL] | 0.113 | 0.334 | 33.9 | 66.5 |
|  | Accuracy [%] | 113 | 111 | 90.4 | 88.7 |
|  | Precision [%] | 23.5 | 8.08 | 8.56 | 6.30 |
| Batch-to-batch |  |  |  |  |  |
|  | Mean [ng/mL] | 0.113 | 0.299 | 35.0 | 67.6 |
|  | Accuracy [%] | 101 | 102 | 93.4 | 90.1 |
|  | Precision [%] | 9.61 | 6.07 | 5.32 | 5.26 |

LLOQ: lower limit of quantification; QC: quality control.	Validation followed the applicable sections of the FDA and EMA recommendations. All experiments were performed with n = 6.

Table S3: Validation results of the UPLC-MS/MS quantification method for 25-O-deacetylrifampicin in LS180 cells.

| 25-O-deacetylrifampicin |  | LLOQ 0.100 ng/mL | Low QC 0.300 ng/mL | Mid QC 37.5 ng/mL | High QC 75.0 ng/mL |
| --- | --- | --- | --- | --- | --- |
| Within-batch |  |  |  |  |  |
| 1 | Mean [ng/mL] | 0.103 | 0.293 | 38.2 | 79.2 |
|  | Accuracy [%] | 103 | 97.8 | 102 | 106 |
|  | Precision [%] | 7.21 | 1.61 | 4.53 | 2.22 |
| 2 | Mean [ng/mL] | 0.113 | 0.313 | 37.6 | 74.4 |
|  | Accuracy [%] | 113 | 104 | 100 | 99.2 |
|  | Precision [%] | 4.16 | 4.39 | 1.49 | 2.18 |
| 3 | Mean [ng/mL] | 0.0890 | 0.267 | 42.6 | 86.7 |
|  | Accuracy [%] | 89.0 | 89.0 | 114 | 116 |
|  | Precision [%] | 9.48 | 6.11 | 1.70 | 2.92 |
| Batch-to-batch |  |  |  |  |  |
|  | Mean [ng/mL] | 0.102 | 0.291 | 39.5 | 80.1 |
|  | Accuracy [%] | 102 | 97.1 | 105 | 107 |
|  | Precision [%] | 12.0 | 7.81 | 6.36 | 6.81 |

LLOQ: lower limit of quantification; QC: quality control.	Validation followed the applicable sections of the FDA and EMA recommendations. All experiments were performed with n = 6.

Table S4: Validation results of the UPLC-MS/MS quantification method for 25-O-deacetylrifabutin in LS180 cells.

| 25-O-deacetylrifabutin |  | LLOQ 0.100 ng/mL | Low QC 0.300 ng/mL | Mid QC 37.5 ng/mL | High QC 75.0 ng/mL |
| --- | --- | --- | --- | --- | --- |
| Within-batch |  |  |  |  |  |
| 1 | Mean [ng/mL] | 0.103 | 0.322 | 35.7 | 70.8 |
|  | Accuracy [%] | 103 | 107 | 95.2 | 94.4 |
|  | Precision [%] | 4.56 | 1.16 | 1.06 | 0.810 |
| 2 | Mean [ng/mL] | 0.110 | 0.302 | 34.1 | 64.9 |
|  | Accuracy [%] | 110 | 101 | 91.0 | 86.5 |
|  | Precision [%] | 5.25 | 6.47 | 1.82 | 2.82 |
| 3 | Mean [ng/mL] | 0.091 | 0.292 | 35.2 | 67.1 |
|  | Accuracy [%] | 90.7 | 97.4 | 93.9 | 89.5 |
|  | Precision [%] | 6.67 | 4.63 | 8.31 | 6.03 |
| Batch-to-batch |  |  |  |  |  |
|  | Mean [ng/mL] | 0.101 | 0.305 | 35.0 | 67.6 |
|  | Accuracy [%] | 101 | 102 | 93.4 | 90.1 |
|  | Precision [%] | 9.61 | 6.07 | 5.32 | 5.26 |

LLOQ: lower limit of quantification; QC: quality control.	Validation followed the applicable sections of the FDA and EMA recommendations. All experiments were performed with n = 6.

Table S5: EC50 [µM] and Emax values (± SD) of CYP3A4 and ABCB1 mRNA expression after exposure of LS180 cells to rifamycins for 24, 96, and 144 h.

|  | Gene | Time point [h] | EC50 ± SD [µM] | Emax ± SD |
| --- | --- | --- | --- | --- |
| Rifampicin | CYP3A4 | 24 | 7.3 ± 0.44 | 3.5 ± 0.13 |
|  |  | 24* | 32 ± 2.0 | 3.5 ± 0.13 |
|  |  | 96 | 3.2 ± 1.1 | 14 ± 1.3 |
|  |  | 144 | 4.8 ± 0.19 | 37 ± 2.8 |
|  | ABCB1 | 24 | 8.6 ± 0.47 | 2.6 ± 0.068 |
|  |  | 24* | 37 ± 2.0 | 2.6 ± 0.068 |
|  |  | 96 | 1.1 ± 0.056 | 5.1 ± 0.12 |
|  |  | 144 | 1.0 ± 0.065 | 7.0 ± 0.53 |
| Rifabutin | CYP3A4 | 24 | 1.2 ± 0.24 | 2.1 ± 0.094 |
|  |  | 24* | Not fitted |  |
|  |  | 96 | 3.6 ± 0.84 | 6.3 ± 0.46 |
|  |  | 144 | 2.0 ± 0.28 | 7.5 ± 0.82 |
|  | ABCB1 | 24 | 1.1 ± 0.027 | 1.4 ± 0.024 |
|  |  | 24* | Not fitted |  |
|  |  | 96 | 1.8 ± 1.1 | 2.8 ± 0.16 |
|  |  | 144 | 1.5 ± 0.72 | 4.5 ± 0.71 |

EC50: effective concentration, leading to 50 % of maximum effect [µM]; Emax (maximum effect) values are demonstrated as fold induction compared to untreated cells; 24* = values generated by using corresponding intracellular concentrations of rifamycins. Data was fitted to an Emax model (four parameter-logistic equation with variable slope) using GraphPad Prism version 9.1.

Table S6: Concentration-dependent induction of CYP3A4 activity after exposure for up to 96 h in LS180 cells and CYP3A4 inhibition by rifamycins.

|  | Time point [h] | EC50 ± SD [µM] | Emax ± SD | IC50 ± SD [µM] |
| --- | --- | --- | --- | --- |
| Rifampicin | 24 | 14 ± 4.2 | 3.5 ± 0.8 | 2.9 ± 0.85 |
|  | 24* | 43 ± 1.5 | 3.5 ± 0.8 |  |
|  | 72 | 13 ± 3.1 | 7.9 ± 1.6 |  |
|  | 96 | 3.8 ± 0.58 | 4.1 ± 0.12 |  |
| Rifabutin | 24 | Not fitted | Not fitted | 11 ± 2.9 |
|  | 24* |  |  |  |
|  | 72 |  |  |  |
|  | 96 |  |  |  |

EC50: effective concentration, leading to 50 % of maximum effect [µM]; Emax (maximum effect) values are demonstrated as fold induction compared to untreated cells; IC50: inhibitory concentration, leading to 50 % of inhibitory effect [µM]; 24* = values generated by using corresponding intracellular concentrations of the rifamycins. Data was fitted to an Emax model (four parameter-logistic equation with variable slope) using GraphPad Prism version 9.1.

Table S7: Concentration-dependent activation (LS180 cells) and inhibition of Pgp by rifamycins (P388/dx cells).

|  | Cell line | EC50/IC50 ± SD [µM] | Emax ± SD |
| --- | --- | --- | --- |
| Rifampicin | LS180 | 1.0 ± 0.0014 | 3.2 ± 0.37 |
|  | P388/dx | 13 ± 4.1 | 4.3 ± 0.53 |
| Rifabutin | LS180 | 1.1 ± 0.16 | 2.8 ± 0.48 |
|  | P388/dx | 0.32 ± 0.10 | 4.9 ± 1.3 |

EC50: effective concentration, leading to 50 % of maximum effect [µM]; Emax (maximum effect) values are demonstrated as fold induction compared to untreated cells; IC50: inhibitory concentration, leading to 50 % of inhibitory effect [µM]. Data was fitted to an Emax model (four parameter-logistic equation with variable slope) using GraphPad Prism version 9.1.