Supplementary Information

Evaluation of isotope-dilution mass spectrometry, internal standard, and external standard methods for PFAS quantification in tap water and soil

Eriko Yamazakia*, Nobuyasu Hanaria, Keisuke Nakamuraa

a Research Institute for Material and Chemical Measurement, National Metrology Institute of Japan, National Institute of Advanced Industrial Science and Technology (NMIJ/AIST), 1-1-1 Umezono, Tsukuba, Ibaraki 305-8563, Japan

*Corresponding author: Eriko Yamazaki

Research Institute for Material and Chemical Measurement,

National Metrology Institute of Japan, National Institute of Advanced Industrial Science and Technology (NMIJ/AIST),

1-1-1 Umezono, Tsukuba, Ibaraki 305-8563, Japan.

Phone: +81 50 3521 3452

E-mail: eriko.yamazaki@aist.go.jp

This supplementary information contains 5 pages, 5 tables, and 1 figure.

Table of Contents

Tables

Table S1. Mass-labeled standards used for IDMS and the internal standard method………..S3

Table S2. HPLC conditions…………….………………………………………....………….S3

Table S3. Mass transitions, Q1 pre-rod bias voltage (V), collision energy (eV), and Q3 pre-rod bias voltage (V) for individual compounds……….………….………………………….S4

Table S4. p-values of homoscedasticity tests…………………………………………...…S5

Table S5. p-values of lack-of-fit (LOF) tests……………………….…………….………S5

Figures

Fig. S1. Residual plots of calibration curves for the IDMS, internal standard, and external standard methods…………………………………………………………………………….S6

Table S1. Mass-labeled standards used for IDMS and the internal standard method

| Target analyte | Calibration method | Mass-labeled standard |
| --- | --- | --- |
| PFHxS | IDMS | ¹³C₃-PFHxS |
|  | IS | ¹⁸O₂-PFHxS |
| PFOS | IDMS | ¹³C₄-PFOS |
|  | IS | ¹⁸O₂-PFHxS |
| PFOA | IDMS | ¹³C₄-PFOA |
|  | IS | ¹⁸O₂-PFHxS |
| PFNA | IDMS | ¹³C₅-PFNA |
|  | IS | ¹⁸O₂-PFHxS |

Table S2. HPLC conditions

| Parameter | Conditions |  |  |
| --- | --- | --- | --- |
| Analytical column | Betasil® C18 (2.1 mm i.d. × 50 mm × 5.0 µm) |  |  |
| Guard column | ZORBAX Eclipse XDB-C8 (2.1 mm id. × 12.5 mm × 5 μm) |  |  |
| Column temperature (℃) | 30 |  |  |
| Injection volume (µL) | 10 |  |  |
| Mobile phase A | 10% methanol in 2 mM ammonium acetate solution |  |  |
| Mobile phase B | MeOH |  |  |
| Flow rate (mL/min) | 0.23 |  |  |
| Gradient | Time (min) | A (%) | B (%) |
|  | 0 | 90 | 10 |
|  | 2 | 50 | 50 |
|  | 10 | 0 | 100 |
|  | 14 | 0 | 100 |
|  | 15 | 90 | 10 |
|  | 20 | 90 | 10 |

Table S3. Mass transitions, Q1 pre-rod bias voltage (V), collision energy (eV), and Q3 pre-rod bias voltage (V) for individual compounds

| Compound | Mass transitions (m/z) | Q1 pre-rod bias voltage (V) | Collision energy (eV) | Q3 pre-rod bias voltage (V) |
| --- | --- | --- | --- | --- |
| PFHxS | 398.95 79.95* | 30 | 43 | 30 |
|  | 398.95 98.9 | 30 | 34 | 34 |
| PFOS | 498.95 79.95* | 34 | 55 | 30 |
|  | 498.95 98.95 | 34 | 43 | 34 |
| PFOA | 412.95 369.00* | 30 | 10 | 40 |
|  | 412.95 169.00 | 30 | 18 | 32 |
| PFNA | 462.95 418.95* | 30 | 11 | 30 |
|  | 462.95 219.00 | 32 | 16 | 34 |
| 13C3-PFHxS | 401.95 79.95 | 44 | 38 | 30 |
|  | 401.95 98.95 | 42 | 33 | 30 |
| 18O2-PFHxS | 402.95 83.95 | 29 | 39 | 13 |
| 13C4-PFOS | 502.95 79.95 | 24 | 55 | 27 |
|  | 502.95 98.95 | 24 | 45 | 27 |
| 13C4-PFOA | 417.00 172.00 | 15 | 19 | 15 |
| 13C5-PFNA | 468.00 423.00 | 17 | 11 | 29 |
| d3-N-MeFOSA | 515.00 168.90 | 36 | 25 | 30 |
|  | 515.00 219.00 | 38 | 24 | 44 |

* used for quantifier.

Table S4. p-values of homoscedasticity tests

|  | PFHxS | PFOS | PFOA | PFNA |
| --- | --- | --- | --- | --- |
| IDMS | 0.264 | 0.569 | 0.243 | 0.896 |
| IS | 0.349 | 0.394 | 0.436 | 0.093 |
| ES | 0.052 | 0.096 | 0.127 | 0.245 |

Table S5. p-values of lack-of-fit (LOF) tests

|  | PFHxS | PFOS | PFOA | PFNA |
| --- | --- | --- | --- | --- |
| IDMS | 0.0517 | 0.164 | 0.0821 | 0.770 |
| IS | 0.00562 ** | 0.000414 *** | 0.00346 ** | 2.13 × 10–8 *** |
| ES | 4.12 × 10–13 *** | 2.40 × 10–10 *** | 3.08 × 10–12 *** | 1.70 × 10–5 *** |

** p < 0.01 *** p < 0.001

Fig. S1. Residual plots of calibration curves for the IDMS, internal standard, and external standard methods