# Supplementary materials for:

Essential and Toxic Elements in Plant-Based Dairy Alternatives: Implications for Vegan Diets

Determination of the elements using HR-CS AAS

Electrothermal and flame high-resolution continuum source atomic absorption spectrometry (HR-CS ETAAS, HR-CS FAAS) was used for determination of elements.

An important aspect of elemental analysis is the correct selection of spectral lines with respect to the expected contents in the samples. Even though the low concentrations of Fe and Zn required the use of a sensitive electrothermal technique, the sensitivity of the measurement had to be slightly reduced to ensure the results fell within the linear calibration range of the instrument. For the determination of Fe, a secondary spectral line at 252.744 nm with a sensitivity of 21% (relative to the most sensitive primary Fe line at 248.327 nm) was selected as suitable with respect to its concentration ranging from units to hundreds of μg/L in the digests. For Zn, the selection of available spectral lines is limited to a primary line, which is overly sensitive (and therefore difficult to use), and a single secondary line, which is very weak. Therefore, the method described in Zvěřina et al., 2020 [1] was adapted. Briefly, this method involves Zn measurement at the primary line of 213.857 nm, though with its oversensitivity compensated by reading the signal at the wings of the line. In this way, the working range can be adapted to the needs of the analysis, i.e., concentrations can start at low tens of micrograms of Zn per liter. This reduction in sensitivity for elements such as Fe and Zn rather than the excessive dilution of samples is beneficial, because the fluctuation of blank signal due to their high abundance in the environment is often the main limiting factor with respect to the power of detection when using the dilution approach with the most sensitive lines [2]. Trace contents of the toxic elements Cd and Pb were determined using their sensitive spectral lines at 228.802 and 283.306 nm, respectively.

Temperature programs of the graphite furnace were optimized as well as the addition of Pd-based matrix modifier (1 g/L Pd in 0.6 g/L Mg(NO3)2). The addition of the modifier also reduces the formation of interfering PO molecules during the P determination.

The additions of CsCl with a concentration of 0.1% Cs in the analytical sample, and 5-sulfosalicylic acid with a concentration of 0.02 mol/L in the analytical sample were used for Ca determination by HR-CS FAAS.

Three parallel analyses were performed for each sample and replicate.

Table S1: Mineral composition of the analyzed samples

| product | group | country of origin | Ca mg/100g | Zn mg/100g | Fe mg/100g | P mg/100g | Pb μg/kg | Cd μg/kg | Ca/P ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| coconut yogurt | yogurt | Germany | 2.2 | 0.080 | 0.118 | 16.2 | <7 | <0.7 | 0.13 |
| coconut yogurt | yogurt | Austria | 7.4 | 0.100 | 0.242 | 24.7 | <7 | 3.3 | 0.30 |
| coconut yogurt | yogurt | Poland | 4.4 | 0.110 | 0.203 | 21.1 | <7 | <0.7 | 0.21 |
| fermented coconut yogurt | yogurt | Germany | 165 | 0.140 | 0.215 | 130 | <7 | <0.7 | 1.27 |
| chocolate coconut yogurt | yogurt | Austria | 6.5 | 0.240 | 0.550 | 33.4 | <7 | 15.1 | 0.19 |
| fermented coconut yogurt | yogurt | Belgium | 5.8 | 0.100 | 0.199 | 23.3 | <7 | <0.7 | 0.25 |
| almond milk | milk | Germany | 5.7 | 0.020 | 0.025 | 5.3 | <7 | <0.7 | 1.08 |
| rice-coconut milk | milk | Belgium | 4.7 | 0.030 | 0.036 | 16.2 | <7 | <0.7 | 0.29 |
| almond milk | milk | Italy | 16.9 | 0.040 | 0.066 | 14.3 | <7 | <0.7 | 1.18 |
| soy milk | milk | Austria | 14.2 | 0.290 | 0.335 | 42.5 | <7 | 2.1 | 0.33 |
| almond milk | milk | Germany | 9.2 | 0.040 | 0.049 | 9.9 | <7 | <0.7 | 0.93 |
| spelt milk | milk | Austria | 8.2 | 0.079 | 0.073 | 19.4 | <7 | <0.7 | 0.42 |
| rice milk | milk | Germany | 3.9 | 0.025 | 0.011 | 3.6 | <7 | <0.7 | 1.09 |
| oat milk | milk | Germany | 2.8 | 0.021 | 0.020 | 9.9 | <7 | <0.7 | 0.28 |
| oat milk | milk | Austria | 4.7 | 0.101 | 0.069 | 21.1 | <7 | 2.0 | 0.22 |
| coconut-rice-cashew milk | milk | Austria | 7.2 | 0.165 | 0.138 | 18.0 | <7 | <0.7 | 0.40 |
| almond milk | milk | Belgium | 9.0 | 0.139 | 0.127 | 25.8 | <7 | <0.7 | 0.35 |
| oat milk | milk | Germany | 3.6 | 0.315 | 0.057 | 20.5 | <7 | <0.7 | 0.17 |
| rice milk | milk | Germany | 3.5 | 0.009 | 0.007 | 5.1 | <7 | <0.7 | 0.69 |
| rice milk | milk | Italy | 1.7 | 0.394 | 0.011 | 13.7 | <7 | 1.6 | 0.12 |
| oat milk | milk | Italy | 1.4 | 0.035 | 0.030 | 17.2 | <7 | <0.7 | 0.08 |
| coconut-rice milk | milk | Germany | 4.3 | 0.023 | 0.024 | 5.3 | <7 | <0.7 | 0.80 |
| spelt milk | milk | Germany | 3.7 | 0.109 | 0.101 | 45.8 | <7 | <0.7 | 0.08 |
| coconut milk | milk | Germany | 2.5 | 0.029 | 0.041 | 23.0 | <7 | <0.7 | 0.11 |
| soy milk | milk | Germany | 11.7 | 0.421 | 0.457 | 49.5 | <7 | 3.8 | 0.24 |
| soy milk | milk | Germany | 10.3 | 0.318 | 0.350 | 59.3 | <7 | 3.0 | 0.17 |
| oat-sunflower seed cream | cream | Germany | 5.7 | 0.055 | 0.070 | 17.4 | <7 | <0.7 | 0.33 |
| oat cream | cream | Netherlands | 3.0 | 0.047 | 0.055 | 16.2 | <7 | <0.7 | 0.19 |
| soy cream | cream | Germany | 16.4 | 0.199 | 0.321 | 30.2 | <7 | <0.7 | 0.54 |
| soy-sunflower seed cream | cream | Germany | 14.6 | 0.389 | 0.475 | 57.9 | <7 | <0.7 | 0.25 |
| soy cream | cream | Belgium | 6.1 | 0.166 | 0.199 | 31.9 | <7 | <0.7 | 0.19 |
| vegan cheese | cheese | France | 603 | 0.043 | 0.139 | 6.0 | <7 | <0.7 | 100 |
| vegan cheese | cheese | Germany | 202 | 0.054 | 0.114 | 23.3 | <7 | <0.7 | 8.65 |
| vegan cheese | cheese | Germany | 32.8 | 0.033 | 0.132 | 18.9 | <7 | <0.7 | 1.73 |
| vegan cheese | cheese | Greece | 6.1 | 0.061 | 0.105 | 8.7 | <7 | <0.7 | 0.70 |
| natural tofu | tofu | UK | 186 | 1.80 | 1.71 | 298 | <7 | 14.6 | 0.62 |
| natural tofu | tofu | Czech Republic | 51.3 | 0.930 | 2.48 | 212 | <7 | 23.1 | 0.24 |
| natural tofu | tofu | Germany | 115 | 1.29 | 2.00 | 211 | <7 | 14.4 | 0.55 |
| natural tofu | tofu | Slovakia | 133 | 1.50 | 1.47 | 286 | <7 | 14.2 | 0.47 |
| flaxseed milk | DIY seed milk | Romania | 45.0 | 0.590 | 0.615 | 78.8 | <7 | 18.3 | 0.57 |
| flaxseed milk | DIY seed milk | Kazakhstan | 38.3 | 0.480 | 0.577 | 63.4 | <7 | 14.6 | 0.60 |
| flaxseed milk | DIY seed milk | Hungary | 41.2 | 0.530 | 0.455 | 59.3 | <7 | 29.4 | 0.69 |
| hemp milk | DIY seed milk | France | 16.0 | 0.790 | 0.749 | 153 | <7 | 1.0 | 0.10 |
| hemp milk | DIY seed milk | Romania | 19.6 | 1.02 | 0.854 | 189 | <7 | 3.2 | 0.10 |
| hemp milk | DIY seed milk | Czech Republic | 11.8 | 0.420 | 0.434 | 99.2 | <7 | 0.9 | 0.12 |
| pumpkin milk | DIY seed milk | China | 10.4 | 0.270 | 0.410 | 88.3 | <7 | 1.2 | 0.12 |
| pumpkin milk | DIY seed milk | Czech Republic | 10.3 | 0.580 | 0.489 | 112 | <7 | 1.4 | 0.09 |
| pumpkin milk | DIY seed milk | Germany | 8.1 | 0.210 | 0.229 | 65.9 | <7 | 1.1 | 0.12 |
| poppy seed milk | DIY seed milk | Slovakia | 50.7 | 0.720 | 0.946 | 125 | <7 | 58.8 | 0.41 |
| poppy seed milk | DIY seed milk | Turkey | 79.1 | 0.810 | 1.00 | 143 | <7 | 13.4 | 0.55 |
| poppy seed milk | DIY seed milk | Czech Republic | 49.3 | 0.880 | 0.896 | 146 | <7 | 85.0 | 0.34 |
| sesame milk | DIY seed milk | Slovakia | 23.0 | 0.610 | 0.633 | 113 | <7 | 0.7 | 0.20 |
| sesame milk | DIY seed milk | Bolivia | 19.4 | 0.560 | 0.627 | 90.6 | <7 | 1.5 | 0.21 |
| sesame milk | DIY seed milk | China | 8.6 | 0.370 | 0.314 | 63.6 | <7 | 0.7 | 0.14 |

References

[1] Zvěřina, O., Coufalík, P., Šimůnek, J., Kachlík, P., Chlupová, R., Pavelková, J., 2020. Inorganic pollutants in the indoor environment of the Moravian Library: assessment of Cd, Pb, Cu, and Zn in total suspended particles and dust using HR-CS GF-AAS. Environ. Monit. Assess. 192, 771. https://doi.org/10.1007/s10661-020-08748-7

[2] Schlemmer, G., Petek, M., 2000. Optimization of a class 100 clean room cabinet for electrothermal atomization AAS. Atomic Spectroscopy 21, 1–4. https://doi.org/10.46770/AS.2000.01.001