Tailoring Microalgal Extract Properties via Non-Thermal Pretreatments: Application-Specific Optimization in Chlorella vulgaris

Ali Saebi1, 2, Ebrahim Mahmoudi1, Masturah Markom1, *, Saeid Minaei2, Alireza Mahdavian2, Mohammad-Taghi Ebadi3, Kim S Siow4‎

Department of Chemical and Process Engineering, Universiti Kebangsaan Malaysia (UKM), Bangi, 43600, Selangor, Malaysia1‎

Biosystems Engineering Department, Tarbiat Modares University (TMU), Tehran, Iran2‎

Department of Horticultural Science, Tarbiat Modares University (TMU), Tehran, Iran3‎

Institute of Microengineering and Nanoelectronics (IMEN), Universiti Kebangsaan Malaysia, (UKM), 43600, Bangi, Selangor, Malaysia4‎

Corresponding author email: masturahmarkom@ukm.edu.my

Materials and Methods

‎Details and Equipment Devices  for Non-Thermal Pretreatments

Table S1 Operational Parameters of Non-Thermal Pretreatments Used in Upstream Processing of C. vulgaris for SC-CO₂ Extraction

| Pretreatment | Equipment Model/Manufacturer | Key Operational Parameters | Notes |
| --- | --- | --- | --- |
| Ultrasound (U) | Ultrasonic bath (S100H, Elma, Singen, Germany) | 1130 W, 120 seconds, pH 9, 50 ºC | Utilizes acoustic cavitation for cell disruption |
| Microwave (M) | Classical microwave oven (MM720, Midea, Guangdong, China) | 800 W, 120 s, 2450 MHz | Employs electromagnetic radiation for dielectric heating |
| Cold Plasma (CP) | Custom-built plasma reactor, Radio-frequency (RF-3-XⅢ) generator | 13.56 MHz, 10 W deposition power, ~582 mTorr (77.60 Pa) working pressure, Nitrogen gas (~300 mTorr N2 for 120 s) | Generates reactive species (ions, electrons, radicals) for surface etching and oxidation |
| Cryogenic (C) | (Implies standard lab equipment, e.g., ceramic mortar, liquid nitrogen dewar) | Liquid nitrogen (-196 ºC), 3 repetitions of freeze-thaw cycles | Rapid freezing induces intracellular ice crystals for mechanical rupture |

Extract Yield and Bioactive Compound Analyses

Total Phenolic Content – Folin–Ciocalteu Method	
The total phenolic content of the extracts was quantified using the Folin–Ciocalteu method, with gallic acid (0–200 µg/mL) serving as the calibration standard. In brief, 100 µL of the sample or standard was mixed with 500 µL of Folin–Ciocalteu reagent, which had been previously diluted 1:10 (v/v) with distilled water. After allowing the mixture to stand for 5 minutes at room temperature, 400 µL of a 7.5% w/v sodium carbonate (Na₂CO₃) solution was added. The reaction mixture was then incubated in the dark at ambient temperature for 20 minutes. Absorbance was recorded at 765 nm using a UV–Vis spectrophotometer. A standard calibration curve was constructed based on gallic acid concentrations, with a typical R² value of approximately 0.998. Final results were expressed as milligrams of gallic acid equivalents per gram of extract (mg GAE/g extract). This protocol was adapted from Drosou et al. (2015) [1], based on foundational methods by Ainsworth & Gillespie (2007) [2].

Antioxidant Capacity– DPPH Radical Scavenging Assay	
The antioxidant capacity of the extracts was evaluated using the DPPH (2,2-diphenyl-1-picrylhydrazyl) radical scavenging assay. A 0.1 mM DPPH solution was freshly prepared in methanol or ethanol, depending on extract compatibility. To each reaction tube, 100 µL of extract was added to 2 mL of the DPPH solution, followed by incubation in the dark at room temperature for 30 minutes. After incubation, absorbance was measured at 517 nm using a UV–Vis spectrophotometer. A Trolox standard curve ranging from 0 to 250 µM was used to quantify antioxidant capacity, with results expressed in mg Trolox equivalents per gram of extract (mg TE/g extract). The calibration curve typically showed a high linearity (R² ≈ 0.997) [3].

TOPSIS-Shannon Entropy Method

To identify the optimal pretreatment for supercritical CO₂ extraction, five criteria (chlorophyll, phenolics, carotenoids, and antioxidant capacity, and yield) were selected based on their impact on extraction [4]. To identify the optimal pretreatment for supercritical CO₂ extraction, five criteria (chlorophyll, phenolics, carotenoids, antioxidant capacity, and yield) were selected based on their impact on extraction. This multi-criteria approach is essential for rationally evaluating complex engineering processes where multiple, sometimes conflicting, objectives must be considered for optimal design and performance. This multi-criteria approach is essential for rationally evaluating complex engineering processes where multiple, sometimes conflicting, objectives must be considered for optimal design and performance. Shannon entropy determined objective weights, emphasizing criteria variability [5]. This method evaluates the dispersion of data across each criterion, where higher variability implies greater decision impact. The entropy (Ej) for each criterion (j) is calculated as (Eq. S1):

Eq. S1   Ej=-ki=1mPij×lnPij, where Pij=Xiji=1mXij, and K=1lnm

Here, xij is the normalized value of alternative i under criterion j, and m is the number of alternatives (pretreatment levels). The weight wj for each criterion is derived as (Eq. S2):

Eq. S2   wj=1-Ejk=1n1-Ej

After determining the weights, the Technique for Order Preference by Similarity to Ideal Solution (TOPSIS) was applied to rank the pretreatment levels. In this method, each alternative is evaluated based on its Euclidean distance from the ideal best (Si⁺) and worst (Si⁻) solutions (Eq. S3):

Eq. S3   Si+=wj×xij-xj+2

Si-=wj×xij-xj-2

The closeness coefficient Ci, determined the relative optimality of each pretreatment (Eq. S4):

Eq. S4   Ci=Si-Si-+Si+

Final ranking results are discussed later.

Results and Discussion

FE-SEM Analysis

FE-SEM micrographs highlighted clear differences in cell wall integrity among ‎pretreatments. The cryogenic-treated biomass (C) exhibited extensive surface collapse ‎and pore formation, suggesting maximal cell disruption, while CP also showed marked ‎deformation. Conversely, M and U samples retained more structural features, indicating ‎less effective disruption. These morphological observations directly correlate with extract ‎yields and reinforce the importance of pretreatment intensity in enhancing CO₂ penetration ‎and bioactive release for nutraceutical applications (Table S2).‎

Table S2 FE-SEM analysis of C. vulgaris residual biomass

| Algae Species Tested | FE-SEM Observations | Industrial Relevance | Reference |
| --- | --- | --- | --- |
| Microalgae powder | Microalgae oil extracted | Biofuel, enriched food supplements | [6] |
| Auxenochlorella protothecoides and C. vulgaris | Permeabilization of cell membranes, | Morphological changes indicate enhanced cell wall permeability, directly impacting mass transfer kinetics and solvent penetration efficiency in extraction processes | [7] |
| Ourococcus multisporus, Nitzschia cf. pusilla, Chlamydomonas mexicana, Scenedesmus obliquus, C. vulgaris, and Micractinium reisseri | Lipid extraction | Wastewater treatment and biodiesel production | [8] |
| Porphyridium purpureum | Cell disruption | Extract Phycoerythrin | [1] |
| Nanochloropsis sp. | Cell disruption and optimizing extraction condition | Lipid and eicosapentaenoic acid | [8] |
| C. vulgaris | Cell disruption | Bioactive extract | This study |

TGA Analysis

TGA data revealed distinct thermal decomposition phases, with major mass loss occurring ‎between 176.66–509.16 °C due to the breakdown of biomolecular constituents. Samples ‎treated with C and M methods exhibited lower residual mass (18.24% and 28.59%, ‎respectively), reflecting more effective cell disruption and reduced ash content. These ‎trends suggest improved extractability and purity of bioactives, particularly relevant for ‎pharmaceutical applications. In contrast, higher residue in UC, U, and CP may favor ‎mineral-enriched formulations. Detailed comparative results are summarized in Table S3.‎

Table S3 TGA Analysis of C. vulgaris residual biomass

| Algae Species Tested | TGA Findings | Comparison to Current Study | Reference |
| --- | --- | --- | --- |
| Tetradesmus dimorphus, Tetradesmus obliquus, Chlorella sp., and Chlorella sorokiniana | 5-7% initial loss, 67-80% decomposition in the second phase, and 3-5% final reduction | Steeper slopes correlate with more residual ash | [9] |
| C. vulgaris | 10-12% initial loss, 60-65% decomposition in the second phase, and 3-5% final reduction | Lower residual mass indicates higher organic matter decomposition, critical for downstream purification and biochar applications | [10] |
| Nannochloropsis salina | 3-5% initial loss, 70-75% decomposition in the second phase, and 10-15% final reduction | Pretreatments and extraction conditions are directly affecting lipid yield | [11] |
| Scenedesmus obliquus, Hlamydomonas sp., and C. vulgaris | 3-5% initial loss, 55-60% decomposition in the second phase, and 5-10% final reduction | Minimal residues improve biochar, biofilm, and biofuel yields | [12] |
| Nannochloropsis sp. | 7-10% initial loss, 70% decomposition in the second phase, and 10% final reduction | Residual weight varies with pretreatment | [13] |
| C. vulgaris | 7-10% initial loss, 45-60% decomposition in the second phase, and 10-15% final reduction | Pretreatments and extraction conditions are directly affecting bioactive extract | This study |

DSC Analysis

Differential Scanning Calorimetry (DSC) analysis revealed distinct thermal behaviors across pretreatment groups, reflecting differences in structural integrity, compound stability, and cellular disruption mechanisms. Table S4 summarizes the key transition parameters—onset temperature (To), peak temperature (Tp), and enthalpy change (ΔH)—for each treatment. UC exhibited sharp low-temperature endothermic peaks (To = 48.33 °C; Tp = 51.67 °C; ΔH = 38.2 J/g), suggesting rapid moisture loss and high retention of thermolabile compounds. In contrast, cryogenic (C) and ultrasound (U) treatments showed moderate transitions near 60 °C, indicative of mild cell permeabilization. Microwave (M) and cold plasma (CP) treatments shifted transitions to higher temperatures (e.g., Tp = 202.77 °C for CP), consistent with stronger matrix binding or partial thermal modification. These findings confirm that pretreatment strategy modulates thermal stability and compound release, impacting downstream formulation and storage.

Table S4 Onset temperature (To), peak temperature (Tp), and enthalpy change (ΔH) for each pretreatment group

| Assigned Transition | ΔH (J/g) | TP (ºC) | TO (ºC) | Treatment |
| --- | --- | --- | --- | --- |
| Moisture loss/ intact volatiles | 38.2 | 51.67 | 48.33 | UC |
| Gentle structural release | 35.7 | 60.48 | 56.12 | C |
| Mild thermal disruption | 32.9 | 66.1 | 59.43 | M |
| Crosslinked matrix destabilization | 29.3 | 202.77 | 85.5 | CP |
| Permeabilization, local shear effects | 33.8 | 63.33 | 58.19 | U |

Also, these thermograms revealed distinct endothermic and exothermic transitions across ‎samples. UC and C extracts exhibited early low-temperature endothermic peaks (∼50–‎‎91 °C), indicative of minimal thermal stress and better preservation of bioactive ‎compounds. In contrast, CP showed delayed endothermic and sharp exothermic ‎responses above 200 °C, suggesting structural rigidity and potential degradation of ‎thermolabile compounds. These thermal patterns support the suitability of UC and C ‎pretreatments for preserving extract integrity in food and pharmaceutical formulations ‎‎(Table S5).‎

Table S5 DSC analysis of C. vulgaris residual biomass

| Algae Species Tested | Effectiveness | DSC finding | Comparison to Current Study | Reference |
| --- | --- | --- | --- | --- |
| C. vulgaris and Arthrospira platensis | Antioxidant products | Oxidation onset varies with algae composition | Evaluate pretreatments for minimal biomass residues | [14] |
| C. vulgaris | Biopolymers 3D | Drug delivery: microalgae for controlled mineral release | Shifts in thermal transitions provide engineering insight into the thermal stability and structural integrity of the biomass, crucial for designing subsequent thermal processing steps and product shelf-life | [15] |
| C. vulgaris and Arthrospira platensis | Oscillatory rheology, particle size distribution and microstructure of microalgae | Water plasticization from high pressure affects the first endotherm | Pretreatment info for food design and gel optimization | [8] |
| C. vulgaris | From raw microalgae to bioplastics | Extraction tracks granule integrity | Pretreatments affect compound thermal behavior | [16] |
| C. vulgaris | Bioactive compounds | Different thermal transitions from pretreatments affects the first endotherm | - | This study |

UV-Vis Analysis

UV-Vis spectral analysis identified a strong chlorophyll α absorption peak at 666 nm, ‎particularly prominent in the UC and CP samples. This signal, consistent with prior reports, ‎reflects efficient pigment preservation under milder pretreatment conditions and highlights ‎the relevance of chlorophyll-rich extracts in antioxidant-focused nutraceuticals and ‎functional foods (Table S6).‎

Table S6 UV-Vis Spectroscopy of C. vulgaris Extracts

| Algae Species Tested | Assigned Compound | wavelength (nm) | Implication | Reference |
| --- | --- | --- | --- | --- |
| N. oculata and C. vulgaris | Phenolics | 407–408 | Antioxidant with bioactivity potential | [17] |
| Nannochloropsis gaditana, Spirulina, and Chlorella | Carotenoids | 418–478 | Renowned for provitamin A and antioxidants | [18] |
| ladophora glomerata, Cladophora rivularis, Ulva flexuosa, and Chlorella sp. | Chlorophyll b | 645 | Boosts natural colorants and antioxidants | [19] |
| Phaedactylium tricornitum, Nannochloropsis gaditana, Nannochloris sp, Tetraselmis suecica, N. oculata and C. vulgaris | Chlorophyll a | 663 | High pigment yield signals antioxidant capacity, enabling rapid in-process quality control for pigment-focused extraction streams | [19] |
| C. vulgaris | Chlorophyll a | 666 | High Carotenoid yield | This study |

FT-IR Analysis

FT-IR spectra showed a prominent O–H stretch (3200–3500 cm⁻¹) across all extracts, with ‎stronger transmittance in M, UC, and U. Notably, C, CP, and UC samples exhibited clear ‎C=O ester bands (1730–1750 cm⁻¹) and C–O peaks (~1100 cm⁻¹), indicative of fatty acids ‎and lipids. These spectral features correspond with enhanced extraction of bioactive ‎compounds, including chlorophylls, phenolics, and lipophilic metabolites, positioning ‎these extracts as suitable candidates for nutraceutical and pharmaceutical applications‎‎ indicating the success of cell wall disruption in liberating specific functional groups for targeted bioproduct formulation. (Table S7).‎

Table S7 FT-IR Spectroscopy of C. vulgaris Extracts

| Algae Species Tested | Functional Group | Wavenumber (cm⁻¹) | Compound Type | Reference |
| --- | --- | --- | --- | --- |
| Chlorella sp. | C-I | 617-734 | Aliphatic character | [20] |
| Chlorella sp., Dunaliella Salina, and Nannochloropsis sp. | C-O-C | 1080-1200 | Polysaccharides and nucleic acid | [21] |
| Nannochloropsis sp., Chlorella sp. and Dunaliella Salina | C-O | 1271-1392 | Phenols and alcoholic group | [22] |
| Nannochloropsis sp., Chlorella sp., Scenedesmus sp. and Dunaliella salina | C=O | 1745-1638 | Fatty acids, Carboxylic group of esters, and Protein (Amide I) | [23] |
| Nannochloropsis sp., Scenedesmus sp., Chlorella sp. and Dunaliella salina | C–H | 2920-2970 | Lipids and β-Carotene | [24] |
| Nannochloropsis sp., Chlorella sp. and Dunaliella salina | O-H/N-H | 3300-3400 | Water, protein | [25] |
| C. vulgaris | All above functional group | 400-4000 | The functional group | This study |

‎ References

1. 	Drosou C, Kyriakopoulou K, Bimpilas A, Tsimogiannis D, Krokida M (2015) A comparative study on different extraction techniques to recover red grape pomace polyphenols from vinification byproducts. Ind Crops Prod 75:141–149. https://doi.org/10.1016/j.indcrop.2015.05.063

2. 	Ainsworth EA, Gillespie KM (2007) Estimation of total phenolic content and other oxidation substrates in plant tissues using Folin–Ciocalteu reagent. Nat Protoc 2:875–877. https://doi.org/10.1038/nprot.2007.102

3. 	Thaipong K, Boonprakob U, Crosby K, Cisneros-Zevallos L, Byrne DH (2006) Comparison of ABTS, DPPH, FRAP, and ORAC assays for estimating antioxidant activity from guava fruit extracts. J food Compos Anal 19:669–675. https://doi.org/10.1016/j.jfca.2006.01.003

4. 	El-Morshedy SE-D, Ibrahim SMA, Alyan A, Abdelmaksoud A (2020) Heat transfer deterioration mechanism for water at supercritical pressure. Int J Thermofluids 7:100020. https://doi.org/10.1016/j.ijft.2020.100020

5. 	Wu H-W, Li E, Sun Y, Dong B (2021) Research on the operation safety evaluation of urban rail stations based on the improved TOPSIS method and entropy weight method. J Rail Transp Plan Manag 20:100262. https://doi.org/10.1016/j.jrtpm.2021.100262

6. 	Zaharescu T, Bumbac M, Nicolescu CM (2024) Stability effects of added biomass on microalgae styrene–butadiene–styrene composites. J Therm Anal Calorim 149:10793–10805. https://doi.org/10.1007/s10973-024-13419-7

7. 	Morais FP, Simões RMS, Curto JMR (2020) Biopolymeric delivery systems for cosmetic applications using chlorella vulgaris algae and tea tree essential oil. Polymers (Basel) 12:2689. https://doi.org/10.3390/polym12112689

8. 	El-Naggar NE-A, Hussein MH, Shaaban-Dessuuki SA, Dalal SR (2020) Production, extraction and characterization of Chlorella vulgaris soluble polysaccharides and their applications in AgNPs biosynthesis and biostimulation of plant growth. Sci Rep 10:3011. https://doi.org/10.1038/s41598-020-59945-w

9. 	Domenici V, Ancora D, Cifelli M, Serani A, Veracini CA, Zandomeneghi M (2014) Extraction of pigment information from near-UV vis absorption spectra of extra virgin olive oils. J Agric Food Chem 62:9317–9325. https://doi.org/10.1021/jf503818k

10. 	Li K, Jiang C, Han SI, Kang S, Chen J, Won D, Kang Y, Bae B, Choi YE, Kim HS, Lee J (2024) Green and efficient method to acquire high-value phycobiliprotein from microalgal biomass involving deep eutectic solvent-based ultrasound-assisted extraction. Food Chem 449:139196. https://doi.org/10.1016/j.foodchem.2024.139196

11. 	Zheng H, Yin J, Gao Z, Huang H, Ji X, Dou C (2011) Disruption of Chlorella vulgaris cells for the release of biodiesel-producing lipids: a comparison of grinding, ultrasonication, bead milling, enzymatic lysis, and microwaves. Appl Biochem Biotechnol 164:1215–1224. https://doi.org/10.1007/s12010-011-9207-1

12. 	Bogaerts A, Neyts E, Gijbels R, Van der Mullen J (2002) Gas discharge plasmas and their applications. Spectrochim Acta Part B At Spectrosc 57:609–658. https://doi.org/10.1016/S0584-8547(01)00406-2

13. 	Ahmed J, Kumar V (2022) Effect of high-pressure treatment on oscillatory rheology, particle size distribution and microstructure of microalgae Chlorella vulgaris and Arthrospira platensis. Algal Res 62:102617. https://doi.org/10.1016/j.algal.2021.102617

14. 	Salmon A (2021) Molecular Resiliency and Aging. Innov Aging 5:161. https://doi.org/10.1093/geroni/igab046.619

15. 	Wen J, Arthur K, Chemmalil L, Muzammil S, Gabrielson J, Jiang Y (2012) Applications of differential scanning calorimetry for thermal stability analysis of proteins: qualification of DSC. J Pharm Sci 101:955–964. https://doi.org/10.1002/jps.22820

16. 	Six A, Dauvillee D, Lancelon-Pin C, Dimitriades-Lemaire A, Compadre A, Dubreuil C, Alvarez P, Sassi JF, Li-Beisson Y, Putaux JL, Le Moigne N (2024) From raw microalgae to bioplastics: Conversion of Chlorella vulgaris starch granules into thermoplastic starch. Carbohydr Polym 342:122342. https://doi.org/10.1016/j.carbpol.2024.122342

17. 	Bhuvana P, Sangeetha P, Anuradha V, Ali MS (2019) Spectral characterization of bioactive compounds from microalgae: N. oculata and C. vulgaris. Biocatal Agric Biotechnol 19:101094. https://doi.org/10.1016/j.bcab.2019.101094

18. 	Haoujar I, Cacciola F, Abrini J, Mangraviti D, Giuffrida D, Oulad El Majdoub Y, Kounnoun A, Miceli N, Fernanda Taviano M, Mondello L, Rigano F (2019) The contribution of carotenoids, phenolic compounds, and flavonoids to the antioxidative properties of marine microalgae isolated from Mediterranean Morocco. Molecules 24:4037. https://doi.org/10.3390/molecules24224037

19. 	Boukaoud A, Chiba Y, Sebbar D (2021) A periodic DFT study of IR spectra of amino acids: An approach toward a better understanding of the NH and OH stretching regions. Vib Spectrosc 116:103280. https://doi.org/10.1016/j.vibspec.2021.103280

20. 	Ganeson Y, Paramasivam P, Palanisamy KM, Govindan N, Maniam GP (2024) LCMS and FTIR profiling of microalga Chlorella sp. for cosmetics and skin care applications. Clean Water 2:100028. https://doi.org/10.1016/j.clwat.2024.100028

21. 	Kanda H, Kusumi K, Zhu L, Wang T (2024) Direct Extraction of Lipids, β-Carotene, and Polyphenolic Compounds from Wet Microalga Dunaliella salina by Liquefied Dimethyl Ether. Mar Drugs 22:438. https://doi.org/10.3390/md22100438

22. 	Murdock JN, Wetzel DL (2009) FT-IR microspectroscopy enhances biological and ecological analysis of algae. Appl Spectrosc Rev 44:335–361. https://doi.org/10.1080/05704920902907440

23. 	Sudhakar K, Premalatha M (2015) Characterization of micro algal biomass through FTIR/TGA/CHN analysis: Application to Scenedesmus sp. Energy Sources, Part A Recover Util Environ Eff 37:2330–2337. https://doi.org/10.1080/15567036.2013.825661

24. 	Meng Y, Yao C, Xue S, Yang H (2014) Application of Fourier transform infrared (FT-IR) spectroscopy in determination of microalgal compositions. Bioresour Technol 151:347–354. https://doi.org/10.1016/j.biortech.2013.10.064

25. 	Calderón-Vergara LA, Díaz-Sánchez LM, Blanco-Tirado C, Combariza MY (2024) Comparative profiling of Chlorella vulgaris cells, extracts, and intact chloroplasts using electron transfer matrix-assisted laser desorption/ionization mass spectrometry (ET-MALDI-MS). Anal Methods 16:5652–5664. https://doi.org/10.1039/D4AY00846D