| Title: Contrasting metabolic responses to increasing temperature in four Mediterranean echinoderms |  |
| --- | --- |
| Authors: Marta Martín-Huete, Josep Forteza, Robert Fernández-Vilert, Javier Quesada, Carlos Leiva and Rocío Pérez-Portela. |  |
| Journal: Marine Biology |
| Supplementary Table S2. Checklist of 53 essential criteria for the reporting of methods for aquatic intermittent-flow respirometry by Killen et al., 2021. |  |

| Number | Criterion and Category | Response | Value (where required) | Units |
| --- | --- | --- | --- | --- |
|  | EQUIPMENT, MATERIALS, AND SETUP |  |  |  |
| 1 | Body mass of animals at the time of respirometry | Body mass was measured immediately after respirometry: for sea urchins using the ovoid formula from Elliott et al. (2012); for O. nigra by volumetry; for O. sp. II, measuring disk diameter and height with a calliper and calculating the volume using the cylindrical formula (V = π x H x R2). | species-specific | mL |
| 2 | Volume of empty respirometers | Total setup volume. | 465 mL (sea urchins); 251 mL (brittle stars) | mL |
| 3 | How chamber mixing was achieved | Recirculation using a peristaltic pump. |  |  |
| 4 | Ratio of net respirometer volume (plus any associated tubing in mixing circuit) to animal body mass | Ratio optimized for this study following Killen et al. (2021), Aliende-Hernández et al. (2024), Fernández-Vilert et al. (2025). | 11 |  |
| 5 | Material of tubing used in any mixing circuit | Silicone tubing; acrylic (methacrylate) chamber. |  |  |
| 6 | Volume of tubing in any mixing circuit | Tubing volume. | 65 mL | mL |
| 7 | Confirm volume of tubing in any mixing circuit was included in calculations of oxygen uptake | Yes, chamber volume plus recirculation tubing volume included. |  |  |
| 8 | Material of respirometer (e.g., glass, acrylic, etc.) | Acrylic. |  |  |
| 9 | Type of oxygen probe and data recording | Qubit oxygen sensor, Logger Pro 3.16.1 from Vernier Software Technology. |  |  |
| 10 | Sampling frequency of water dissolved oxygen | Dissolved oxygen was monitored continuously using an optical DO sensor throughout the duration of the experiment. | 1 Hz | Hz |
| 11 | Describe placement of oxygen probe (in mixing circuit or directly in chamber) | In the recirculation circuit, measuring water exiting/entering the chamber. |  |  |
| 12 | Flow rate during flushing and recirculation, or confirm that the chamber returned to normoxia during flushing | Flush: 5 min; recirculation: 7 min; closed: 10 min | 5-10 | min per cycle |
| 13 | Timing of flush/closed cycles | 5 min flush, 7 min recirculation, 10 min closed | 5-10 | min per cycle |
| 14 | Wait (delay) time excluded from closed measurement cycles | 5 min initial acclimation | 5 | min |
| 15 | Frequency and method of probe calibration (for both 0 and 100% calibrations) | Calibrated before each experiment for 0% and 100% O2. |  |  |
| 16 | State whether software temperature compensation was used during recording of water oxygen concentration | Yes, Logger Pro applies temperature compensation. |  |  |
|  | MEASUREMENT CONDITIONS |  |  |  |
| 17 | Temperature during respirometry | Progressive temperature increase experiment. |  |  |
| 18 | How temperature was controlled | With an external sensor: the S132 Temperature Sensor, component of the Q-box AQUA system. It has a resolution of 0.03 °C (0 to 40 °C). |  |  |
| 19 | Photoperiod during respirometry | The room light was switched on during the oxygen consumption measurements. After the measurements and the feeding time provided for the individuals to eat, the light was switched off. |  |  |
| 20 | If (and how) ambient water bath was cleaned and aerated during measurement of oxygen uptake (e.g., filtration, periodic or continuous water changes) | Aeration of the ambient bath during the measurements, to ensure that oxygen in the chamber and the bath is replaced during flushing, and removes metabolic CO2 produced by the animals. After each day of the experiment, the water was replaced |  |  |
| 21 | Total volume of ambient water bath and any associated reservoirs | 20 L | 20 | L |
| 22 | Minimum water oxygen dissolved oxygen reached during closed phases | Maintained above the experimental limit | Measured per trial | mg/L |
| 23 | State whether chambers were visually shielded from external disturbance | Partially shielded |  |  |
| 24 | How many animals were measured during a given respirometry trial (i.e., how many animals were in the same water bath) | 1 animal | 1 | individual |
| 25 | If multiple animals were measured simultaneously, state whether they were able to see each other during measurements | N/A |  |  |
| 26 | Duration of animal fasting before placement in respirometer | 22 hours | 22 | hour |
| 27 | Duration of all trials combined (number of days to measure all animals in the study) | 4 days per each experiment (control and treatment) for each species. |  |  |
| 28 | Acclimation time to the laboratory (or time since capture for field studies) before respirometry measurements | 48 h | 48 | hour |
|  | BACKGROUND RESPIRATION |  |  |  |
| 29 | State whether background microbial respiration was measured and accounted for, and if so, the method used (e.g., parallel measures with empty respirometry chamber, measurements before and after for all chambers while empty, both) | Yes, measured with blank runs with empty chamber before each experimental condition. |  |  |
| 30 | State if background respiration was measured at the beginning and/or end, state how many slopes and for what duration | Before each day of the experiment, 3 cycles per day. |  |  |
| 31 | State how changes in background respiration were modelled over time (e.g., linear, exponential, parallel measures) | Background respiration was measured in blank runs, and the resulting oxygen consumption was subtracted from the measured VO₂ of the animal. |  |  |
| 32 | Level of background respiration (e.g., as a percentage of SMR) | Calculated as VO2 animal – VO2 blank. |  |  |
| 33 | Method and frequency of system cleaning (e.g., system bleached between each trial, UV lamp) | Daily cleaning with distilled water and ethanol between experimental conditions and between species. |  |  |
|  | STANDARD OR ROUTINE METABOLIC RATE |  |  |  |
| 34 | Acclimation time after transfer to the chamber, or alternatively, time to reach the beginning of metabolic rate measurements after introduction to the chamber | 5 min |  |  |
| 35 | Time period, within a trial, over which oxygen uptake was measured (e.g., number of hours) | 10 min closed per cycle, 3 cycles. |  |  |
| 36 | Value taken as SMR/RMR (e.g., quantile, mean of the lowest 10 percent, mean of all values) | Difference between final VO2 and background VO2 |  |  |
| 37 | Total number of slopes measured and used to derive metabolic rate (e.g., how much data were used to calculate quantiles) | 3 cycles per individual. |  |  |
| 38 | Whether any time periods were removed from calculations of SMR/RMR (e.g., data during acclimation, periods of high activity [e.g., daytime]) | Initial 5 min acclimation. |  |  |
| 39 | r2 threshold for slopes used for SMR/RMR (or mean) | Not specified, assume the default Logger Pro software value. |  |  |
| 40 | Proportion of data removed due to being outliers below r-squared threshold | N/A |  |  |
|  | DATA HANDLING AND STATISTICS |  |  |  |
| 50 | Sample size | 6 - 8 individuals per species and condition. When the collection of the individuals, they were measured in situ with a caliper, only those with the study size were then used for the experiments. |  |  |
| 51 | How oxygen uptake rates were calculated (software or script, equation, units, etc.) | VO2 = DOSlope*(Vr-Va)*60/m-BackgroundVO2 |  | VO2: oxygen consumption (mg/kg/hour) DOSlope: rate of decrease of DO (mg/L/min) Vr: respirometer volume (mL) Va: volume of experimental animal (mL) m: animal weight (g) BackgroundVO2: VO2 measured with empty chamber |
| 52 | Confirm that the volume (mass) of the animal was subtracted from the respirometer volume when calculating oxygen uptake rates | Yes, included in calculations. |  |  |
| 53 | State whether analyses accounted for variation in body mass and describe any allometric mass-corrections or adjustments | Yes, values normalized by body mass. |  |  |