Imaging protocol for our study-

1. T1-Weighted Imaging:

- Sequence Type: Spin Echo (SE)

- TR (Repetition Time): 500 ms

- TE (Echo Time): 10 ms

- Flip Angle: 90° (for SE), typically 20-30° (for GRE)

- Slice Thickness: 3 mm ( without gap)

- Field of View (FOV): 220 mm

- Matrix Size: 512 x 512

- Number of Slices: 20 slices

- Acquisition Time: 4 minutes

2. T2-Weighted Imaging:

- Sequence Type: Spin Echo (SE)

- TR: 6000 ms

- TE:120 ms

- Flip Angle: 90°

- Slice Thickness: 3 mm (without gap)

- Field of View (FOV): 220 mm

- Matrix Size: 512 x 512

- Number of Slices: 20 slices

- Acquisition Time: 4 minutes

3. FLAIR (Fluid-Attenuated Inversion Recovery):

- Sequence Type: Inversion Recovery

- TR: 6000 ms

- TE: 120 ms

- TI (Inversion Time): 2500 ms

- Flip Angle: 90°

- Slice Thickness: 3 mm (without gap)

- Field of View (FOV): 220 mm

- Matrix Size: 512 x 512

- Number of Slices: 20 slices

- Acquisition Time: 6 minutes

4. Diffusion Weighted Imaging (DWI):

- Sequence Type: Echo-Planar Imaging (EPI)

- TR: 8000 ms

- TE: 100 ms

- b-values: 0, 1000 s/mm²

- Slice Thickness: 4 mm (no gap)

- Field of View (FOV): 220 mm

- Matrix Size: 256 x 256

- Number of Slices: 20 slices

- Acquisition Time: 2 minutes

5. Post-Gadolinium T1-Weighted Imaging (Axial):

- Sequence Type: Gradient Echo (GRE)

- TR: 500 ms

- TE: 3 ms

- Flip Angle: 70°

- Slice Thickness: 3 mm (without gap)

- Field of View (FOV): 220 mm

- Matrix Size: 512 x 512

- Contrast Agent: Gadolinium-based contrast (typically after a 0.1 mmol/kg dose)

- Number of Slices: 20 slices

- Acquisition Time: 4 minutes

6. MPRAGE (Magnetization Prepared Rapid Acquisition Gradient Echo):

- Sequence Type: Gradient Echo (GRE) with Inversion Recovery Preparation

- TR: 3000 ms

- TE: 2 ms

- TI (Inversion Time): 1200 ms

- Flip Angle: 8° (low flip angle)

- Slice Thickness: 1 mm (no gap)

- Field of View (FOV): 220 mm

- Matrix Size: 512 x 512

- Number of Slices: 200 slices (with high resolution)

- Acquisition Time: 8 minutes