SCORING OF HIP MRI FOR JIA

SYNOVITIS (see atlas) Based on (late*) post-contrast 3-D GRE images (if present) and coronal T2-W FS images. Synovitis is first scored according to enhancement and second to the degree of synovial thickening and third to the degree of overall inflammatory changes (enhancement/synovial thickness) and finally to the degree of overall inflammation (including effusion).

Online Supplementary Material 1: Hip MRI Scoring System for JIA

| 1. Enhancement intensity (as compared to pre-contrast FS images. In cases of patchy increased enhancement, the predominant appearance should be scored). First, at a 0–3 scale: Score 0: no, or very subtle synovial enhancement Score 1: mildly increased synovial enhancement Score 2: moderately increased synovial enhancement (SI<vessels) Score 3: severely increased synovial enhancement (SI≥vessels) And second, at a 0–2 scale (=wrist score): Score 0: no, or very subtle synovial enhancement Score 1: mildly increased synovial enhancement (more than muscle, less than vessel) Score 2: moderate to severely increased synovial enhancement (≥vessel) 2. Assessment of the synovium (based on post-contrast image) - Thickening of the synovium (one layer) as: 0 = no thickening 1 = mild thickening 2 = moderate 3 = severe - Thickening of the synovium (in mm) measured medially and laterally on coronal images (mid-section). In case one cannot differentiate between the layers, measure all the capsule–osseous distance and divide in two. Add measurement on axial reconstructed images (only for OPBG cases). - Enhancement no/yes: 0/1 3. Overall synovial inflammation (both enhancement intensity and synovial thickness) (at a 0–3 scale): Score 0: no Score 1: mild overall involvement Score 2: moderate overall involvement Score 3: severe overall involvement 4. Effusion (based on T2-W FS images) 0 = no 1 = a sliver 2 = mild 3 = moderate amount 4 = large amount of fluid 5. Overall degree of inflammation (=wrist score, i.e. enhancement intensity, synovial thickness and effusion): Score 0: no inflammation Score 1: mild inflammation Score 2: moderate inflammation Score 3: severe inflammation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SYNOVITIS SCORING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Enhancement intensity |  |  | Synovial thickness |  |  |  |  | Synovial enhancement |  | Overall synovial inflammation |  | Effusion | Overall degree of inflammation |  |
|  | 0–3 | 0–2 |  | 0–3 |  | In mm M L |  |  | 0–1 |  | 0–3 |  | 0–4 | 0–3 |  |
| Rt hip |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lt hip |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BONE MARROW CHANGES (BMC) Defined as high signal intensity on T2-W FS/STIR images with corresponding low SI on T1-W images. Assess BMC in the: 1) femoral head, based on the proportion of bone involved (volume): score 0 = 0%, score 1 = 1–33%, score 2 = 34–66%, score 3 = 67–100%; 2) acetabulum: 0 = no, 1 = mild, 2 = moderate/significant; and 3) femoral neck (0 = no, 1 = yes). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BMC (0–3) |  |  | Right femoral epiphysis |  |  |  |  |  |  |  | Left femoral epiphysis |  |  |  |  |
| BMC (0–2) |  |  | Right acetabulum |  |  |  |  |  |  |  | Left acetabulum |  |  |  |  |
| BMC (0–1) |  |  | Right femoral neck |  |  |  |  |  |  |  | Left femoral neck |  |  |  |  |
| DESTRUCTIVE CHANGES: FLATTENING, EROSIONS, CYSTS Based on 3-D spin-echo (SE) T1-W images and fluid-sensitive and post-contrast images when appropriate: Flattening = a flattened femoral head as seen in the coronal plane (mid-section) compared to what is expected for age Erosion = a bony depression in at least 2 planes Active erosion = an erosion filled with enhancing pannus Bone cysts = sharply delineated, enhancing lesions with high signal on fluid-sensitive sequences |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | A. Flattening of the femoral head is first assessed subjectively at a 0–4 scale, and thereafter using a Mose circle, in increments of 25%: 0 = <10%, 1 = 10–25%, 2 = 26–50%, 3 = 51–75%, 4 = 76–100% |  |  |  |  |  |  |  |  |  |  |  |
| Flattening (0–4) |  |  |  |  |  |  | Right femoral head |  |  |  |  | Left femoral head |  |  |
|  |  |  | Subjectively |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Mose circle |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | B.1 Erosion. Assess the femoral head for bony erosions based on the proportion of femoral head volume involved, in increments of 25%: 0 = 0%, 1 = 1–25%, 2 = 26–50%, 3 = 51–75%, 4 = 76–100% |  |  |  |  |  |  |  |  |  |  |  |
| Erosion (0–4) |  |  | Right femoral head volume |  |  |  |  |  |  |  | Left femoral head volume |  |  |  |
|  |  |  | Right femoral head |  |  |  |  |  |  |  | Left femoral head |  |  |  |
| Active erosion (no/yes = 0/1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Right femoral head |  |  |  |  |  |  |  | Left femoral head |  |  |  |
| Bone cysts (no/yes = 0/1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | B.2 Assess the femoral neck for bony erosions (0=no, 1=yes) and cyst. Small irregularities at the physis are be scored as 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Right femoral neck |  |  |  |  |  |  |  | Left femoral neck |  |  |  |
| Erosion (0–1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bone cysts (no/yes = 0/1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | B.3 Assess the acetabulum for bony erosions, based on the surface of involvement of the acetabular margin, as 0 = no erosion, 1 = 1–33% of margin eroded, 2 = 34–66% of margin eroded, 3 = 67–100% of margin eroded. Simplified scoring for acetabulum: 0 = no irregularities, 1 = bone irregularities, 2 = bone erosions (0/2) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Right acetabulum |  |  |  |  |  |  |  |  | Left acetabulum |  |  |
| Erosion (0–3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Simplified erosion (0–2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bone cysts (no/yes = 0/1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CARTILAGE DAMAGE Measure the joint cartilage thickness (mm) on post-contrast 3-D GRE T1-W sequences superiorly (mid-weight-bearing area) and next, score subjectively as normal, mildly, moderately or severely narrowed (0–3 scale). Assess the cartilage in terms of signal abnormalities and morphological changes scored in a 0–4 scale: 0 = no cartilage changes, 1 = signal abnormalities without morphological defects, 2 = morphological changes involving 0–33% of the joint surface, 3 = morphological changes involving 34–66% of the joint surface, 4 = morphological changes involving 67–100% of the joint surface. Assess symmetry: right vs. left joint space height (JSH): 0 = symmetrical, 1 = right<left, 2 = right>left |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Right hip |  |  |  |  |  |  | Left hip |  |  |  |
| Joint space width (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Narrowed JSH (0–3) (no–mild–moderate–severe) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cartilage changes (0–4) (signal/loss %) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Symmetry (0–2) (only one score for both) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| POTENTIAL GROWTH ABNORMALITIES (assessed similarly to radiographs) Based on coronal T1-W MRI sequences |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Right hip |  |  | Left hip |  |  |
| Femoral neck width (in mm, outer cortex) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Femoral head/neck length (in mm, outer cortex) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CCD-angle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Troch–fem head distance (outer cortex) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Closed physis (0=no, 1=yes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coxa magna (0=no, 1=yes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coxa brevis (0=no, 1=yes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Protrusio acetabulae (0=no, 1=yes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fovea enlargement (0=no, 1=probably, 2=definitely) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECONDARY POST-INFLAMMATORY CHANGES Based on both TSE T1 and fluid-sensitive FS sequences |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Right hip |  |  | Left hip |  |  |
| Osteophytes (0=no, 1=yes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sclerotization (0=no, 1=yes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

3-D three-dimensional, FS fat saturated, GRE gradient echo, JIA juvenile idiopathic arthritis, Lt left, OPBG Bambino Gesù Children’s Hospital, Rt right, SI signal intensity, STIR short tau inversion recovery, T1-W T1-weighted, T2-W T2-weighted

AROME 30.10.16, adjusted in Rome as of 18.11.16, in Amsterdam as of 22.10.17 and in Rome as of 15.01.18