Supplementary information

Supplemental Table Institutional clinical MR enterography protocol at 1.5-T

| Sequence | Excitation Flip Angle | Slice Thickness / Gap (mm) | Echo Time (ms) | Repetition Time (ms) | Number of Signal Averages | b-values (s/mm2) / Averages | Fat Suppression Method |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coronal SSFSE | 90 | 5 / 1 | 80 | NA | 1 | NA | NA |
| Axial SSFSE | 90 | 5 / 1 | 80 | NA | 1 | NA | NA |
| Axial SSFSE with fat-saturation | 90 | 5 / 1 | 80 | NA | 1 | NA | SPAIR |
| Axial diffusion-weighted imaging | 90 | 6 / 1 | Minimum (~62) | 3000 | 1 | 0 /2, 100 /2, 800 / 4 | SPIR |
| Coronal precontrast 3D T1-weighted* | 15 | 5# | Minimum (both echoes) | Minimum | 1 | NA | Dixon |
| Coronal postcontrast 3D T1-weighted | 15 | 5# | Minimum (both echoes) | Minimum | 1 | NA | Dixon |
| Axial postcontrast 3D T1-weighted | 15 | 5# | Minimum (both echoes) | Minimum | 1 | NA | Dixon |

NA=not applicable; SPAIR=spectral adiabatic inversion recovery; SPIR=spectral presaturation with inversion recovery; SSFSE=single-shot fast spin-echo

*3D two-echo radiofrequency spoiled gradient echo Dixon sequence

#reconstructed at 2.5 mm increments