Supplementary material

Table 1. Primary MRI acquisition parameters per sequence.

| Sequence | Voxel size (mm) | FOV (mm) | Resolution (base phase) | TR (msec) | TE (msec) | Slice thickness (mm) | Flip angle |
| --- | --- | --- | --- | --- | --- | --- | --- |
| t1_tse_COR | 2.0×1.5×3.0 | 480 | 320 75% | 658 | 8.8 | 3 | 150 |
| t1_tse_SAG | 2.0×1.5×4.0 | 480 | 320 75% | 658 | 8.8 | 4 | 150 |
| t1_tse_AX | 1.2×1.2×5.0 | 300 | 256 100% | 850 | 9.1 | 5 | 160 |
| STIR_AX | 1.2×1.2×5.0 | 300 | 256 100% | 6550 | 87 | 5 | 140 |
| T2_se_AX_FS | 1.2×1.2×5.0 | 300 | 256 100% | 1500 | 9.6 | 5 | 180 |

COR= coronal; SAG= sagittal; AX= axial; TR= Repetition time; TE= Time to echo; tse= turbo spin echo; se= spin echo; STIR= short TI inversion recovery; FS= Fat suppression.

Table 2. Definition and scoring criteria of the evaluated MRI parameters.

| Oedema | Definition | Increased intramuscular signal intensity within muscle tissues on STIR-weighted images. |
| --- | --- | --- |
|  | Scoring | 0 = none. 1 = up to 1/3 of muscle volume involved. 2 = 1/3 – 2/3 of muscle volume involved. 3 = greater than 2/3 of muscle volume involved. |
| Fatty infiltration | Definition | Fatty replacement of muscle tissue, defined as intramuscular T1 hyper intense signal, which suppresses on STIR or fat-saturated images. |
|  | Scoring | 0 = none. 1 = up to 1/3 of muscle volume involved. 2 = 1/3 – 2/3 of muscle volume involved. 3 = greater than 2/3 of muscle volume involved. |
| Atrophy | Definition | Evident reduction of muscle volume based on the subjective assessment of the muscle’s cross-sectional area at the mid-belly, compared to other muscle groups. |
|  | Scoring | 0 = none (no loss of bulk). 1 = up to 1/3 loss of bulk. 2 = 1/3 – 2/3 loss of bulk. 3 = greater than 2/3 loss of bulk. |

Table 3 Inter-reader agreement for MRI scores of IIM.

| Muscle | ICC | 95% CI |
| --- | --- | --- |
| Vastus lateralis | .98 | .96, .99 |
| Rectus Femoris | .98 | .95, .99 |
| Vastus Medialis | .99 | .95, .99 |
| Vastus Intermedius | .96 | .92, .98 |
| Biceps brachii | .97 | .93, .98 |
| Biceps Femoris | .98 | .95, .99 |
| Semitendinosus | .98 | .96, .99 |
| Semimembranosus | .95 | .91, .98 |
| Vastus lateralis-stretched | .99 | .98, .99 |
| Rectus Femoris-stretched | .85 | .68, .92 |
| Vastus Medialis-stretched | .94 | .88, .97 |
| Vastus lateralis-stretched | .93 | .87, .97 |

Table 4. SWE correlations with clinical and muscle test variables for the IIM patients.

|  |  | VL | RF | VM | VI | BF | ST | SM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Age | Coeff | -.31 | -.22 | -.25 | -.18 | -.34 | -.05 | -.34 |
|  | p value | .15 | .3 | .26 | .42 | .12 | .82 | .13 |
| BMI | Coeff | -.08 | -.39 | .21 | -.52* | -.08 | -.2 | -.19 |
|  | p value | .72 | .07 | .33 | .013 | .73 | .38 | .41 |
| Disease duration (months) | Coeff | .01 | -.03 | .1 | -.17 | .26 | -.09 | .14 |
|  | p value | .98 | .91 | .66 | .46 | .24 | .68 | .53 |
| Serum creatinine (IU/L) | Coeff | .02 | .13 | .28 | .04 | -.3 | -.34 | -.2 |
|  | p value | .92 | .56 | .19 | .86 | .17 | .12 | .37 |
| ETGUGT, sit to stand (sec) | Coeff | -.69** | -.12 | -.17 | -.33 | -.32 | -.13 | -.04 |
|  | p value | .001 | .63 | .49 | .2 | .22 | .63 | .89 |
| ETGUGT, Gait initiation (sec) | Coeff | -.43 | -.34 | .22 | -.70** | -.44 | -.36 | -.27 |
|  | p value | .07 | .17 | .39 | .002 | .08 | .16 | .29 |
| ETGUGT, Walk 1 (sec) | Coeff | -.55* | -.33 | -.02 | -.70** | -.57* | -.07 | -.37 |
|  | p value | .019 | .19 | .93 | .002 | .017 | .8 | .14 |
| ETGUGT, Turn around (sec) | Coeff | -.49* | -.32 | .01 | -.60* | -.44 | -.19 | -.27 |
|  | p value | .040 | .2 | .95 | .010 | .08 | .46 | .3 |
| ETGUGT, Walk 2 (sec) | Coeff | -.42 | -.36 | .07 | -.64** | -.45 | .02 | -.3 |
|  | p value | .09 | .14 | .78 | .006 | .07 | .93 | .24 |
| ETGUGT, Slow down, stop (sec) | Coeff | -.27 | -.19 | .01 | -.4 | -.19 | .02 | -.16 |
|  | p value | .28 | .46 | .96 | .11 | .46 | .94 | .55 |
| ETGUGT, Total time (sec) | Coeff | -.56* | -.37 | .02 | -.64** | -.51* | -.17 | -.32 |
|  | p value | .017 | .14 | .94 | .006 | .036 | .52 | .21 |
| 30-sec Chair stand test | Coeff | .51* | -.07 | .37 | .14 | .4 | .08 | .31 |
|  | p value | .012 | .74 | .08 | .53 | .07 | .73 | .15 |
| Handgrip Strength (kg) | Coeff | .47* | -.02 | .01 | .05 | .62** | .27 | .45* |
|  | p value | .025 | .94 | .97 | .83 | .002 | .22 | .033 |
| Knee extension torque (Nm/kg) | Coeff | .33 | -.12 | -.06 | -.02 | .47* | .32 | .38 |
|  | p value | .15 | .62 | .79 | .95 | .034 | .16 | .09 |
| Knee flexion torque (Nm/kg) | Coeff | .42 | .11 | .01 | .19 | .60** | .2 | .31 |
|  | p value | .06 | .64 | .97 | .42 | .005 | .4 | .18 |
| Knee extension power (W/kg) | Coeff | .33 | -.09 | -.06 | .02 | .48* | .26 | .36 |
|  | p value | .14 | .71 | .8 | .94 | .033 | .26 | .12 |
| Knee flexion power (W/kg) | Coeff | .43 | .09 | .12 | .19 | .53* | .19 | .25 |
|  | p value | .05 | .71 | .6 | .42 | .015 | .42 | .3 |

Coeff= correlation coefficient.

*. Correlation is significant at the 0.05 level (2-tailed).

**. Correlation is significant at the 0.01 level (2-tailed).

Table 5. MRI muscle characteristics of the IIM patients.

| Muscle ¦ MRI score |  | Oedema |  | Fatty infiltration |  | Atrophy |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Count | % | Count | % | Count | % |
| Vastus lateralis (VL) | Normal | 12 | 52.2% | 8 | 34.8% | 15 | 65.2% |
|  | Mild | 5 | 21.7% | 9 | 39.1% | 2 | 8.7% |
|  | Moderate | 0 | 0% | 1 | 4.3% | 2 | 8.7% |
|  | Severe | 6 | 26.1% | 5 | 21.7% | 4 | 17.4% |
| Rectus Femoris (RF) | Normal | 13 | 56.5% | 13 | 56.5% | 15 | 65.2% |
|  | Mild | 4 | 17.4% | 7 | 30.4% | 4 | 17.4% |
|  | Moderate | 0 | 0% | 2 | 8.7% | 2 | 8.7% |
|  | Severe | 6 | 26.1% | 1 | 4.3% | 2 | 8.7% |
| Vastus Medialis (VM) | Normal | 14 | 60.9% | 12 | 52.2% | 17 | 73.9% |
|  | Mild | 4 | 17.4% | 6 | 26.1% | 2 | 8.7% |
|  | Moderate | 2 | 8.7% | 3 | 13.0% | 3 | 13.0% |
|  | Severe | 3 | 13.0% | 2 | 8.7% | 1 | 4.3% |
| Vastus Intermedius (VI) | Normal | 12 | 52.2% | 14 | 60.9% | 16 | 69.6% |
|  | Mild | 3 | 13.0% | 3 | 13.0% | 2 | 8.7% |
|  | Moderate | 3 | 13.0% | 2 | 8.7% | 1 | 4.3% |
|  | Severe | 2 | 21.7% | 4 | 17.4% | 4 | 17.4% |
| Biceps Femoris (BF) | Normal | 10 | 43.5% | 6 | 26.1% | 15 | 65.2% |
|  | Mild | 7 | 30.4% | 15 | 65.2% | 6 | 26.1% |
|  | Moderate | 5 | 21.7% | 2 | 8.7% | 1 | 4.3% |
|  | Severe | 1 | 4.3% | 0 | 0% | 1 | 4.3% |
| Semitendinosus (ST) | Normal | 13 | 56.5% | 7 | 30.4% | 13 | 56.5% |
|  | Mild | 7 | 30.4% | 10 | 43.5% | 6 | 26.1% |
|  | Moderate | 3 | 13.0% | 5 | 21.7% | 3 | 13.0% |
|  | Severe | 0 | 0% | 1 | 4.3% | 1 | 4.3% |
| Semimembranosus (SM) | Normal | 12 | 52.2% | 6 | 26.1% | 13 | 56.5% |
|  | Mild | 7 | 30.4% | 14 | 60.9% | 7 | 30.4% |
|  | Moderate | 3 | 13.0% | 2 | 8.7% | 3 | 13.0% |
|  | Severe | 1 | 4.3% | 1 | 4.3% | 0 | 0% |