Supplementary material and methods 1

Scheme S1. Synthesis of 18F-FPIA. Reaction conditions: a) 18F-fluoride, K222, KHCO3, DMSO, 120 C, 25 min; b) NaOH, then HCl.

Radiochemistry

Automated radiosynthesis of 18F-FPIA using the GE FASTlab™ platform.

An aqueous solution of 18F-fluoride in oxygen-18 enriched water (2 – 3 mL) was transferred from the cyclotron target to the hot-cell containing the FASTlab module with a sweep of argon gas. The 18F-fluoride was trapped on a Waters Sep-Pak QMA-carbonate light cartridge and eluted into the reactor with a solution (700 uL) containing MeCN (550 uL), H2O (150 uL), Kryptofix K222 (10.1 mg) and KHCO3 (1.7 mg). The 18F-fluoride was dried by evaporation in vacuo (120 °C,  -1000mBar) and a low flow of nitrogen (200 mBar) for 8 min. To the reactor containing dry 18F-fluoride was added 1 (5 - 7 mg) in anhydrous DMSO (500 µL) and heated at 120 °C for 15 min to produce 18F-2. After cooling to 60 °C, a solution of NaOH (1 mL, 2M) was added to the reactor and heated to 50 °C for 15 min to produce 18F-FPIA. The reactor was cooled and to the reactor containing crude reaction mixture was added HCl(aq) (1 mL, 2M). The contents of the reactor were transferred to an off-board bottle containing sodium phosphate buffer (17 mL, 58 mM) and phosphoric acid (500 µL, 1M) to achieve a final pH of 2.4 for semi-preparative HPLC purification. The HPLC injection loop (10 mL) was filled and 18F-FPIA was purified using an Agilent Eclipse XDB-C18 5µm, 250 × 9.4 mm column with a mobile phase containing 15% EtOH and 85% sodium dihydrogen phosphate buffer (58 mM, pH 4.5) at a flow rate of 4 mL/min. A radioactive peak corresponding to 18F-FPIA eluted at ca. 12 min, which was cut for 1 min into a sterile vial and formulated in water for injection (5 mL) to give a final buffered formulation pH 4.5 – 5.0. The final formulation was filtered through a PALL Acrodisc 0.2 m sterile filter into a final sterile vial (Figure S1).

|  | Manifold position |
| --- | --- |
| 18O water collection | 1 |
| QMA eluent | 2 |
| QMA-carb SPE | 4-5 |
| 18F-inlet | 6 |
| Reaction vessel | 7,8,25 |
| FPIA precursor (1) | 12 |
| NaOH | 14 |
| HCl | 16 |
| Water | 15 |
| HPLC dilution vial | 9 |
| HPLC loop | 10 |
| Product vial | Off cassette |

Figure S1. Schematic representations of the GE FASTLab™ cassette with HPLC purification.

Table S1. 18F-FPIA product description

|  | Criteria |
| --- | --- |
| Appearance pH Chemical puritya Kryptofix 222 Stable FPIA FPIA precursor (1) Hydroxypivalic acid Unknown impurities Residual solvents Acetonitrile DMSO Ethanol Radionuclide identity Radiochemical purity Radiochemical stability Sterility Endotoxina | Clear, colourless, practically free from particles 4.5 – 7.0 ≤ 2.2 mg ≤ 30 μg ≤ 12 μg ≤ 4.1 mg ≤ 50 mg 3 – 10% 105 – 115 min ≥ 95 % ≥ 95 %b Sterile 175 EU |

a in total administered dose
b at end of shelf-life

Supplementary Results 1

| A |  |
| --- | --- |
| B 18F-FPIA Parent | C Pivaloylcarnitine |
| D Radioactive metabolites in urine |  |

Fig S2. A) Putative mechanism for the in vivo synthesis of 18F-FPIA-carnitine based on a published mechanism for the in vivo metabolic fate of pivalic acid via activation by coenzyme A (11). B) Radio-HPLC chromatogram showing 18F-FPIA parent radiotracer, tR = 09:43 min:sec. C) UV-HPLC chromatogram of pivaloylcarnitine obtained from a commercial supplier, tR = 04:40 min:sec, λ = 210 nm. D) Representative radio-HPLC chromatogram showing a main radioactive metabolite (tR = 05:09 min:sec) in urine post PET scan. There is a 20 – 30 sec delay between the UV detector and γ-detector on the HPLC system used.