Supplemental Table 1. MRI acquisition parameters

| Image contrast | T1-W | DWI* | T2-W | T1-W+ |
| --- | --- | --- | --- | --- |
| Sequence | DIXON 3D FLASH | DW-EPI | HASTE | DIXON 3D FLASH |
| Imaging plane | Axial | Axial | Axial | Coronal |
| Number of slices per imaging station | 40 | 40 | 40 | 128 |
| Acquired Slice Thickness (mm) | 10 | 5 | 5 | 10 |
| Reconstructed Slice Thickness (mm) | 5 | 5 | 5 | 2.0 |
| Slice gap (mm) | 0 | 0 | 0 | 0 |
| FOV (mm) | 500 | 500 | 500 | 500 |
| Acquired voxels (mm x mm) | 2 x 1.6 | 3.9 x 3.9 | 2.0 x 2.0 | 1.9 x.17 |
| Reconstructed matrix | 640 | 256 | 512 | 288 |
| Reconstructed voxels (mm x mm) | 0.8 x 0.8 | 2.0 x 2.0 | 1.0 x 1.0 | 1.7 x 1.7 |
| Phase-encoding direction | AP | AP | AP | FH |
| TR (ms) | 6.62 | 6270 | 400 | 6.76 |
| TE (ms) | TE1 = 2.39, TE 2 = 4.77 | 67 | 92 | TE1 = 2.39, TE2 = 4.77 |
| Flip angle /  | 10 | 90 | 90; refocusing angle 180 | 10 |
| Number of signal averages | 1 | 2 (b50), 5 (b900) | 1 | 1 |
| Fat suppression | N/A | STIR | None | N/A |
| Acquisition Time (per station) | 10s | 3min 22s | 16s | 20s |
| *b-values: 50 & 900 s/mm2; + pre and post gadolinium contrast agent |  |  |  |  |