Supplementary materials

Online Resource 1: Study flowchart

Online Resource 2: Labeling and purification of PSMA-617 with 177Lu

177LuCl3 was obtained from ITG (Garching, Germany). GMP-grade PSMA-617 was obtained from ABX (Radeberg, Germany). The radiolabeling of PSMA-617 was performed on GRP synthesis module (Scintomics, Fürstenfeldbruck, Germany) using sterile and GMP-grade SC-105 kits. In brief, 4 mg gentisic acid and the PSMA-617 peptide were dissolved in 500 μL WFI and added to the reaction vessel. After addition of the 177LuCl3 in sodium acetate buffer and ascorbic acid the reaction was incubated at 100 °C for 20 minutes. After cooling down, the product was diluted to 16.5 ml with saline/DTPA to which 0.9 ml ethanol has been added. The radioactive solution was filtered through a 0.22 μm filter (Millex GV. Merck, Amsterdam, The Netherlands) and dispensed into a closed glass type I container. Microbiological monitoring in class C was performed during synthesis, filtration and dispensing. Assembling of the dispensing and filtration system was performed in a class A isolator with a class B airlock (in a class C background). The radiolabeled PSMA-617 was measured for total radioactivity in a calibrated activity dose calibrator prior to injection and injected within 6 hours after radiolabeling.

Online Resource 3: Imaging protocols

[68Ga]Ga-PSMA -11-PET/CT

Patients received [68Ga]Ga-PSMA -11-PET/CT prior to radioligand therapy. Imaging was performed 60 ± 10 minutes post-injection on a Biograph mCT system (Siemens Healthineers, Erlangen, Germany) scanning cranium to trochanter major. For the pelvis region, data were acquired using 4 minutes per bed position, whereas 3 minutes per bed position were applied for the other regions. PET data were reconstructed using ordinary Poisson ordered-subset expectation maximization with time-of-flight modelling (OP-OSEM-TOF) with 2 iterations and 21 subsets, matrix size of 400 (resulting in a cuboid-shape voxel size of approximately 2.0 mm3) and a smoothing Gaussian filter of 3 mm. The estimated reconstructed PET spatial resolution expressed as full width at half maximum (FWHM) was 6.3 mm. A low dose CT was performed (average dose length product (DLP) of 291 mGy·cm) and the CT data were reconstructed using 3.0 mm slice thickness and kernel B19f; the reconstructed transverse CT images had a voxel size of 1.0 x 1.0 x 3.0 mm3. For all PET images, standard corrections for CT-based attenuation, scatter, decay, and dead-time were performed.

[177Lu]Lu-PSMA SPECT/CT

Patients received a therapeutic activity of 3 GBq (3057 ± 38 MBq) [177Lu]Lu-PSMA. SPECT/CT imaging was performed at 1, 24, 48, 72 and 168 hours after administration on either a Symbia T16 or Symbia Intevo Bold system (Siemens Healthineers, Erlangen, Germany). SPECT/CT scans were acquired at three body regions: the pelvis, abdomen, and head-neck region (64 projections per detector, time per projection of 14 s, a 20% photon energy window at 208 keV, and dual-energy window for Compton scattering. SPECT data were reconstructed using OSEM reconstruction (Flash 3D with collimator detector response) using 4 iterations and 8 subsets, matrix size of 128 (resulting in a cuboid-shape voxel size of 4.8 mm3) and a smoothing Gaussian filter of 8.4 mm. The estimated reconstructed SPECT spatial resolution was 15 mm (FWHM). A low dose CT was performed (average DLP of 130 mGy·cm ) and the data were reconstructed using B31s kernel and 3.0 mm slice thickness resulting in a voxel size of 1.0 x 1.0 x 3.0 mm3. Of note, the SPECT image reconstruction approach takes into account corrections for scatter, CT-based attenuation, and dead-time

Online Resource 4: Parametrization of the uptake curves depending on tissue type

For kidneys and liver, a mono-exponential clearance with an effective half-life Teff,1 up to 72 h and thereafter a second mono-exponential clearance with an effective half-life Teff,2 was observed (Figure 1A). Mathematically, the uptake curve is given as:

For salivary glands, kinetics was assessed for the whole organ instead of separate glands. An instant uptake was observed and its value remained almost constant with an (average) value of U0 up to 24 h. Thereafter, a mono-exponential clearance with an effective half-life Teff was found (Figure 1B), according to

For lesions, a linear increase was observed with a slope, α = U(1)t1 (in %/h), to a maximum uptake, Umax, followed by a mono-exponential decay with an effective half-life Teff (Figure 1C), with the intercept of both functions at time tmax,according to:

Online Resource 5: Projections of the functions to estimate the TIAC based solely PET uptake

The projection of the functions given in Online Resource 10 led to the following expressions for TIAC in each organ or lesion based solely on the projected PET uptake ULu(tPET). In the following, the uptake value ULu(tPET) given in the equations are non-percentage values:

Online Resource 6: Uncertainty analysis

Data collection:

  - Quantitative SPECT at 5 time-points, within 45 minute scan-time

  - Quantitative PET at 1 time point

  - Organ specific tracer kinetics based on PET

Uncertainty in SPECT cumulated activity

- Drawing of VOIs in SPECT data over organs and tumor lesions to determine counts:

- Large spherical VOI over lesions with background correction; error: 10%

- CT based VOI over organs; error 5%

- Fit to Time-Activity Curve on SPECT

- Mono-exponential fit between activity on each subsequent SPECT time point, considered an approximation of trapezoid integration

- Integration over time of the TAC, to determine the time-integrated activity Ã on SPECT (reference absorbed dose)

- Trapezoid integration method; error:

δAA2=iδA(ti)A(ti)2

Uncertainty in PET cumulated activity

- Drawing of VOIs in PET data over organs and tumor lesions to determine counts. Error in counts δSUV

- Large spherical VOI over lesions with background correction; error: 10%

- CT based VOI over organs; error 5%

- Determination of average organ specific tracer kinetics based on SPECT. Uncertainty δk is based on the SD of the patient average kinetics

  - Lesions: 30%

  - Organs: 10%

- Determination of PET time-integrated activity Ã. Error:

δAA2=δSUVSUV2+δkk2

General uncertainty

- Error in SPECT camera specific calibration factor for 177Lu: 5% [Peters, 2020]

CF = 10.6 ± 0.5 cps/MBq

- Determination of lesion and organ volume:

- Lesion and organ volume based on PET/CT and diagnostic CT [Jentzen, 2015];

Voxelisation and resolution error volume with lesion diameter d and voxel size a: 	δVV2=9 δdd2=1.5ad2 [Gear, 2018]

- Determine volume specific lesion S-factor with power-function on S-values spheres S=81.7V-0.988; error:

δSS2=0.988δVV2

- Absorbed dose calculation with MIRD equation D=A×S; error:

δDD2=δAA2+δSS2

Online Resource 7: patient characteristics and administered activities

| Patient # | Age (yrs) | Weight (kg) | Administerd activity (GBq) |
| --- | --- | --- | --- |
| 1 | 61 | 89 | 3.0 |
| 2 | 62 | 91 | 3.1 |
| 3 | 77 | 77 | 3.1 |
| 4 | 66 | 96 | 3.1 |
| 5 | 68 | 78 | 3.0 |
| 6 | 65 | 86 | 3.0 |
| 7 | 71 | 75 | 3.1 |
| 8 | 71 | 59 | 3.1 |
| 9 | 69 | 90 | 3.0 |
| 10 | 62 | 85 | 3.0 |

Online Resource 8: Individual kidney volumes and absorbed dose from 177Lu-PSMA-SPECT and 68Ga-PSMA-PET

| Patient nr. | Kidney volume (ml) |  | Absorbed dose (Gy/GBq)a ± error |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Right | Left | SPECT | PET unscaled | PET scaledb |
| 1 | 183 | 200 | 0.42 ± 0.04 | 0.80 ± 0.09 | 0.37 ± 0.04 |
| 2 | 213 | 196 | 0.88 ± 0.03 | 1.16 ± 0.14 | 0.53 ± 0.06 |
| 3 | 184 | 201 | 0.67 ± 0.01 | 1.70 ± 0.20 | 0.77 ± 0.09 |
| 4 | 186 | 179 | 0.57 ± 0.05 | 1.56 ± 0.18 | 0.71 ± 0.08 |
| 5 | 157 | 151 | 0.80 ± 0.07 | 2.18c ± 0.26 | 0.99 ± 0.12 |
| 6 | 187 | 187 | 0.51 ± 0.04 | 1.41 ± 0.17 | 0.64 ± 0.08 |
| 7 | 183 | 164 | 1.10 ± 0.10 | 2.43 ± 0.29 | 1.11 ± 0.13 |
| 8 | 149 | 148 | 1.03 ± 0.03 | 2.41 ± 0.29 | 1.10 ± 0.13 |
| 9 | 200 | 253 | 0.88 ± 0.13 | 1.49 ± 0.18 | 0.68 ± 0.08 |
| 10 | 164 | 175 | 1.19 ± 0.09 | 2.35 ± 0.28 | 1.07 ± 0.13 |
| Mean ± SD | 181 ± 19 | 185 ± 29 | 0.81 ± 0.25 | 1.75 ± 0.54 | 0.80 ± 0.25 |
| Median (range) | 184 (149 – 213) | 183 (148 – 253) | 0.84 (0.42 – 1.19) | 1.63 (0.80 – 2.43) | 0.74 (0.37 – 1.11) |

a: Absorbed dose was determined as the mean absorbed dose for the left and right kidney. b: determined as the product between the absorbed dose for PET unscaled and scaling factor S = 2.21. c: Pre-treatment 68Ga-PSMA-PET not available, therefore absorbed dose was determined on post-treatment 68Ga-PSMA-PET.

Online Resource 9: Individual liver volumes and absorbed dose from 177Lu-PSMA-SPECT and 68Ga-PSMA-PET

| Patient nr. | Liver volume (ml) | Absorbed dose (Gy/GBq) ± error |  |
| --- | --- | --- | --- |
|  |  | SPECT | PET |
| 1 | 2382 | 0.091 ± 0.003 | 0.086 ± 0.01 |
| 2 | 1902 | 0.17 ± 0.01 | 0.16 ± 0.02 |
| 3 | 1979 | 0.15 ± 0.01 | 0.20 ± 0.02 |
| 4 | 1809 | 0.15 ± 0.01 | 0.18 ± 0.02 |
| 5 | 1799 | 0.17 ± 0.01 | 0.18a ± 0.02 |
| 6 | 1798 | 0.15 ± 0.01 | 0.15 ± 0.02 |
| 7 | 1627 | 0.15 ± 0.01 | 0.20 ± 0.02 |
| 8 | 1185 | 0.24 ± 0.01 | 0.25 ± 0.02 |
| 9 | 1862 | 0.18 ± 0.01 | 0.17 ± 0.02 |
| 10 | 1513 | 0.18 ± 0.02 | 0.21 ± 0.02 |
| Mean ± SD | 1786 ± 296 | 0.16 ± 0.04 | 0.18 ± 0.04 |
| Median (range) | 1804 (1185 – 2382) | 0.16 (0.091 – 0.24) | 0.18 (0.086 – 0.25) |

a: Pre-treatment 68Ga-PSMA-PET not available, therefore absorbed dose was determined on post-treatment 68Ga-PSMA-PET.

Online Resource 10: Individual parotid gland volumes and absorbed dose from 177Lu-PSMA-SPECT and 68Ga-PSMA-PET

| Patient nr. | Parotid gland volume (ml) |  | Absorbed dose (Gy/GBq)a ± error |  |
| --- | --- | --- | --- | --- |
|  | Right | Left | SPECT | PET |
| 1 | 61 | 47 | 0.81 ± 0.04 | 1.09 ± 0.13 |
| 2 | 27 | 26 | 0.70 ± 0.02 | 0.89 ± 0.11 |
| 3 | 26 | 23 | 1.15 ± 0.04 | 1.22 ± 0.15 |
| 4 | 31 | 37 | 0.75 ± 0.07 | 1.11 ± 0.13 |
| 5 | 16 | 19 | 1.49 ± 0.06 | 1.54b ± 0.19 |
| 6 | 40 | 38 | 1.27 ± 0.04 | 1.22 ± 0.15 |
| 7 | 26 | 30 | 0.74 ± 0.03 | 0.41 ± 0.05 |
| 8 | 20 | 23 | 1.24 ± 0.05 | 1.60 ± 0.19 |
| 9 | 27 | 28 | 1.06 ± 0.05 | 0.80 ± 0.10 |
| 10 | 40 | 39 | 0.38 ± 0.01 | 0.54 ± 0.06 |
| Mean ± SD | 31 ± 12 | 31 ± 9 | 0.96 ± 0.32 | 1.04 ± 0.37 |
| Median (range) | 27 (16 – 61) | 29 (19 – 47) | 0.94 (0.38 – 1.49) | 1.10 (0.41 – 1.60) |

a: Absorbed dose was determined as the mean absorbed dose for the left and right parotid gland. b: Pre-treatment 68Ga-PSMA-PET not available, therefore absorbed dose was determined on post-treatment 68Ga-PSMA-PET.

Online Resource 11: Individual submandibular gland volumes and absorbed dose from 177Lu-PSMA-SPECT and 68Ga-PSMA-PET

| Patient nr. | Submandibular gland volume (ml) |  | Absorbed dose (Gy/GBq)a ± error |  |
| --- | --- | --- | --- | --- |
|  | Right | Left | SPECT | PET |
| 1 | 17 | 14 | 0.57 ± 0.03 | 1.05 ± 0.13 |
| 2 | 11 | 11 | 1.30 ± 0.05 | 1.49 ± 0.18 |
| 3 | 9 | 7 | 1.00 ± 0.04 | 1.35 ± 0.17 |
| 4 | 17 | 16 | 0.93 ± 0.09 | 1.29 ± 0.16 |
| 5 | 10 | 11 | 1.57 ± 0.06 | 1.93b ± 0.24 |
| 6 | 14 | 12 | 1.48 ± 0.06 | 1.31 ± 0.16 |
| 7 | 10 | 11 | 0.98 ± 0.04 | 0.60 ± 0.07 |
| 8 | 11 | 12 | 1.10 ± 0.05 | 1.50 ± 0.18 |
| 9 | 9 | 10 | 1.68 ± 0.08 | 1.35 ± 0.17 |
| 10 | 13 | 12 | 0.92 ± 0.03 | 1.25 ± 0.15 |
| Mean ± SD | 12 ± 3 | 12 ± 2 | 1.15 ± 0.33 | 1.31 ± 0.32 |
| Median (range) | 11 (9 – 17) | 11 (7 – 16) | 1.05 (0.57 – 1.68) | 1.33 (0.60 – 1.93) |

a: Absorbed dose was determined as the mean absorbed dose for the left and right submandibular gland. b: Pre-treatment 68Ga-PSMA-PET not available, therefore absorbed dose was determined on post-treatment 68Ga-PSMA-PET.