Table S1: Technical information for PET/CT imaging procedures of the five different institutions.

| PET Parameters | Saarland University Medical Center Homburg | University Hospital Ludwig Maximilian University of Munich | Johannes Gutenberg University Medical Center (in collaboration with the Radiology and Nuclear Medicine Practice at the Köln Triangle, Cologne) | Charité Universitätsmedizin Berlin | University Medical Center Würzburg |
| --- | --- | --- | --- | --- | --- |
| Tracer | [68Ga]Ga-PSMA-11 | [18F]PSMA-1007 | [68Ga]Ga-PSMA-11 [18F]PSMA-1007 | [68Ga]Ga-PSMA-11 | [68Ga]Ga-PSMA I&T [18F]PSMA-1007 |
| Time: injection to imaging [min] | 60 min | 60 min | [68Ga]Ga-PSMA-11: ~ 60 min [18F]PSMA-1007: ~ 90 min | 60 min | [68Ga]Ga-PSMA I&T: ~ 60 min [18F]PSMA-1007: ~90 min |
| PET/CT scanner Model | Siemens Biograph mCT 40 | GE Discovery 690 | Philips Gemini TF 16 | Philips Gemini TF 16 | Siemens Biograph mCT 128 flow |
| Pixel matrix size | 200x200 | 256x256 | 168x168 | 144x144 | 200x200 |
| Acquisition time/bed position [min] | 2 min | 2 min | 2 min. | 3 min | 2 min |
| Extended field of view [cm] | 21.4 cm | 15.4 cm | 21.8 cm | 18.0 cm | 78.0 cm |
| Slice thickness [mm] | 3.0 mm | 3.27 mm | 3.0 mm | 4.0 mm | 3.0 mm |
| Reconstruction Parameters |  |  |  |  |  |
| No. of iterations | 3 | 3 | 2 | 3 | 3 |
| No. of subsets | 21 | 36 | 14 | 33 | 21 |
| Reconstructed slice thickness [mm] | 5.0 mm | 5.0 mm | 4.0 mm | 4 mm | 5.0 mm |
| Low dose CT / Diagnostic CT | Low dose CT | Diagnostic CT | Low-dose CT | Low dose CT | Low-dose CT |