# Supplementary information

Supplementary Table 1: Acquisition and reconstruction parameters for 124I and [18F]FDG PET/CT imaging. [18F]FDG PET/CT acquisition and reconstruction were compliant with EARL 18F standards 1. Scan parameters are the same for both isotopes unless otherwise specified.

| Parameter | Values |  |
| --- | --- | --- |
| Scanner | Vereos (Philips Healthcare) | Omni Legend 32 (GE Healthcare) |  |
| Sensitivity (cps/kBq) | 5.2 [1] | 47.6 [2] |
| Acquisition duration per bed position [18F]FDG [124I]NaI | 3 min. 4 min. | 1 min. 4 min. |
| Energy window | 449.7-613.2 keV | 435-580 keV |
| Reconstruction Method Iterations Subsets Smoothing | OSEM 5 15 5.5 mm FWHM Gaussian | VPDH 4 12 7.0 mm FWHM Gaussian |
| Corrections applied | attenuation, decay, scatter, randoms | attenuation, decay, scatter, randoms |
| Low-dose CT Exposure time Pitch Collimation Tube voltage Tube current | 0.6 s 0.83 0.625 mm 120 kV variable, 20-80 mA | 0.5 s 0.98 0.625 mm 120 kV variable, 20-80 mA |

OSEM: ordered-subsets expectation-maximization; VPDH: Vue Point HD; FWHW: full width at half maximum

Supplementary Table 2: Acquisition parameters for 124I whole body counting.

| Parameter | Value |
| --- | --- |
| Scanner | Discovery 670 Pro (GE Healthcare) |
| Collimator | None |
| Detector configuration | L-mode, start angle 279° |
| Acquisition duration | 1 min (anterior) + 1 min (posterior) |
| Energy window | 390-650 keV |
| Matrix | 256x256 |
| Zoom | 1.0 |
| Distance detector-patient | 2 m |

Supplementary Table 3: Tumor characteristics and details regarding prior 131I therapy per patient

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tumor histology | oncocytic | oncocytic | oncocytic | oncocytic | oncocytic | oncocytic | papillary | papillary | papillary |
| Time from initial diagnosis to RAI-R disease | 2 years | 12 years | 4 years | 4 months | 1 year | 2 months | 2.5 years | 6 years | 3 years |
| Number of prior 131I therapy cycles | 4 | 2 | 3 | 1 | 2 | 1 | 5 | 5 | 4 |
| Uptake on last 131I post-therapy scan? | yes | mixed | mixed | no | no | mixed | mixed | yes | mixed |

Supplementary Table 4: Change from baseline in dosimetric results of the target lesions per subgroup (estimated for 131I based on 124I data)

| Parameter | Median change (IQR) |  |  |  |
| --- | --- | --- | --- | --- |
|  | wk0-6 |  | wk0-12 |  |
|  | Oncocytic | Papillary | Oncocytic | Papillary |
| No. of patients | 5 | 2 | 6 | 2 |
| No. of target lesions | 12 | 5 | 15 | 5 |
| Absorbed dose | 27% (-47-146%) | 130% (89-301%) | 113% (18-212%) | 71% (12-212%) |
| Uptake at 24h p.i. | -50% (-75-20%) | 21% (-39%-461%) | -1% (-74-110%) | 22% (-86-380%) |
| Residence time | -52% (-77- -26%) | 40% (21-182%) | -29% (-74-12%) | 10% (-37-33%) |

IQR: interquartile range, p.i.: post-ingestion

Supplementary Figure 1: Lesion dose per administered activity (LdpA) per target lesion in time (estimated for 131I based on 124I data).

Supplementary Figure 2: Residence time per target lesion in time (estimated for 131I based on 124I data).

Supplementary Figure 3: Uptake (%) at 24 hours post-ingestion per target lesion in time (estimated for 131I based on 124I data).

# References

1.	Rausch, I., et al., Performance Evaluation of the Vereos PET/CT System According to the NEMA NU2-2012 Standard. J Nucl Med, 2019. 60(4): p. 561-567.

2.	Kennedy, J.A., et al., An extended bore length solid-state digital-BGO PET/CT system: design, preliminary experience, and performance characteristics. Eur J Nucl Med Mol Imaging, 2024. 51(4): p. 954-964.