Supplementary Table 2. Comparison of qualitative image quality scores for the abdominal portal venous phase CT series between MD-iodine, 55 keV, and 70 keV

| Reader, Parameters, Subgroups (Distribution of scores 1/2/3/4/5 given as percentages) |  | MD-iodine | 55 keV | 70 keV | p value |
| --- | --- | --- | --- | --- | --- |
| Reader 1 |  |  |  |  |  |
|  | Lesion conspicuity | 8.0/20.0/34.0/28.0/10.0ab | 0/6.0/4.0/14.0/76.0b | 6.0/4.0/14.0/48.0/28.0 | < 0.001 |
|  | Image noise | 0/22.0/42.0/26.0/10.0ab | 0/0/6.0/14.0/80.0 | 0/0/2.0/2.0/96.0 | < 0.001 |
|  | Pancreatic and surrounding structures | 0/14.0/60.0/18.0/8.0ab | 0/0/0/2.0/98.0 | 0/0/2.0/10.0/88.0 | < 0.001 |
|  | Overall image quality | 0/26.0/40.0/24.0/10.0ab | 0/0/0/4.0/96.0 | 0/0/2.0/6.0/92.0 | < 0.001 |
| Reader 2 |  |  |  |  |  |
|  | Lesion conspicuity | 0/12.0/32.0/34.0/22.0a | 0/2.0/6.0/24.0/68.0b | 0/10.0/24.0/60.0/6.0 | < 0.001 |
|  | Image noise | 0/4.0/36.0/46.0/14.0b | 0/0/18.0/78.0/4.0b | 0/0/0/2.0/98.0 | < 0.001 |
|  | Pancreatic and surrounding structures | 0/6.0/14.0/32.0/48.0b | 0/2.0/4.0/28.0/66.0b | 4.0/6.0/48.0/38.0/4.0 | < 0.001 |
|  | Overall image quality | 0/10.0/32.0/50.0/8.0a | 0/2.0/4.0/16.0/78.0b | 2.0/8.0/42.0/46.0/2.0 | < 0.001 |
| Reader 3 |  |  |  |  |  |
|  | Lesion conspicuity | 0/4.0/64.0/30.0/2.0ab | 0/0/4.0/58.0/38.0 | 0/0/6.0/40.0/54.0 | < 0.001 |
|  | Image noise | 0/0/62.0/38.0/0ab | 0/0/8.0/70.0/22.0b | 0/0/0/44.0/56.0 | < 0.001 |
|  | Pancreatic and surrounding structures | 0/0/56.0/42.0/2.0ab | 0/0/4.0/56.0/40.0 | 0/0/4.0/32.0/64.0 | < 0.001 |
|  | Overall image quality | 0/0/68.0/32.0/0ab | 0/0/0/64.0/36.0 | 0/0/2.0/38.0/60.0 | < 0.001 |

keV kiloelectron Volt, MD-iodine material density iodine

Post hoc pairwise multiple comparisons procedure with the Dunn-Bonferroni test showed a statistically significant (p <0.05) difference between means when compared with 55 keV (a), and 70 keV (b)