Supplementary Material:

Description of bioanalytical method of lurbinectedin

Lurbinectedin plasma concentrations were measured using a validated ultra-performance liquid chromatography tandem mass spectrometry in human K3-EDTA plasma, adding deuterated lurbinectedin (PM01183 –d4 or PM040038) as internal standard. The sample extraction was performed using supported liquid extraction. Chromatographic separation was performed using a C18, 1.7 µm, 50 x 2.1 mm column and gradient elution with 0.1% ammonium hydroxide in water, acetonitrile. Detection was by triple quadrupole mass spectrometry system with electrospray ionization in positive ion mode; desolvation temperature was established at 400 ºC, capillary voltage at 1 kV, and lurbinectedin and internal standard parent to product transition (m/z) were 767.7 to 273.0 and 771.7 to 277.1, respectively. The calibration curves for lurbinectedin displayed good linearity over the concentration range of 0.1 to 50 ng/mL. The intra and inter day precisions ranged from 2.7 to 12.9% and from 5.1 to 10.7%, respectively. Similarly, the within and between day accuracy (bias) ranged from -10 to 12% and -5 to 6%, respectively.