Hyphae tip isolation procedure and RNA extraction:

- streak the yeast cells in the middle of the YEG (2% D-glucose, 1% yeast extract (Scharlau), 10% gelatin) plates with sterile inoculation loop

- incubate the plates at 30°C for 10 days *

- excise the tips of the growing hyphae with sterile scalpel

- transfer the slices to a new, sterile petri dish **

- incubate the slices at 37°C for 5 min ***

- carefully transfer the melted gelatin-hyphae suspension to empty 2ml Eppendorf tube(s) up to 500 µl

- add immediately room temperature DEPC water up to 2 ml and suspend it. This step will dilute the gelatin and prevent to re-solidify

- centrifuge the tubes for 1 min at 2000 rpm, then discard the supernatant

- wash the hyphae with 1 ml ice-cold DEPC water

- centrifuge the tubes for 1 min at 2000 rpm

- repeat step 8) and 9) for two times

- prepare 2 ml Eppendorf tubes with 100-150 µl glass-beads

- add 750 µl TES solution to the pelleted hyphae then transfer the suspension to the Eppendorf tube with glass-beads

- add immediately 750 µl acidic phenol to the suspension and vortex the tubes briefly

- incubate the tubes at 65°C in dry/water block for 1 hour

- vortex the tubes for 10-15 seconds in every 10 minutes

- chill the tubes on ice then vortex for 15-20 seconds

- centrifuge the samples at 10000 rpm for 10 minutes at 4°C

- transfer the water phase to a new 2 ml Eppendorf tube then add equal amount of acidic phenol then vortex the tubes for 15-20 seconds

Optional: use phase-lock tubes (MaXtract High Density, Cat. No:129056) to efficiently eliminate the phenol (DO NOT VORTEX THE TUBES)

- centrifuge the samples at 10000 rpm for 10 minutes at 4°C

- transfer the water phase to a new 2 ml Eppendorf tube then add 0.5 volume Na-acetate (3M) and 2.5 volume absolute ethanol

- mix the tubes by inverting then incubate the samples at -20°C for 30-40 min

Pause point: incubation can be extended to overnight

- centrifuge the tubes at 10000 rpm for 15 min at 4°C then discard the supernatant

- add 100 µl 70% ethanol (made with DEPC water, DO NOT VORTEX)

- quick spin the tubes (few seconds) and discard the 70% ethanol (be careful to not touch the pellet with the pipette tip)

- dry the pellet in sterile box

- add 100 µl of DEPC water to the pellet and incubate the tubes at 65°C for 1 min

Store RNA samples at -70°C

TES solution: (Lyne et al. 2003).

- 10 mM TRIS-HCl pH:7,5

- 10 mM EDTA pH:8,0

- 0,5% SDS

Use DEPC treated water to make all your solutions.

* the incubation time depends on the strain/species

** in case of low amount of slices the Eppendorf tubes is better than petri dishes

*** gelatin slices will quickly melt at 37°C, but the time depends on the amount of slices and the percentage of gelatin in the media

Materials and equipment:

- Eppendorf tubes (2ml)

- Scalpel (or any sterile knife)

- 37°C incubator

- Dry or water block with 65°C temperature

- Vortex

- Ice

- Glass-beads (200-300 µm)

- DEPC water

- Phenol/Chloroform/Isoamyl-alcohol (25:24:1 mixture, pH:5,2)

- Ethanol (absolute)

- Sodium-acetate (3M)