Supplement to:  An assessment on the incremental value of high-resolution magnetic resonance imaging to identify culprit plaques in atherosclerotic disease of the middle cerebral artery

Imaging parameters of high-resolution, multi-contrast MRI

3D time-of-flight (TOF) magnetic resonance angiography (MRA) was performed with the following parameters: TR/TE =29/3.4 ms, field of view (FOV) =24×21 cm2, slice thickness =1.2 mm, NEX =1, matrix =384×192, sequence duration =4.78 mins. The parameters for T2-weighted images were: TR/TE =2883/50 ms, FOV =10×10 cm2, NEX =3, matrix =320×256, echo-train length (ETL) =20, slice thickness =2 mm, and sequence duration =3.85 mins; the parameters for T1-weighted images were as follows: TR/TE =567/16 ms, FOV =10×10 cm2, NEX =2, matrix =320×256, ETL =6, slice thickness =2 mm, and sequence duration =5 mins; for contrast enhancement, a dose of 0.2 mmol/kg Gd-DTPA was injected by power injector at a speed of 2 ml/s followed by 15 ml of physiological saline. Twelve slices were acquired for T1-, T2- and contrast enhanced T1-weighted sequences with a 2.5-mm slice gap and 0.39-mm in-plane resolution.

Table I. Comparisons of plaque characteristics between culprit lesions and the ones on the contralateral in symptomatic patients

|  | Culprit (n=26) | Contralateral non-culprit (n=26) | p-value |
| --- | --- | --- | --- |
| TOF-defined luminal stenosis, median [IQR] (%) | 71.1 [56.1, 79.2] | 48.8 [32.4, 72.3] | 0.016* |
| No stenosis (<29%), n (%) | 0 (0) | 5 (19.2) | 0.051 |
| Mild (30-49%), n (%) | 0 (0) | 8 (30.8) | 0.004* |
| Moderate (50-69%), n (%) | 11 (42.3) | 5 (19.2) | 0.132 |
| Severe (70-99%), n (%) | 10 (38.5) | 5 (19.2) | 0.220 |
| Occlusion (100%), n (%) | 5 (19.2) | 3 (11.6) | 0.703 |
| Plaque burden | 0.90 [0.85, 0.94] | 0.75 [0.66, 0.88] | 0.0002* |
| Minimum luminal area (mm2) | 1.26 [0.65, 1.95] | 2.70 [2.31, 2.93] | 0.0002* |
| Plaque volume (mm3) | 49.16 [25.54, 67.40] | 27.82 [19.20, 44.29] | <0.0001* |
| Plaque length (mm) | 10 [8, 12] | 6 [4, 9.5] | <0.0001* |
| Normalized T1 signal intensity >1.0, n (%) | 1 (3.8) | 0 (0) | 1 |
| Normalized T2 signal intensity >1.0, n (%) | 2 (7.7) | 4 (15.4) | 0.668 |
| Normalized CE-T1 signal intensity >1.0, n (%) | 10 (38.5) | 2 (7.7) | 0.019* |

Table II. Plaque characteristics of the 10 lesions from patients with chronic infarcts

| Stenosis (%) | 64.7 [51.8, 79.1] |
| --- | --- |
| Plaque burden (%) | 91 [83, 96] |
| Minimum luminal area (mm2) | 1.06 [0.39, 2.17] |
| Plaque volume (mm3) | 46.1 [23.9, 61.9] |
| Plaque length (mm) | 8 [6, 12] |
| Enhancement | 2 (20%) |