Supplementary Digital Content

Supplemental Table 1. 3T breast MRI imaging protocol

| Sequences | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Series Descriptor | LOC | Calibration | Axial T2 FSE | Axial DWI | Axial T1 | 3D DISCO |
| Plane |  |  | Axial | Axial | Axial | AXIAL |
| Phase FOV |  |  | 1 | 0.95 | 1 | 1 |
| Field of View (cm) | 48 | 48 | 30–36 | 30–36 | 30–36 | 30–36 |
| Slice Thickness (mm) | 12 | 10 | 3 | 3.9 | 1 | 1.1 |
| Gap (mm) |  |  | 3 | 3.9 | 1 | 1.1 |
| Saturation |  |  |  | Fat Special |  |  |
| TE1 / TE2 | 80 |  | 100 | Minimum |  | Minimum |
| TR | Min |  | 2500–6000 | 2000–17000 |  |  |
| TI TIME (Prep) |  |  |  | 210 |  |  |
| Flip Angle |  |  | 111 | 90 | 10 | 12 |
| Bandwidth (kHz) | 83kHZ | 62.5 | 83 | 250 | 62.5 | 166.67 |
| NEX |  |  | 1 | 1 (b = 0); 4 (b = 800) |  |  |
| Direction Dir. |  |  |  | ALL |  |  |
| # of Directions |  |  |  | 3 |  |  |
| b-values |  |  |  | 0, 800 |  |  |
| Matrix size | 256x256 |  | 512x512 | 256x256 | 512x512 | 512x512 |
| Frequency Direction |  | A/P | A/P | A/P | A/P | A/P |

Abbreviations: FSE: Fast Spin Echo; DISCO: Differential subsampling with cartesian ordering; A/P: Anterior-posterior; NEX: number of averages

Supplemental Table 2. Average of ADC mean values for each reader in benign and malignant lesions.

|  | Reader 1 | Reader 2 | Reader 3 |
| --- | --- | --- | --- |
| Benign lesions | 1.45±0.25 | 1.47±0.27 | 1.40±0.29 |
| Malignant lesions | 1.05±0.16 | 1.08±0.17 | 0.91±0.20 |

*Units: × 10-3 mm2/s for all the values

Supplemental Table 3. DW image quality score mean values for all the readers across the b-values

| b-value | Reader 1 | Reader 2 | Reader 3 | Mean |
| --- | --- | --- | --- | --- |
| 800 | 2.13 ± 0.69 | 2.15 ± 0.69 | 2.27 ± 0.5 | 2.18 ± 0.07 |
| 1000 | 2.12 ± 0.69 | 2.13 ± 0.69 | 2.26 ± 0.5 | 2.17 ± 0.07 |
| 1200 | 2.12 ± 0.69 | 2.13 ± 0.68 | 2.26 ± 0.5 | 2.17 ± 0.07 |
| 1500 | 2.12 ± 0.69 | 2.11 ± 0.67 | 2.26 ± 0.5 | 2.16 ± 0.08 |
| 1800 | 2.10 ± 0.69 | 2.03 ± 0.64 | 2.25 ± 0.49 | 2.13 ± 0.11 |

Visual grading image quality score (1– bad quality, 2– average, 3– good quality)

Supplemental Table 4. Lesions missed with DWI across all readers and different b-values

| Reader | b-value | Benign | Malignant | Total of lesions |  |
| --- | --- | --- | --- | --- | --- |
| 1 | b-800 | 10 (7.6 mm; 5–20 mm) | 8 (7.8 mm; 5–14 mm) | 18 |
|  | b-1000 | 10 (7.6 mm; 5–20 mm) | 8 (7.8 mm; 5–14 mm) | 18 |
|  | b-1200 | 10 (7.6 mm; 5–20 mm) | 8 (7.8 mm; 5–14 mm) | 18 |
|  | b-1500 | 13 (7.4 mm; 5–20 mm) | 10 (7.6 mm; 5–14 mm) | 23 |
|  | b-1800 | 17 (7.3 mm; 5–20 mm) | 11 (8.2 mm; 5–14 mm) | 28 |
| 2 | b-800 | 12 (7.4 mm; 5–20 mm) | 10 (10 mm; 5–22mm) | 22 |
|  | b-1000 | 13 (7.3 mm; 5–20 mm) | 10 (10 mm; 5–22mm) | 23 |
|  | b-1200 | 14 (7.1 mm; 5–20 mm) | 10 (10 mm; 5–22 mm) | 24 |
|  | b-1500 | 18 (7.3 mm; 5–20 mm) | 12 (9.8 mm; 5–22 mm) | 30 |
|  | b-1800 | 27 (7.9 mm; 5–20 mm) | 14 (12.5 mm; 5–50 mm) | 41 |
| 3 | b-800 | 13 (7.8 mm; 5–22 mm) | 6 (8 mm; 5–14 mm) | 19 |
|  | b-1000 | 13 (7.8 mm; 5–22 mm) | 6 (8 mm; 5–14 mm) | 19 |
|  | b-1200 | 13 (7.8 mm; 5–22 mm) | 6 (8 mm; 5–14 mm) | 19 |
|  | b-1500 | 19 (8.6 mm; 5–20 mm) | 6 (8 mm; 5–14 mm) | 25 |
|  | b-1800 | 27 (8.7 mm; 5–22 mm) | 11 (12.9 mm; 5–50 mm) | 38 |

Supplemental Table 5. False negative cases with DWI

| Reader | Histology | Tumor type | Mean size mm (range) | Total of false negatives |
| --- | --- | --- | --- | --- |
| 1 | 13 IDC 2 IDC+DCIS 3 DCIS 3 ILC | 13 masses 1 NMLE/1mass 1 NMLE/2 masses 1 NMLE/2 masses | 14 (6–31) 30.5 (11–50) 8.3 (7–10) 11 (8–16) | 21 |
| 2 | 10 IDC 5 IDC+DCIS 4 DCIS 4 ILC | 10 masses 2 NMLE/3masses 2 NMLE/2 masses 1 NMLE/3 masses | 14.4 (6–31) 21.8 (11–50) 11.5 (7–21) 11 (8–16) | 23 |
| 3 | 10 IDC 6 IDC+DCIS 3 DCIS 4 ILC | 10 masses 2 NMLE/4masses 2 NMLE/1 mass 1 NMLE/3 masses | 12.9 (6–28) 21 (11–50) 12.6 (7–21) 11 (8–16) | 23 |

Supplemental Table 6. Number of false positive cases stratified by lesion size

| Lesions ≤ 10 mm |  |  |  |  |  | Lesions > 10 mm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCE-MRI |  |  | mpMRI |  |  | DCE-MRI |  |  | mpMRI |  |  |
| r1 | r2 | r3 | r1 | r2 | r3 | r1 | r2 | r3 | r1 | r2 | r3 |
| 8 | 14 | 6 | 6 | 11 | 5 | 2 | 3 | 2 | 1 | 1 | 1 |