| Volcano | NZVEWS threat level1 | Last known eruption/unrest (defined as VAL>1)2 | Number of data collection locations as of January 20243,4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Seismic5 | Acoustic | GNSS | Camera | DOAS | Environmental |  |
|  |  |  |  |  |  |  |  | Automatic | Manually Collected |
| Auckland Volcanic Field | High | ~1400/nil | 9 | 0 | 1 | 0 | 0 | 0 | 0 |
| Northland | Low | ????/nil | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Okataina | High | 1886/nil | 11 | 0 | 11 | 0 | 0 | 2 | 12 |
| Raoul Island | Low/Medium | 2006/2006 | 2 | 1 | 1 | 1 | 0 | 1 | 5 |
| Rotorua | Medium | ????/nil | 9 | 0 | 9 | 0 | 0 | 0 | 6 |
| Ruapehu | High | 2007/present | 10 | 10 | 8 | 3 | 4 | 1 | 3 |
| Taranaki | High | ~1790/nil | 5 | 3 | 3 | 1 | 0 | 1 | 0 |
| Taupō | High | ~1800 BP/2023 | 12 | 0 | 11 | 0 | 0 | 0 | 24 |
| Tongariro-Ngauruhoe | High | 2012/2015 | 14 | 14 | 11 | 4 | 2 | 7 | 22 |
| Tūhua/Mayor Island | Low/Medium | ????/nil | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Whakaari/White Island | High | 2019/present | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| Whakaari/White Island before 20196 | High | 2019/present | 2 | 2 | 2 | 4 | 2 | 1 | 93 |
| Total7 |  |  | 76 | 28 | 55 | 10 | 6 | 12 | 73 |

1 Miller et al, 2022.

2 Last eruption based on GNS Science (2024). Unrest based on volcanic alert level of one or higher.

3 Seismic, acoustic, GNSS, and DOAS are determined as the number of locations within a set distance of the volcano. For cone volcanoes, 15 km; for calderas and volcanic fields, 30 km. For seismic, acoustic, and GNSS, a few collection locations are counted for more than one volcano. Automatically and manually collected use the same distance cutoff but are validated against which volcano is normally considered to be relevant for the monitored feature. Camera is determined by which volcano is in a camera’s field of view; one camera is counted twice. Locations at Whakaari/White Island that have failed to operate or have not been used for manual collection have been removed.

4For comprehensive, up-to-date information on data collection refer to https://www.geonet.org.nz/data/network/sensor/search.

5 Strong motion is excluded, broadband and short-period are not distinguished.

6 GNS Science have been unable to visit Whakaari/White Island to maintain monitoring equipment since the eruptive activity in 2019. As of February 2024, all on-island monitoring has ceased, and monitoring is limited to visual and aerial gas flux observations. GNS Science hopes to re-establish on-island monitoring in the future.

7 Total is as of January 2024, and does not include locations active before the 2019 Whakaari/White Island eruption.