Appendix A: Life cycle inventory

| Manufacturing |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input | Quantity D1 | Quantity D2 | Quantity H1 | Quantity H2 | Quantity R1 | Dataset | Source |
| ABS | - | 64 g | - | - | - | market for acrylonitrile-butadiene-styrene copolymer {GLO} | FTIR |
| HDPE | 17.883 g | 1.8721 g | - | 1.3 g | - | market for polyethylene, high density, granulate {GLO} | FTIR |
| Metal | 0.5901 g | 13.9 g | 1.229 g | 0.621 g | 1.714 g | self-modeled | 1, 2 |
| PC | 138.6 g | - | 55.2 g | 34.3 g | - | market for polycarbonate {GLO} | FTIR |
| PEEK | - | - | 0.275 g | 0.118 g | 0.398 g | market for polyphenylene sulfide {GLO} | FTIR, 7 |
| PET | 5.1 g | - | - | - | - | market for polyethylene terephthalate, granulate, amorphous {GLO} | FTIR |
| Polyamide | - | 58.6302 g | 0.407 mg | - | 0.407 mg | market for glass fibre reinforced plastic, polyamide, injection moulded {GLO} | FTIR, 8 |
| POM | - | - | - | 0.2895 g | - | market for nylon 6 {RER} | FTIR |
| PP | 7.6 g | 0.4231 g | - | - | - | market for polypropylene, granulate {GLO} | FTIR |
| PVC | - | - | - | 0.062 g | - | market for polyvinylchloride, bulk polymerized {GLO} | FTIR |
| Synthetic rubber | 30.949 g | 7.0862 g | 0.5667 g | 15.6975 g | 0.5667 g | market for synthetic rubber {GLO} | FTIR |
| Injection moulding | 200.132 g | 73.3814 g | 56.0417 g | 51.767 g | 0.9647 g | injection moulding {RER} | - |
| Metal working | 0.5901 g | 13.9 g | 1.229 g | 0.621 g | 1.714 g | metal working, average for chromium steel product manufacturing {RER} | - |
| Packaging plastic | 31.9 g | 22.4704 g | 8.9150 g | 12.7215 g | 0.5358 g | market for packaging film, low density polyethylene {GLO} | Weighed |
| Packaging paper | 9.3 g | 9.6302 g | 3.6432 g | 2.7 g | - | market for kraft paper {RER} | Weighed |
| Manuals and/or stickers | - | - | 1.6569 g | 1.7665 g | 2.1433 g | market for printed paper {GLO} | Weighed |
| Radiation sterilization | 0.2408 kWh | 0.1780 kWh | 0.0673 kWh | 0.0819 kWh | - | market for electricity, medium voltage {location in country of manufacturing} | 9, 10 |
| Transport | A :0.0482 tkm B: 0.0422 tkm C: 0.0130 tkm | A: 0.1479 tkm B: 0.2074 tkm C: 0.1757 tkm | A: 0.0395 tkm B: 0.0562 tkm C: 0.0460 tkm | A: 0.0747 tkm B: 0.0722 tkm C: 0.0781 tkm | A: 0.0030 tkm B: 0.0042 tkm C: 0.0035 tkm | Market for transport, freight, lorry >32 metric ton, EURO6 {RER} | - |

| Use phase (only applicable to trocars H1, H2 and R1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Input | Quantity Hospital A | Quantity Hospital B | Quantity Hospital C | Dataset | Source |
| Electricity (gas) | 0.0310 kWh | 0.0385 kWh | - | electricity production, natural gas, conventional power plant {NL} | - |
| Electricity (general) | - | - | 0.1552 kWh | market for electricity, medium voltage {NL} | - |
| Electricity (solar) | - | 0.0385 kWh | 0.0010 kWh | electricity production, photovoltaic, 3kWp slanted-roof installation, single-Si, panel, mounted {NL} | - |
| Electricity (wind) | 0.0864 kWh | - | - | electricity production, wind, 1-3MW turbine, onshore {DK} | - |
| Packaging | 102 g | - | 94 g | market for textile, nonwoven polypropylene {GLO} | - |
| Pressurized air | 0.0067 m3 | 0.0050 m3 | 0.0103 m3 | market for compressed air, 1000 kPa gauge {RER} | - |
| Soap 1 | 0.2094 ml | 0.1675 ml | - | self-modeled | 3 |
| Soap 2 | 8.7938 ml | 7.035 ml | 17.2286 ml | self-modeled | 4 |
| Soap 3 | 0.5025 ml | 0.402 ml | - | self-modeled | 5 |
| Steam | 1.2707 kg | 0.8332 kg | 1.6872 kg | market for steam, in chemical industry {RER} | - |
| Transport | - | 0.0166 km | - | market for transport, passenger car, electric {GLO} | - |
| Wastewater | -14.1301 L | -9.3943 L | -17.2287 L | market for wastewater, average {Europe without Switzerland} | - |
| Water (deionized) | 1.3607 L | 1.0886 L | 7.9563 L | market for water, deionized {Europe without Switzerland} | - |
| Water (softened) | 12.76 L | 8.2982 L | 9.2552 L | market for water, completely softened {RER} | - |
| End-of-life |  |  |  |  |  |
| Input | Quantity Hospital A | Quantity Hospital B | Quantity Hospital C | Dataset | Source |
| 50L medical waste bin | D1: 12.7218 g D2: 9.2479 g H1: 3.6298 g H2: 4.2330 g R1: 0.1698 g | D1: 12.7218 g D2: 9.2479 g H1: 3.6298 g H2: 4.2330 g R1: 0.1698 g | D1: 12.7218 g D2: 9.2479 g H1: 3.6298 g H2: 4.2330 g R1: 0.1698 g | self-modeled | RIVM |
| Medical waste incineration | D1: 213.4439 g D2: 155.1595 g H1: 60.9009 g H2: 71.0210 g R1: 2.8489 g | D1: 213.4439 g D2: 155.1595 g H1: 60.9009 g H2: 71.0210 g R1: 2.8489 g | D1: 213.4439 g D2: 155.1595 g H1: 60.9009 g H2: 71.0210 g R1: 2.8489 g | self-modeled | 6 |
| Plastic garbage bag | D1: 0.0236 g D2: 0.0187 g H1: 0.0666 g H2: 0.0683 g R1: 0.0599 g | D1: 0.0236 g D2: 0.0187 g H1: 0.0666 g H2: 0.0683 g R1: 0.0599 g | D1: 0.0236 g D2: 0.0187 g H1: 0.0666 g H2: 0.0683 g R1: 0.0599 g | market for packaging film, low density polyethylene {GLO} | - |
| Plastic waste recycling | - | - | D1: -31.2182 g D2: -22.4835 g H1: -3.1246 g H2: -4.5844 g R1: -0.5478 g | market for waste polyethylene, for recycling, sorted {Europe without Switzerland} | - |
| Regular waste incineration | D1: 40.5236 g D2: 32.1193 g H1: 14.2817 g H2: 17.2563 g R1: 2.7389 g | D1: 40.5236 g D2: 32.1193 g H1: 14.2817 g H2: 17.2563 g R1: 2.7389 g | D1: 9.3054 g D2: 9.6358 g H1: 11.1570 g H2: 12.6719 g R1: 2.1911 g | market for process-specific burdens, municipal waste incineration {Europe without Switzerland} | - |
| Transport | D1: 0.0458 tkm D2: 0.0337 tkm H1: 0.0135 tkm H2: 0.0158 tkm R1: 0.0010 tkm | D1: 0.0527 tkm D2: 0.0384 tkm H1: 0.0151 tkm H2: 0.0176 tkm R1: 0.0008 tkm | D1: 0.0248 tkm D2: 0.0184 tkm H1: 0.0078 tkm H2: 0.0092 tkm R1: 0.0006 tkm | market for transport, freight, lorry >32 metric ton, EURO6 {RER} | - |

References
(1) What is surgical steel? The role of stainless in healthcare. 2022.

(2) Stolarski, T.; Nakasone, Y.; Yoshimoto, S. Engineering Analysis with ANSYS Software. In Elsevier eBooks, Elsevier, 2006.

(3) neodisher Dekonta Med; 2024. https://www.drweigert.com/target/SDB/neodisher-Dekonta-Med_SDB_NLnl_PN4212_2024-03-11.PDF.

(4) neodisher MediClean; 2023. https://www.drweigert.com/target/SDB/neodisher-MediClean_SDB_NLnl_PN4043_2022-08-25.PDF.

(5) neodisher MediKlar; 2023. https://www.drweigert.com/target/SDB/neodisher-MediKlar_SDB_NLnl_PN4045_2022-03-18.PDF.

(6) Zhao, W.; van der Voet, E.; Huppes, G.; Zhang, Y. Comparative life cycle assessments of incineration and non-incineration treatments for medical waste. The International Journal of Life Cycle Assessment 2009, 14 (2), 114-121. DOI: 10.1007/s11367-008-0049-1.

(7) JiangSu JunHua HPP Co; 2025. https://www.junhuapeek.com/what-plastics-are-similar-to-peek.html

(8) Jiskar; 2024. https://snijlab.nl/en/blogs/news-en/pom-delrin-vs-pa6-nylon-engineering-plastics-compared?srsltid=AfmBOooXpwRN4iLXKJ8LQn66Su2ZvG5KWG0Jrk5TLNZY7sLshyZNv6zF

(9) European Medicines Agency; 2023. https://www.ema.europa.eu/system/files/documents/other/qa-xray-sterilisation_final_en.pdf

(10) Geltser I.; 2023. https://indico.fnal.gov/event/60757/attachments/170337/230837/01_02_Geltser_In-line%20e-Beam%20Sterilization%20-%20Terumo%20-%20Industry%20Needs%202023%2009%2020.pdf?