Supplementary Materials

Table S1. Comparison of instruments employed in studies covered by the present study.

| Instrument/Logger | Parameters Measured | Sensor Type | Accuracy | Sampling/Response Time | Mounting Type | Source |
| --- | --- | --- | --- | --- | --- | --- |
| TESTO 480 / 440 | Air temperature (Ta), Relative humidity (RH), Wind speed (v), Globe temperature (Tg), Surface temperature (Ts) | Vane probe, Humidity/temp probe, Waterproof surface probe, Type K thermocouple globe (150 mm) | Ta: ± 0.2 °C; RH: ± 1.0%; v: ± 0.03 m/s; Tg: ± 1 °C | 5 s steps or 1 min intervals | Backpack or Tripod (1.4–1.7 m) | Ali Smail et al., 2024 Chokhachian et al., 2018 Kim & Kim, 2024 Kim et al., 2022 |
| Kestrel 5400 Heat Stress Tracker | Ta, RH, v, Tg, UTCI | Integrated meteorological sensors | Ta: < ± 0.5 °C; RH: < ± 2%; v: < ± 3%; Tg: < ± 1.4 °C | 2 s to 5 s steps | Backpack or Foldable tripod | Deng et al., 2023 Lam et al., 2024 Peng et al., 2022 |
| Trolley-based Environmental Monitoring System (TEMS) | Ta, Tg, RH, v, PM, Ts, GPS, SVF | Type K thermocouple, HOBO MX1101, Graywolf PC-3500, Kimo CTV110, Flir T335 | Ta: ± 0.21 °C; RH: ± 2%; v: ± 3%; Ts: ± 2 °C | 1 s (Datalogger); 1 min (PM) | Trolley (sack truck) | Lam et al., 2025 |
| MWS 1 & 2 | Ta, Tg, v, RH, Radiation (SW/LW), GPS, Atmospheric pressure (P), PM, sound, UTCI | Tinkerforge and Meter sensors; Gills Maximet GMX500, Apogee SN500, Campbell globe | Ta: ± 0.1 to 0.3 °C; RH: ± 2%; v: 0.3 to 1.1 m/s; Tg: ± 0.3 to 0.7 °C | 5 s | Backpack or All-terrain cart | Silva et al., 2025 |
| Pedestrian-carried / Mobile station | Ta, RH, v, Radiation (SW/LW) | Vaisala HMP155A, Gill 2D WindSonic, Hukseflux NR01 (x3) | Ta: ± 0.2 °C; v: ± 2%; Radiation: ± 10% | 0.5 Hz sampling; 18–20 s response | Modified bicycle trailer or climate cart | Requena-Ruiz et al., 2025 Siret et al., 2025 |
| PLEMS (ESP32-based) | Ta, RH, Tg, v, CO2, Illuminance, Sound pressure, GPS | Multi-sensor (AHT10, DS18B20, ANBR-1, MQ-135, BH1750-FVI, KY-038) | Ta: ± 0.3 °C; RH: ± 2%; Tg: ± 0.5 °C; v: < ± 5%; CO2: ± 5% | 60 s cycle | Backpack | Krüger et al., 2024 |
| TRM-ZS2-BX Weather Station | Ta, RH, v, Solar radiation (G), Tg | PTS-3, EC-A2 ultrasonic, TBB-2 net radiation, PTWD-2A globe | Ta: ± 0.2 °C; RH: ± 2%; v: ± 2%; G: ± 1 W/m2; Tg: ± 0.2 °C | 1 min intervals | Stationary sidewalk (1.5 m) | Huang et al., 2025 |
| Delta OHM HD 32.2 / 32.3 | Ta, RH, v, Tg, Sound level | Multi-probe (50 mm globe); SMART SENSOR AS-824 | Ta: ± 0.21 °C; RH: ± 2.5%; v: 0.1 m/s resolution | 15 s to 1 min | Backpack or Hand-carried | Li et al., 2025 Qi et al., 2021 Speak & Salbitano, 2022 |
| MeteoTracker (IoTopon Srl) | Ta, RH, P | Capacitive–resistive sensors | Ta: ± 0.5 °C; RH: ± 2%; P: ± 50 Pa | $\geq$ 1 Hz | Magnetic base or String | Barbano et al., 2024 |
| Davis Vantage Pro 2 Plus | Ta, RH, v, G | Silicon diode, Film capacitor, Solid state magnetic sensor | Ta: ± 0.3 °C; RH: ± 2%; v/G: ± 5% | 1 min steps | Wheeled tripods | Kim et al., 2022 |
| Atmos 41 | Ta, RH, v, G | Integrated weather station | Ta: ± 0.6 °C; RH: ± 1.5%; v: ± 3%; G: ± 5% | 1 min | Wheeled tripods (1.1 m) | Kim & Kim, 2024 |
| LI-COR LI-1500 | G, GPS tagging | Pyranometer LI-200R, RADIONOVA RF Antenna | G: 0.183 W/m2; GPS: 2.5 m | 5 s steps | Backpack | Chokhachian et al., 2018 |

Table S1 (continued). Comparison of instruments employed in studies covered by the present study.

| Instrument/Logger | Parameters Measured | Sensor Type | Accuracy | Sampling/Response Time | Mounting Type | Source |
| --- | --- | --- | --- | --- | --- | --- |
| MS1 & MS2 (Mobile Stations) | Ta, RH, v, Tg, G | Delta OHM HD32.3, HD2102.2, HD52.3D17 | Not in source | Not in source | Backpack or Cart | Deng et al., 2025 |
| HOBO U23 Pro v2 / U12-014 | Ta, RH, Tg | Type-T Copper-Constantan thermocouple (40 mm ball) | Ta: ± 0.2 °C; Tg: ± 1.5 °C | 1 min planned stops | Walking tripod (1.6 m) | Chàfer et al., 2022 |
| Wearable system / Station | Ta, CO2, RH, v, P, G, Illuminance, PM | SHT35, TDS0037 (NDIR), calibrated sensors | Ta: ± 0.1 °C; CO2: ± 2% | t90 < 8 to 30 s | Wearable or Backpack | Cureau et al., 2022 Grapas et al., 2025 |
| Multi-parametric station | Ta, RH, v | Not in source | Ta: ± 0.3 °C; RH: ± 2%; v: ± 3% | 10 s | Van | Grapas et al., 2025 |
| Helmet/Backpack Sensors | Ta, RH, P, G, v, CO2, CO, VOC, Ts | BME280, SP-215, CV7-OEM, DYNAMENT MSH-P-CO2, FLIR vue Pro R | Ta: ± 0.5 °C; v: 0.13 m/s; CO2: ± 2%; Ts: ± 5 °C | 1 s to 30 s (t90) | Helmet or Backpack | Pigliautile & Pisello, 2018 |
| Stop-and-go station (Parison 2023) | Ta, RH, Tg, v | Sheltered Pt100, Capacitive hygrometer, Hot-wire anemometer | Ta: ± 0.1 °C; RH: ± 1.5%; Tg: ± 0.15 °C | 10–20 min stabilization | Mobile station | Parison et al., 2023 |
| MaRTy (mobile platform) | Ta, RH, Radiation (SW/LW), v | Six-direction sensors | Not in source | Not in source | Human-biometeorological cart | Dzyuban et al., 2022 |
| MoBio Platform | Ta, RH, v, G, Fluxes | Integrated (Hukseflux, Gill, Vaisala) | Not in source | 2 s intervals | Pedestrian-carried / Push-cart | Ouyang et al., 2025 |
| HR-500 | PM1, PM2.5, PM10, CO2 | Not in source | PM: 0.1 g/m3; CO2: 1 ppmv | 1 s | Backpack | Lam et al., 2024 |
| TES 1339R lux meter | Illuminance (lx) | Lux meter | ± 3% of readings | 1 s | Backpack | Deng et al., 2023 Lam et al., 2024 |
| IoT MF-300 | Ta, GPS | Temperature probe | Ta: ± 0.2 °C; GPS: 3.0 m | 1 s | Backpack (cycling) | Žgela et al., 2024 |
| Custom-made bike station | Ta, Ts, RH | HD 500 thermologger | Not in source | 1 measurement/s | Bicycle | Jato-Espino et al., 2025 |
| Fluke TI400 | Ts | Thermal imaging camera | Range -20 to 80 °C | Site-specific scans | Handheld | Jato-Espino et al., 2025 |
| Backpack station (helper) | Ta, RH, v, Tg | Tailor-made globe (38 mm); Thermocouple wire | Ta: ± 0.2 °C; RH: ± 1.0%; v: ± 0.03 m/s; Tg: ± 1.1 °C | 1 s interval | Backpack | Lau et al., 2019 |
| Self-built device | Ta, RH, P, Ts, G, Tg | Pyranometer; GNSS | VDI 2023 standards | 1 s recording | Pedestrian (GoPro stick) | Gallacher & Boehnke, 2025 |
| Campbell Scientific CR800 | Ta, RH, Tg, v, Illuminance | CS215, CT100, Ultrasonic 2D, Skye lux meter | Not in source | Not in source | Aluminum trolley (1.75 m) | Vasilikou & Nikolopoulou, 2020 |
| Gill GMX501 System | Ta, RH, v, G | Sonic anemometer, CNR4 net-radiometer, Apogee thermistor | Not in source | 15 s to 5 min | Stationary tripod or Cart | Aleksandrowicz & Pearlmutter, 2023 |
| Sound Level Meters | Sound pressure level / Decibel | AS-K8; C.A. 832 | ± 1.5 dB | 1 s | Backpack | Cureau et al., 2022 Lam et al., 2024 |