Implications for sedimentary transport processes in southwestern Africa – A combined zircon morphology and age study including extensive geochronology databases

Andreas Gärtner, Mandy Hofmann, Johannes Zieger, Anja Sagawe, Rita Krause, Marika Stutzriemer, Subani Gesang, Axel Gerdes, Linda Marko, Cristiano Lana, Ulf Linnemann

Table S1: Settings for the instruments used in the geochronological Laboratory (GeoPlasmaLab Dresden) of the Senckenberg Naturhistorische Sammlungen Dresden, (Excimer Laser, ASI RESOlution SE S155, 193 nm, excimer) and (ICP-MS, Thermo Fisher, Element 2 XR).

| ICP-MS | Finnigan Element 2 XR |
| --- | --- |
| Forward Power | 1300 W |
| Gas flow rate | 15.06 l min-1 (cool gas) |
|  | 0.74 l min-1 (auxiliary gas) 1.0 l min-1 (sample gas) |
| Scan mode | triple |
| Scanned masses | 202, 204, 206, 207, 208, 232, 235, 238, |
| Mass resolution | 300 |
| Dead time | 18 ns |
| Oxide UO+/U+ | < 1% |
| Dwell time | 4 ms |
| Settling time | ≤ 1 ms/amu |
| Number of scans | 1500 |
| Background | 15 s |
| Ablation time | 30 s |
| Integration time | 1.4 s (=25 scans) |
| Laser system | asi RESOlution SE S155 193 nm, excimer |
| Nominal spot diameter | 25-35 µm (unknowns) |
|  | 25-35 µm (reference zircon) |
| Carrier gas | 0.5 l min-1 He |
|  | 1.0 l min-1 Ar 0.07 l min-1 N2 (pure) |
| Laser settings Repetition rate Laser energy Fluence | 5 Hz 25 % attenuator 1.3-1.6 mJ cm-2 |
| Laser pit depth | ~ 1.7 µm (spot) |
| Laser cell | 2 volume laser cell Laurin Technic S155 size 155 × 105 mm |
| Sensitivity | 6 × 106 counts/pg U |