Supporting Material for

Deformation behavior and seismic characteristics of sandy facies Opalinus Clay during triaxial deformation under dry and wet conditions

Valerian Schuster1*, Erik Rybacki1, Audrey Bonnelye1,2 and Georg Dresen1, 3

1Helmholtz Centre Potsdam, GFZ German Research Centre for Geosciences, 14473 Potsdam, Germany
2GeoRessources Laboratory, Université de Lorraine, 54000 Nancy, France
3Institute for Earth and Environmental Science, University of Potsdam, 14476 Potsdam, Germany
*Corresponding author: Valerian Schuster (valerian.schuster@gfz-potsdam.de)

Contents of this file

Table 1 and 2
Figures S1 to S3

Table 1 Summary of mechanical and acoustic data

| Sample Orientation | Sample Condition | pc (MPa) | E1 (GPa) | E2 (GPa) | ν12 (-) | ν21 (-) | ν23 (-) | Ap (-) | Total Number of AE | Differential Stress at ci (MPa) | Differential Stress at cd (MPa) | σmax (MPa) | εmax (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P | dry | 50 | - | 13.9 |  | 0.13 | 0.09 | 0.22a 0.18p | 248 | 94 | 133 | 152 | 0.0315 |
| P | wet | 50 | - | 9.3 |  | 0.18 | 0.17 | 0.06a 0.08p | 90 | 44 | 99 | 99 | 0.0314 |
| S | dry | 50 | 8.9 | - | 0.11 | - | - | 0.28 | 261 | 78 | 178 | 148 | 0.0345 |
| S | wet | 50 | 6.1 | - | 0.28 | - | - | 0.11 | 39 | 39 | 65 | 80 | 0.0373 |

pc confining pressure, E Young’s modulus, ν Poisson’s ratio, Ap initial P-wave velocity anisotropy coefficient (c.f., Chapter 4.1 of the main text), a azimuthal direction, p polar direction, ci stress at onset of microcracking, cd stress at onset of net dilation, σmax peak strength, εmax total axial strain at peak strength

Table 2 Summary of compositional data of different shales sorted by increasing QFP content

|  | Clay (vol%) | Quartz (vol%) | Carbonate (vol%) | Feldspar (vol%) | Pyrite (vol%) | TOC (vol%) | Porosity (vol%) | QFP (vol%) | Cb (vol%) | Clay + TOC + Porosity (vol%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Opalinus Clay Shaly Faciesa | 62.3 | 7.1 | 8.9 | 3.9 | 0.4 | 1.6 | 15.7 | 11.5 | 8.9 | 79.6 |
| Whitby Mudstoneb | 50.6 | 11.6 | 9.5 | 0.5 | 3.6 | 11.2 | 13.0 | 15.7 | 9.5 | 74.8 |
| Tournemire Shalec | 61.0 | 12.8 | 12.3 | 2.4 | 1.1 | 1.9 | 8.5 | 16.4 | 12.3 | 71.4 |
| Callovo-Oxfordian Shaled | 43.5 | 15.3 | 25.4 | 2.5 | 1.4 | 1.8 | 10.0 | 19.3 | 25.4 | 55.3 |
| Muderong Shalee | 57.4 | 22.5 | 0.8 | 2.4 | 0.9 | 0.9 | 15.0 | 25.8 | 0.8 | 73.3 |
| Norwegian Sea Shalef | 51.7 | 26.8 | 0.8 | 2.3 | 0.8 | 3.6 | 14.0 | 29.9 | 0.8 | 69.3 |
| Northwest Shelf Shaleg | 37.4 | 34.6 | 10.9 | 3.3 | 0.3 | 0.4 | 13.0 | 38.2 | 10.9 | 50.9 |
| Opalinus Clay Sandy Faciesa | 33.1 | 36.4 | 8.8 | 9.3 | 0.5 | 0.7 | 11.1 | 46.2 | 8.8 | 44.9 |
| Officer Basin Shaleh | 37.7 | 17.9 | 6.5 | 28.5 | 1.1 | 1.5 | 6.7 | 47.6 | 6.5 | 45.9 |
| Sichuan Basin Longmaxi Shalei | 16.4 | 60.3 | 16.3 | 0.0 | 0.0 | 4.9 | 2.0 | 60.3 | 16.3 | 23.4 |

QFP Quartz + Feldspar + Pyrite, Cb Carbonates, TOC Total organic carbon
 aSchuster et al.2022; bHouben et al. 2016; cTremosa et al. 2012; dKlinkenberg et al. 2009; eDewhurst et al. 2002; fDewhurst et al. 2011; gDelle Piane et al. 2009, 2011; hKuila et al. 2011, iLi et al.

Fig. S1 Photographs of undeformed samples (length 10 cm, diameter 5 cm) of the sandy facies of Opalinus Clay prepared parallel (a) and perpendicular (b) to bedding

Fig. S2 Backscattered electron microscope images of undeformed Opalinus Clay of the sandy facies showing dominant mineral phases as well as the microstructural differences between grain supported sand layers (a) and matrix supported clay layers (b)

Fig. S3 Sensor map with the position of piezoelectric P-wave transducers (bold circle: transmitter; open circle: receiver) and strain gauges projected on the sample surface (a). Schematic side view of the prepared samples, showing the used nomenclature in this technical note (b)