Electronic supplementary information

Quantitative titanium imaging in fish tissues exposed to titanium dioxide nanoparticles by laser ablation-inductively coupled plasma-mass spectrometry

Cristian Suárez-Oubiña*1, Annarosa Mangone2, Lorena C. Giannossa2, Laura Nuñez-González3, Paloma Herbello-Hermelo1, Pilar Bermejo-Barrera1, Antonio Moreda-Piñeiro1

Table 1. LA-ICP-MS operating conditions.

| ICP-MS parameter (units) | Value |
| --- | --- |
| Instrument | NexIon 2000 |
| Nebulizer gas flow (L min-1) | 0.90 - 1.00 (daily optimised) |
| Auxiliary gas flow rate (L min-1) | 1.2 |
| Plasma gas flow (L min-1) | 15 |
| ICP RF power (W) | 1600 |
| Isotopes monitored | 25Mg, 26Mg, 27Al, 31P, 42Ca 46Ti, 47Ti, 48Ti, 49Ti |
| Dwell time (ms) | 50 (25Mg, 26Mg, 27Al, 31P, 42Ca) 10 (46Ti, 47Ti, 48Ti, 49Ti) |
| LA-ICP-MS parameter (units) | Value |
| Laser | Nd:YAG (213 nm) |
| Tygon® interface tube | 3.5 m length – 1/16 inch i.d. |
| Ablation mode | Scan line |
| He flow rate (mL min-1) | 900 |
| Spot Size (µm) | 110 |
| Fluence (J cm-2) | 0.40 |
| Scan speed (µm s-1) | 50 |
| Repetition rate (Hz) | 20 |
| Ablation depth (µm) | 0 |
| Laser warm-up (s) | 10 |
| Laser wash-out (s) | 15 |

Table S2. Total titanium and TiO2 NPs in fish tissues from exposed sea bream (N1) and unexposed sea bream (N2).

| Sample tissue | sp-ICP-MS (particles g-1) | SD | ICP-MS (µg g-1) | SD |
| --- | --- | --- | --- | --- |
| Kidney N1 | 3.88×106 | 1.78×105 | 0.83 | 0.20 |
| Kidney N2 | --- | --- | 0.12 | 0.04 |
| Liver N1 | 6.67×106 | 3.65×105 | 0.51 | 0.05 |
| Liver N2 | --- | --- | 0.21 | 0.17 |
| Muscle N1 | 8.14×105 | 8.90×104 | 0.23 | 0.15 |
| Muscle N2 | --- | --- | < LOD | --- |

Figure S1. Representative TEM image of citrate-45nm TiO2 NPs (JEOL JEM 1010 transmission electron microscope operating at 100 kV).

Figure S2. Thick paraffin slides (5.0 µm) of fish tissues obtained after sample pre-treatment: (A) under UV light and (B) natural light. Sample description: kidney tissue from sea bream exposed at 1.5 mg kg-1 TiO2 NPs (45 nm) for 90 days.

Figure S3. Exported images from ActiveView™ software (dark line under sample tissue) using laser energies for (A) sample fish tissue and (B) paraffin polymer under several laser fluencies.

Figure S4. Exported images from ActiveView™ software illustrating the heterogeneity of the embedded samples (A,B) and the borders between paraffin-embedded tissue / paraffin-free tissue and the glass holder (C).

Figure S5. LA-ICP-MS time vs intensity plots for a kidney tissue obtained at scanning rates of 30 µm s-1 (A), 60 µm s-1 (B), and 90 µm s-1 (C)

Figure S6. LA-ICP-MS time vs intensity plots for a kidney tissue by recording 48Ti (red), 46Ti (blue), and 26Mg (black). Scale referred only to 46Ti intensities: scan implying the presence of TiO2 NPs (A and B) and the absence of TiO2 NPs (C) in tissues from exposed sea bream.

Figure S7. Images for a kidney tissue from a sea bream specimen exposed to 45 nm TiO2 NPs (dietary exposure at 1.5 mg kg-1) for 90 days tissues:  sample image under UV light (A), 26Mg map intensities (B), 48Ti map concentrations (C), 48Ti map intensities (D), 46Ti map concentrations (E), 46Ti map intensities (F).

Figure S8. Images for a kidney tissue from an un-exposed sea bream specimen (sampling at 75 days):  sample image under UV light (A), 26Mg map intensities (B), 48Ti map concentrations (C), 48Ti map intensities (D), 46Ti map concentrations (E), 46Ti map intensities (F).

Figure S9. Images for a liver tissue from a sea bream specimen exposed to 45 nm TiO2 NPs (dietary exposure at 1.5 mg kg-1) for 75 days tissues:  sample image under UV light (A), 26Mg map intensities (B), 48Ti map concentrations (C), 48Ti map intensities (D), 46Ti map concentrations (E), 46Ti map intensities (F).

Figure S10. Images for a liver tissue from an un-exposed sea bream specimen (sampling at 45 days):  sample image under UV light (A), 26Mg map intensities (B), 48Ti map concentrations (C), 48Ti map intensities (D), 46Ti map concentrations (E), 46Ti map intensities (F).

Figure S11. Images for a muscle tissue from a sea bream specimen exposed to 45 nm TiO2 NPs (dietary exposure at 1.5 mg kg-1) for 75 days tissues:  sample image under UV light (A), 26Mg map intensities (B), 48Ti map concentrations (C), 48Ti map intensities (D), 46Ti map concentrations (E), 46Ti map intensities (F).

Figure S12. Images for a liver tissue from an un-exposed sea bream specimen (sampling at 45 days):  sample image under UV light (A), 26Mg map intensities (B), 48Ti map concentrations (C), 48Ti map intensities (D), 46Ti map concentrations (E), 46Ti map intensities (F).