Comparative Life Cycle Assessment of Sequential Chemical and Electrochemical Processes for the Treatment of Industrial Textile Wastewater

Luis Miguel Salazar Sogamoso1, Miguel-Ángel Gómez-García1, Izabela Dobrosz-Gómez2*

1Universidad Nacional de Colombia - Sede Manizales; Departamento de Ingeniería Química; Facultad de Ingeniería y Arquitectura; GI-PRISMA: Grupo de Investigación en Procesos Reactivos Intensificados y Materiales Avanzados; LM&PR: Laboratorio de Materiales y Procesos Reactivos; Campus La Nubia, Manizales, 170003, Colombia. lumsalazarso@unal.edu.co, magomez@unal.edu.co

2Universidad Nacional de Colombia - Sede Manizales; Departamento de Física y Química; Facultad de Ciencias Exactas y Naturales; GI-PRISMA: Grupo de Investigación en Procesos Reactivos Intensificados y Materiales Avanzados; LM&PR: Laboratorio de Materiales y Procesos Reactivos; Campus La Nubia, Manizales, 170003, Colombia. idobrosz-gomez@unal.edu.co

*Corresponding author: (57) 606 8879300, idobrosz-gomez@unal.edu.co, Universidad Nacional de Colombia, Campus La Nubia, Bloque L, 226, Manizales, 170003, Colombia.

Supplementary Material

Table S1. Physicochemical (following standard methods [73]) and biological (following ISO procedures) parameters of the ITWW and the supernatant from the Coagulation-Flocculation treatment (ITWW-CF), objects of this study. The values ​​reported for ITWW are average values ​​of 3 samplings carried out monthly. The values ​​reported for ITWW-CF are average values ​​of the supernatant of the CF process and correspond to approx. 36 L of treated ITWW, specifically 18 jars of 2 liters each. The maximum permissible discharge limits (*) ​​for textile industry effluents are also included according to current environmental regulations [30].

Table S2. The main characteristics of the CF, CF-F-N and CF-EF-N processes implemented in the treatment of textile effluent

Figure S1. Relative impact of the most harmful reagents in (a) F-N and (b) EF-N during their life cycle assessment according to the ReCiPe-2016 method at the midpoint level

Figure S2. Relative impact of electricity in sequential processes, EF-N and F-N, estimated using ReCiPe-2016 method at the midpoint level

Note: The numbering of bibliographic references coincides with that of the main text for the reader's convenience.

Table S1. Physicochemical (following standard methods [73]) and biological (following ISO procedures) parameters of the ITWW and the supernatant from the Coagulation-Flocculation treatment (ITWW-CF), objects of this study. The values ​​reported for ITWW are average values ​​of 3 samplings carried out monthly. The values ​​reported for ITWW-CF are average values ​​of the supernatant of the CF process and correspond to approx. 36 L of treated ITWW, specifically 18 jars of 2 liters each. The maximum permissible discharge limits (*) ​​for textile industry effluents are also included according to current environmental regulations [30].

| Parameter | Unit | Standard method [73] / ISO procedure | Discharge limit* [30] | Average values |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ITWW | ITWW-CF |
| General |  |  |  |  |  |
| pH | - | 4500-H+ B | 6.00 a 9.00 | 7.76 ± 0.39 | 7.24 ± 0.91 |
| Chemical Oxygen Demand (COD) | mg/L O2 | 5220 D | 400.00 | 3375 ± 300.3 | 1282 ± 111 |
| Biochemical Oxygen Demand (BOD5) | mg/L O2 | 5210 B | 200.00 | 686.6 ± 46.3 | 310.3 ± 104 |
| Total Suspended Solids (TSS) | mg/L | 2540 D | 50.00 | 90.2 ± 40.5 | 10.88 ± 1.30 |
| Oil and Grease | mg/L | 5520 D | 20.00 | 9.82 ± 1.3 | 3.5 ± 0.3 |
| Phenols | mg/L | 5530 B,C | 0.20 | 0.61 ± 0.07 | N. D. |
| Hydrocarbons |  |  |  |  |  |
| Hydrocarbons | mg/L | 5520 F | 10.00 | 1.27 ± 0.09 | 0.24 ± 0.06 |
| Nitrogen Compounds |  |  |  |  |  |
| Nitrogen (N) | mg/L | 4500-N | A&R | 705.2 ± 697.7 | 367 ± 256.3 |
| Ions |  |  |  |  |  |
| Chloride (Cl-) | mg/L | 4500-Cl- B | 1200.00 | 192.0 ± 95.8 | 123.3 ± 19.43 |
| Sulfate (SO42-) | mg/L | 4500-SO42- E | A&R | 1480 ± 156.6 | 1526 ± 451 |
| Metals and Metalloids |  |  |  |  |  |
| Aluminum (Al) | mg/L | 3111 B | N.A. | 0.58 ± 0.28 | 0.73 ± 0.12 |
| Cadmium (Cd) | mg/L | 3111 B | 0.02 | < LDM | < LDM |
| Cobalt (Co) | mg/L | 3111 B | 0.50 | < LDI | < LDI |
| Cupper (Cu) | mg/L | 3111 B | 1.00 | < LDM | < LDM |
| Chromium (Cr) | mg/L | 3111 B | 0.50 | 23.18 ± 6.09 | 1.55 ± 0.81 |
| Iron (Fe) | mg/L | 3111 B | N.A. | 1.46 | < LCM |
| Nickel (Ni) | mg/L | 3111 B | 0.50 | < LCM | < LCM |
| Zinc (Zn) | mg/L | 3111 B | 3.00 | 0.20 | < LCM |
| Other Parameters for Analysis and Report |  |  |  |  |  |
| True color (Absorbance measurements at different wavelengths: 436, 525 and 620 nm) | m-1 | ISO 7887: 2012-04 B, 2120-D | A&R | a147.1 ± 37.1 a149.2 ± 36.8 a140.8 ± 31.9 | 104.5 ± 51.7 66.5 ± 41.6 53.8 ± 33.7 |
| Additional parameters of interest (not included in current environmental regulation) |  |  |  |  |  |
| Total Organic Carbon (TOC) | mg/L C | 5310 B | N.A. | 1330 ± 55 | 530 ± 70 |
| Conductivity | mS/cm | 2510 B | N.A. | 2.27 ± 0.43 | 4.07 ± 0.69 |
| Hexavalent Chromium | mg/L | 3500-Cr B | N.A. | 21.39 ± 2.51 | 0.34 ± 0.02 |
| Dissolved Oxygen | mg/L | 4500-O B | N.A. | 0.9 ± 0.7 | 1.69 ± 0.25 |
| Turbidity | NTU | 2130 B | N.A. | 261.6 ± 92.4 | 86.5 ± 0.85 |
| Color | mg/L | 2120 C | N.A. | 1048 ± 85 | 35.31 ± 23.10 |
|  | U. Pt-Co | 2120 B | N.A. | 44232 ± 2187 | 2522 ± 1447 |
|  | RGB Scale | 2120 E | N.A. | [7; 0; 0] | [153; 122; 94] |
|  | Tone | N.A. | N.A. |  |  |
| Median lethal concentration (LC50 Artemia salina) | mg/L | ISO 6341:2012 ISO 5667-16:2000 | N.A. | 15.1 ± 2.0 | 51.1 ± 3.3 |

N.A.: Does not apply; A & R: Analysis and Report; Dilution factor used for measurement: a = 1:6.67; LDI (Instrument Detection Limits): Cd = 0.003 mg/L; Co = 0.01 mg/L; Cu=0.005 mg/L; LCM (Limit of quantification): Al = 1.86 mg/L; Cr = 0.75 mg/L; Ni = 0.61 mg/L; Zn = 0.13 mg/L.

Table S2. The main characteristics of the CF, CF-F-N and CF-EF-N processes implemented in the treatment of textile effluent.

| Experimental setup configuration |  |  | Primary treatment | Sequential AOP |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Coagulation-Flocculation | Fenton | Electro-Fenton |
| Reactor type |  |  | Rectangular jar with paddle stirrer (ASTM D2035) | Batch, under continuous stirring, jacketed bath (F-12-CM, Julabo GmbH) with temperature control (PT-100, 0.01 K) | Electrochemical cell, under continuous stirring, single compartment, monopolar configuration of electrodes arranged in parallel, galvanostatic mode of operation, jacketed bath (F-12-CM, Julabo GmbH) with temperature control (PT-100, 0.01 K) |
| Reactor material |  |  | Polycarbonate | Borosilicate glass | Borosilicate glass |
| Reactor volume (L) |  |  | 2.0 | 0.2 | 0.2 |
| Electrode material |  |  | N.A. | N.A. | Boron-doped diamond anode (BDD/Si ​​NeoCoat®-Electrodes, Switzerland; submerged area of 20 cm2); Commercial activated carbon felt cathode (ACF1600, 1500 m2/g, Ceramaterials LLC, USA; submerged area of 30 cm2) |
| Distance between electrodes (cm) |  |  | N.A. | N.A. | 1 |
| Reactor inputs |  |  | ITWW Al2(SO4)3·14H2O (98% p/p, Protokimica, Colombia) Ca(OH)2 (99% p/p, BDH La, Colombia) | ITWW-CF FeSO4.7H2O (Extra Pure Grade, Duksan, Korea); H2O2 (30% p/p, Biopharchem, Sigma Aldrich, USA) H2SO4 (95-97%, 1.84 g/cm3, Merck KGaA, Germany) Ca(OH)2 (g/L) (97% m/m, BDH La, Colombia) | ITWW-CF External DC power supply 0-30V, 0-6A (OWON® ODP3063, China); FeSO4.7H2O (Extra Pure Grade, Duksan, Korea); NaCl (ACS, para análisis, Merck, Germany); H2SO4 (95-97%, 1.84 g/cm3, Merck KGaA, Germany) NaOH (g/L) (≥97% m/m, Carlo Erba, France) |
| Reactor outputs |  |  | ITWW-CF Sludge (12.4 g/L) | ITWW-CF-F-N Sludge (7.65 g/L) | ITWW-CF-EF-N Sludge (0.21 g/L) |
| Aeration flow (L/min) |  |  | N.A. | N.A. | 1.6 |
| Stirring velocity (rpm) |  |  | Alkalinization (Ca(OH)2): 150 rpm – 5 min Fast mixing stage (Al2(SO4)3·14H2O): 150 rpm – 1 min Slow mixing stage: 40 rpm – 20 min Settling – 15 min | 190 | 500 |
| Optimization of operating conditions |  |  |  |  |  |
| Independent variables |  |  | [Al2(SO4)3·14H2O] = 7-22 g/L Ca(OH)2 = 3-9 g/L | pH = 2.82–5, [Fe2+] = 564–2000 mg/L, [H2O2] = 3153–8000 mg/L | j = 3.33–13.33 mA/cm2, [Fe2+] = 0.1–3 mM, [NaCl] = 0–2000 mg/L |
| Dependent variables |  |  | %RCOD %RColor TOpC GS pHfinal | %RCOD TOpC | COD/COD0 TOC/TOC0 TOpC |
| Optimization methodology |  |  | CCD (two levels and two factors), RSM | CCD (two levels and three factors), RSM | CCD (two levels and three factors), RSM |
| Optimization conditions |  |  | Multivariable, restricted: maximize: RCOD + RColor, + minimize: (TOpC) + pHfinal ≥ 6 | Multivariable, restricted: maximize: RCOD subject to RCOD 71% [30] + minimize: TOpC | Multivariable, restricted Minimize: COD/COD0 subject to COD/COD0 ≤ 0.36 [30] + minimize: TOC/TOC0 + minimize: TOpC) |
| Optimization tool |  |  | Design Expert® D=(d1w1d2w2)1/wi MatLab® (functions: patternsearch (which determines the highest “desirability” optimization parameter) and fmincon (optimization) | Design Expert® D=(d1w1d2w2)1/wi MatLab® (functions: patternsearch (which determines the highest “desirability” optimization parameter) and fmincon (optimization) | Design Expert® D=(d1w1d2w2d3w3)1/wi MatLab® (functions: patternsearch (which determines the highest “desirability” optimization parameter) and fmincon (optimization) |
| Optimal operating conditions |  |  |  |  |  |
| Al2(SO4)3·14H2O (g/L) |  |  | 16.09 | N.A. | N.A. |
| Ca(OH)2 (g/L) |  |  | 5.16 | N.A. | N.A. |
| [H2O2] (mg/L) |  |  | N.A. | 3679 | N.A. |
| [Fe2+] (mg/L) |  |  | N.A. | 1977 | 104.77 |
| [NaCl] (mg/L) |  |  | N.A. | N.A. | 107.43 |
| pH |  |  | > 12.1 | 2.31 | 3.0 |
| Current density (j) (mA/cm2) |  |  | N.A. | N.A. | 5.1 |
| Treatment time (min) |  |  | 41 | 180 | 120 |
| Temperature (oC) |  |  | 20 | 20 | 20 |
| NaOH (g/L) (≥97% m/m, Carlo Erba, France) |  | Neutralization | N.A. | N.A. | 0.40 |
| Ca(OH)2 (g/L) (97% m/m, BDH La, Colombia) |  |  | N.A. | 4.63 | N.A. |
| Organic compound removal performance |  |  |  |  |  |
| %RCOD |  |  | 61.01 ± 3.32 | 71.25 ± 0.31 | 66.86 ± 2.81 |
| %RTOC |  |  | 60.15 ± 4.10 | 66.09 ± 0.48 | 61.84 ± 1.74 |
| %RColor |  |  | 97.14 ± 1.18 | 99.58 ± 0.05 | 96.25 ± 1.86 |
| Operating costs (TOpC) (USD/m3) |  |  | 5.99 ± 0.11 TOpCC-FUSDm3=Palum[alum]Purityalum +PCa(OH)2[Ca(OH)2]PurityCa(OH)2 +Psludge×Csludge | 7.67 ± 0.02 | 5.92 ± 0.10 TOpCEFUSDm3=PSalt-Fe×[Salt-Fe]PzaSalt-Fe+PH2SO4×[H2SO4]PzaH2SO4+Pelectricity×Ec |
| %RCOD-sequential |  |  | N.A. | 88.28 ± 0.27 | 89.34 ± 1.68 |
| %RTOC- sequential |  |  | N.A. | 86.39 ± 0.62 | 86.92 ± 0.75 |
| %RColor- sequential |  |  | N.A. | 99.99 ± 0.01 | 99.76 ± 0.07 |
| pHfinal |  |  | 7.24 ± 0.96 | 6.1 ± 0.10 | 7.9 ± 0.20 |
| Total operational costs (TOpCtotal) (USD/m3) |  |  | 5.99 ± 0.11 N.A. | 13.66 ± 0.12 TOpCF-NUSDm3=TOpCF +PCa(OH)2×[Ca(OH)2]PurityCa(OH)2+ Psludge×Csludge | 11.91 ± 0.21 TOpCEF-NUSDm3=TOpCEF +PNaOH×[NaOH]PurityNaOH+ Psludge×Csludge |
| Medium Lethal Concentration (LC50 Artemia salina) (mg/L) |  |  | 51.1 ± 3.3 | Non-toxic | Non-toxic |
| Reference |  |  | [67] | [65] | [66] |

N.A.: Does not apply; CCD - Central Composite experimental Design; RSM - Response Surface Methodology; ,  , , 
, , , represented the costs (USD/kg) of: slaked lime, alum, sludge disposal, iron sulfate heptahydrate, hydrogen peroxide, sulfuric acid and sodium hydroxide, respectively; Pelectricity represented the costs (USD/kWh) of electricity consumption; Ec correspond to electricity consumption (kWh/m3), Purityi accounted for the purity of different reagents; [Ca(OH)2], [alum], [H2SO4], [NaOH], [Salt-Fe] and [H2O2] corresponded to concentrations of slaked lime, alum, sulfuric acid, sodium hydroxide, iron sulfate heptahydrate and hydrogen peroxide, respectively; Csludge (kg/m3) was the concentration of the sludge generated (mass of sludge/total volume of effluent treated in the jar)

Figure S1. Relative impact of the most harmful reagents in (a) F-N and (b) EF-N during their life cycle assessment according to the ReCiPe-2016 method at the midpoint level

Figure S2. Relative impact of electricity in sequential processes, EF-N and F-N, estimated using ReCiPe-2016 method at the midpoint level