P-recovery versus current sewage sludge treatment policy in the Czech Republic and Japan

Matěj Hušeka,b,c, Ryosuke Hommaa, Jaroslav Moškob,c, Michael Pohořelýb,c,*, Kazuyuki Oshitaa

a. Department of Environmental Engineering, Graduate School of Engineering, Kyoto University, Nishikyo-ku, Katsura C-1-3, Kyoto, 615-8540, Japan

b. Department of Power Engineering, Faculty of Environmental Technology, University of Chemistry and Technology, Prague, Technická 5, 166 28, Prague 6, Czech Republic

c. The Czech Academy of Sciences, Institute of Chemical Process Fundamentals, Rozvojová 135, 165 00, Prague 6 – Suchdol, Czech Republic

* Corresponding author: michael.pohorely@vscht.cz; University of Chemistry and Technology Prague, Faculty of Environmental Technology, Department of Power Engineering, Technická 5, 166 28 Prague 6, Czech Republic.

SFigure 1 Sewage sludge treatment statistics – Czech Republic (CZSO, 2022, 2021, 2016)

Sludge production and treatment

SFiguer 2 Sewage sludge treatment statistics – Japan (JSWA, 2022, 2018)

SFiguer 2 Sewage sludge treatment statistics – Japan

- soil application: sum of fertiliser, soil improver, artificial soils, and composting

- incineration, solid fuel, melting, carbonization, and cement production,

- other: sum of construction materials, landfilling, and others.

STable 1 Sludge statistics by JSWA (JSWA, 2022, 2018)

|  | 2015 | 2020 |
| --- | --- | --- |
| Raw sludge [m3] | 3,903 | 4,841 |
| Thickened sludge [m3] | 67,143 | 65,491 |
| Digested sludge [m3] | 4,296 | 3,399 |
| Subtotal [m3] | 75,341 | 73,731 |
| Dewatered sludge [t] | 2,103,180 | 2,144,799 |
| Mobile dewatering truck sludge [t] | 7,409 | 6,589 |
| Machine-dried sludge [t] | 62,036 | 38,629 |
| Sun-dried sludge [t] | 911 | 583 |
| Carbonized sludge [t] | 9,518 | 27,039 |
| Incinerated ash [t] | 185,451 | 155,848 |
| Molten slag [t] | 15,134 | 3,810 |
| Compost [t] | 25,451 | 15,783 |
| Subtotal [t] | 2,409,091 | 2,393,080 |

# Legislation

## Sewage sludge

According to microbiological criteria (STable 2), sludges in the Czech Republic are evaluated as sludges of category I and II. In the case of sludge II (more benevolent), the following conditions apply above and beyond the regular ones: at least one winter period must elapse between sludge application and harvest, except for the cultivation of crops to be used exclusively for technical purposes, and field vegetables, potatoes, and intensive fruit planting can not be grown on the part of the soil block where the sludge was applied for three years after sludge application.

STable 2 Limits of pathogens in sewage sludge for agricultural use in the Czech Republic

| Sludge category | Salmonella sp. in 50 g | Enterococci | Thermophilic coliform bacteria |  |
| --- | --- | --- | --- | --- |
| I.(1) | negative | < 103 CFU.g-1 (4 samples of 5) < 5.103 CFU.g-1 (1 sample of 5) no occurrence in 50 g | not evaluated | Regulation No. 273/2021 Coll. |
| II. | not evaluated | < 106 CFU.g-1DM (5 samples) | < 106 CFU.g-1DM (5 samples) |  |

- the presence of Escherichia coli can be determined instead of enterococci

The limit values for the concentrations of hazardous substances and elements in sludge (Table 3), the precautionary values for heavy metals in soil (STable 3) and the maximum sludge dose per hectare have been set to prevent the introduction of hazardous substances into the soil over a period of 10 years. There are no direct limits as laid down in Directive 86/278/EEC (STable 4).

STable 3 Limit values for concentrations of heavy metals in soil with a pH of 6 to 7

|  | European Union(1) |  | Czech Republic |  |
| --- | --- | --- | --- | --- |
|  |  | common soils(2) |  | light soils(3) |
|  | mg.kg-1 DM |  |  |  |
| Cd | 1.00–3.00 |  | 0.50 | 0.40 |
| Cu | 50.0–140(4) |  | 60.0 | 45.0 |
| Ni | 30.0–75.0(4) |  | 50.0 | 45.0 |
| Pb | 50.0–300 |  | 60.0 | 55.0 |
| Zn | 150–300(4) |  | 120 | 105 |
| Hg | 1.00–1.50 |  | 0.30(5) | 0.30(5) |
| Cr | – |  | 90.0 | 55.0 |
| As | – |  | 20.0 | 15.0 |
| Be |  |  | 2.00 | 1.50 |
| Co |  |  | 30.0 | 20.0 |
| V |  |  | 130 | 120 |
| PCB | – |  | 0.02(6) | 0.02(6) |
| PAH | – |  | 1.00(7) | 1.00(7) |
|  | Directive 86/278/EEC |  | Regulation No. 273/2021 Coll |  |
| - Member States may permit the limit values they fix to be exceeded in the case of the use of sludge on land which at the time of notification of this Directive is dedicated to the disposal of sludge but on which commercial food crops are being grown exclusively for animal consumption. Member States must inform the Commission of the number and type of sites concerned. They must also seek to ensure that there is no resulting hazard to human health or the environment. - Common soils: loam, silty, silty-clay and clay soils, which occupy the majority of agricultural land. These are soils with a normal variation of elements, with normal soil development under different geomorphological conditions, including soils on carbonate rocks. - Light soils: soils formed on very light and poor parent rocks such as sands and gravels. The definition of these soils is based on the proportion of fine particles (up to 0.01 mm), which shall not exceed 20 %. These soils are characterised by a very low absorption capacity. - Member States may permit the limit values they fix to be exceeded in respect of these parameters on soil with a pH consistently higher than 7. The maximum authorised concentrations of these heavy metals must in no case exceed those values by more than 50 %. Member States must also seek to ensure that there is no resulting hazard to human health or the environment and in particular to groundwater. - Hg content is determined as the total content; the contents of the other elements, i.e. As, Be, Cd, Co, Cr, Cu, Ni, Pb, V, and Zn are determined by extraction with aqua regia. - Sum of 7 congeners: 28+52+101+118+138+153+180. - Sum of anthracene, benzo(a)anthracene, benzo(b)fluoranthene, benzo(k)fluoranthene, benzo(a)pyrene, benzo(ghi)perylene, phenanthrene, fluoranthene, chrysene, indeno(1,2,3–cd)pyrene, naphthalene, and pyrene. |  |  |  |  |

STable 4 Limit values for amounts of heavy metals which may be added annually to agricultural land,
based on a 10-year average (Directive 86/278/EEC)

|  | European Union(1) |
| --- | --- |
|  | mg.kg-1 DM |
| Cd | 0.15 |
| Cu | 12.0 |
| Ni | 3.00 |
| Pb | 15.0 |
| Zn | 30.0 |
| Hg | 0.10 |

The use of sewage sludge in the Czech Republic is prohibited:

- on agricultural land that is part of protected nature and landscape areas under the Nature and Landscape Protection Act,

- on forest land normally used for forest management,

- in protection zones of water sources, on flooded soils and in waterlogged areas,

- in the protection zones of natural medicinal resources and sources of natural medicinal waters according to the Spa Act,

- on permanent grassland and grassland on arable land during the growing season until the last mowing,

- in intensive fruiting fruit plantations,

- on land used for the cultivation of field vegetables in the calendar year of their cultivation and in the preceding calendar year,

- during the growing season, when growing forage crops, maise and sugar beet using the sward for feeding,

- where soil analyses show that the content of selected risk substances in the average sample exceeds one of the values laid down in the ministerial decree,

- on soils with a soil exchange reaction value below pH 5.6,

- on areas designated for recreation and sport or public spaces,

- on agricultural land where the precautionary value under the Agricultural Land Protection Act has been found to be exceeded, or

- where the sludge does not meet the microbiological criteria laid down by ministerial decree; use of microbially contaminated sludge may be made only after treatment of the sludge. (Act No. 542/2020 Coll.)

STable 5 Judgment criteria for industrial waste containing metal and other contaminants (Prime Minister’s Office Regulation No. 5 of 1973)

| mg.l-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| alkylmercury compounds | 0.00 | organophosphorus compounds | <1.00 | 1,3-dichloropropene | <0.02 |
| Hg | <0.005 | trichloroethylene | <0.10 | 1,1,1-trichloroethane | <3.00 |
| Cd | <0.09 | tetrachloroethylene | <0.10 | thiram | <0.06 |
| Pb | <0.30 | dichloromethane | <0.20 | simazine | <0.03 |
| Cr+VI | <1.50 | carbon tetrachloride | <0.02 | thiobencarb | <0.20 |
| As | <0.30 | 1,2-dichloroethane | <0.04 | benzene | <0.10 |
| Se | <0.30 | 1,1-dichloroethylene | <1.00 | 1,4-dioxane | <0.50 |
| CN− | <1.00 | cis-1,2-dichloroethylene | <0.40 | dioxins(1)) | <3x10-6 |
| PCB | <0.003 | 1,1,2-trichloroethane | <0.06 |  |  |

- dioxins prescribed in Article 2, Paragraph 1 of the Act on Special Measures against Dioxins (Act No. 105 of 1999)

The plant damage test in Japan examines using sludge fertilisers and sample fertilisers specified in Article 6 of the Fertilizer Control Act, with each sample validated at n=8. The standard used is a pot with an 11.3 cm inner diameter and 6.5 cm height, the test plant: Komatsuna (Japanese mustard spinach), the seed quantity: 20 grains per test container, time: 3 weeks, temperature 15–25 °C. The fertiliser will be rejected if the average weight is 5 % lower than the standard value (Fertilizer Control Act).

STable 6 Target levels of selected pollutants in the soil (Environmental Agency Notification No. 46)

| mg.l-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| alkylmercury compounds | 0.00 | organophosphorus compounds | 0.00 | 1,3-dichloropropene | 0.002 |
| Hg | 0.0005 | trichloroethylene | 0.01 | 1,1,1-trichloroethane | 1.00 |
| Cd(1) | 0.003 | tetrachloroethylene | 0.01 | thiram | 0.006 |
| Pb(2) | 0.01 | dichloromethane | 0.20 | simazine | 0.003 |
| Cr+VI | 0.05 | carbon tetrachloride | 0.002 | thiobencarb | 0.02 |
| As | 0.01 | 1,2-dichloroethane | 0.004 | benzene | 0.01 |
| Cu | (3) | 1,1-dichloroethylene | 0.01 | 1,4-dioxane | 0.05 |
| Se | 0.01 | F | 0.80 | 1,2-dichloroethylene | 0.04 |
| CN− | 0.00 | cis-1,2-dichloroethylene | 0.40 | B | 1.00 |
| PCB(4) | 0.00 | 1,1,2-trichloroethane | 0.006 | Chloroethylene(5) | 0.002 |

- and not more than 0.4 mg.kg-1 of rice on agricultural land

- and less than 15 mg.kg-1 of soil for agricultural land (limited to rice fields)

- agricultural land (limited to rice fields), in agricultural land (limited to rice paddies), the concentration shall be less than 125 mg per kg of soil

- see JIS K 0093:2006 and Environmental quality standards regarding water pollution

- aka vinyl chloride or vinyl chloride monomer

## Sludge compost

Czech limits are set for compost used on agricultural and forestry land (Group I) and outside agricultural and forestry land (Group II): Class I (intended for greenery at sports and recreational facilities), Class II (urban greenery, greenery of parks and forest parks, reclamation ), and Class III (creation of reclamation layers of secure landfills) (STable 7) (Regulation No. 273/2021 Coll.; Regulation No. 474/2000 Sb). Sewage sludge must not make up more than 40 % of the total weight of waste and other materials in compost. (Regulation 474/2000 Coll.). The compost pathogen limits are stricter than for direct sludge use in agriculture (STable 8). As part of compost production, hygienization must take place, which should be guaranteed by a minimum temperature for a given period (STable 9). Additional quality parameters for Group II are available in STable 10. The Japanese compost with sludge is controlled for the presence of heavy metals and radioactive caesium, and leach and ecotoxicological tests are required (STable 7).

STable 7 Limit values for pollutants in composts with sewage sludge

|  | Czech Republic |  |  |  | Japan |
| --- | --- | --- | --- | --- | --- |
|  | Group I. | Group II. |  |  |  |
|  | agricultural and forestry land | outside of agricultural and forestry land |  |  |  |
|  |  | Class I. | Class II. | Class III. |  |
|  | mg.kg-1DM |  |  |  |  |
| Cd | 2.00 | 1.70 | 3.00 | 4.00 | 5.00 |
| Pb | 100 | 100 | 300 | 400 | 100 |
| Hg | 1.00 | 0.80 | 1.50 | 2.00 | 2.00 |
| As | 30.0 | 10.0 | 20.0 | 30.0 | 50.0 |
| Cr | 100 | 100 | 250 | 300 | 500 |
| Cu | 150 | 170 | 400 | 400 | – |
| Ni | 50.0 | 65.0 | 100 | 120 | 300 |
| Zn | 600 | 500 | 1,200 | 1,500 | – |
| PCB | – | 0.02(1) | 0.20(1) | – | – |
| PAH | – | 3.00(2) | 6.00(2) | – | – |
| radioactive Cs | – | – | – | – | 200(3) |
| Impurities > 2.0 mm | – | < 0.50 % | < 0.50 % | – | – |
| Non-degradable and undesirable impurities | – | < 5.00 % | < 5.00% | – | – |
| pathogen limits | YES | YES | YES | YES | – |
| leach test | – | – | – | – | YES |
| ecotoxicology | – | – | – | – | YES |
|  | Regulation No. 474/2000 Sb | Regulation No. 273/2021 Coll. |  |  | Fertilizer Control Act |

- types of cogenerates are not specified

- types of PAHs are not specified

- radioactive caesium concentration in Bq.kg-1 (MAFF, 2012, 2022a), content is measured in the raw (wet) state of used sludge or ash, only applies to Iwate prefecture, Miyagi prefecture, Akita prefecture, Yamagata prefecture, Fukushima prefecture, Ibaraki prefecture, Tochigi prefecture, Gunma prefecture, Saitama prefecture, Chiba prefecture, Tokyo, Kanagawa prefecture, Nagano prefecture, Yamanashi prefecture, Shizuoka prefecture, Niigata prefecture (MAFF, 2022b)

At the EU level, the pollutant content of compost is not regulated centrally, just at the level of individual member states. Limits exist only for EU fertilising products sold on the European market. The product must have the CE certificate and comply with Regulation (EU) 2019/1009. Part of the regulation includes a ban on the use of sewage sludge for EU fertilising products containing compost and digestate as a precautionary principle (Regulation (EU) 2019/1009).

STable 8 Limits of pathogens in compost with sludge in the Czech Republic

|  | Number of samples |  | Limit |  |  |
| --- | --- | --- | --- | --- | --- |
| Salmonella sp. | 5 |  | no occurrence in 50 g |  | Regulation No. 273/2021 Coll. |
| Escherichia coli or Enterococci | 4/5 | 1/5 | < 103 CFU.g-1 | < 5.103 CFU.g-1 |  |

STable 9 Minimum hygienisation time at a given temperature (Regulation No. 273/2021 Coll)

| Temperature limit | Time interval |
| --- | --- |
| ≥ 70°C | continuously for min. 3 days |
| ≥ 65°C | continuously for min. 5 days |
| ≥ 60°C | continuously for min. 7 days |
| ≥ 55°C | continuously for min. 14 days |

STable 10 Compost quality characteristics – Group II (Regulation No. 273/2021 Coll)

| Parameter | Value |
| --- | --- |
| moisture (% wt.) | 30.0–65.0 |
| combustible matter (% wt.DM) | min. 20.0 |
| total nitrogen (% wt.DM) | min. 0.60 |
| C:N ratio | max. 30.0 |
| pH | 6–9 |
| Non-degradable impurities > 20 mm (% wt.DM) | < 3.00 |
| Undesirable impurities > 5 mm (% wt.DM) | < 0.50 |
| Sprouted seeds in 1 l of compost (psc) | ≤ 3 |

## Sludge ash, sludge-char, and slag

In addition to the limits for sewage sludge, there are also limits for the products resulting from the thermal treatment of sewage sludge (STable 11), such as ash, sludge-char (pyrolysed/carbonized sludge) and slag (not set in the Czech Republic), which can be used as a fertiliser or soil improver. Both Japanese and Czech legislation regulate the heavy metal content of these products. Additionally, Japanese legislation has a limit for Ni and radioactive caesium, leachate and ecotoxicological tests (Chapter 3.2.1) and the Czech Republic for PAHs. The limits for organic pollutants should not be forgotten. If measured in the final product, it indicates an incomplete or poorly functioning thermal processing. These substances (PAHs, PCBs, or PCDD/Fs) can be toxic, mutagenic, or carcinogenic and can be released into the environment.

At the EU level, it is possible to use thermal oxidation materials or derivatives (ash or slag) to produce EU fertilising products. To be used for further production of EU fertilising products, these products must be registered and meet the limits in STable 11 (Regulation (EU) 2019/1009). Additional limits are then applied depending on the type of product manufactured (for example, limits for inorganic fertilisers or soil improvers). The use of sludge-char in EU fertilising products is prohibited by Regulation (EU) 2019/1009 due to the lack of information on the ability of pyrolysis to remove organic pollutants (Huygens et al., 2019). However, this does not concern the legislation of individual EU member states. Therefore, it is possible to use sludge-char in the Czech Republic if the limits are met, the char is certified by the Central Institute for Supervising and Testing in Agriculture, and its use is allowed (the end-of-waste status obtained) by the local authority (Regulation No. 474/2000 Coll.; Act No. 156/1998 Coll.). Outside the Czech Republic, at the level of the EU Member States, it is currently possible to use sludge-char in Sweden and Denmark (EBI, 2023).

STable 11 Pollutant limits for thermally treated sludge products used on agricultural land (CZK, JPN) and EU fertiliser products

|  | EU |  | Czech Republic |  |  | Japan |
| --- | --- | --- | --- | --- | --- | --- |
|  | thermal oxidation materials or derivates | ash, sludge-char |  | ash, slag, carbonized sludge |  |  |
|  | mg.kg-1DM |  |  |  |  |  |
| Cd | – | 5.00 |  |  | 5.00 |  |
| Pb | – | 100 |  |  | 100 |  |
| Hg | – | 0.50 |  |  | 2.00 |  |
| As | – | 30.0 |  |  | 50.0 |  |
| Cr | 400 | 100 |  |  | 500 |  |
| Corg | < 30,000 | – |  |  | – |  |
| Cl- | 30,000(1) | – |  |  | – |  |
| Ni | – | – |  |  | 300 |  |
| Tl | 2.00 | – |  |  | – |  |
| PAH | 6.00(2) | 20.0(3) |  |  | – |  |
| PCDD/F | 20.0(4) | – |  |  | – |  |
| radioactive Cs | – | – |  |  | 200(5) |  |
| leach test | – | – |  |  | YES |  |
| Ecotoxicology | – | – |  |  | YES |  |
|  | Regulation (EU) 2019/1009 |  | Regulation No. 474/2000 Coll. |  |  | Fertilizer Control Act |

- this limit value shall not apply to EU fertilising products produced through a manufacturing process where a Cl- containing compound has been added with the intention of producing alkali metal salts or alkaline earth metal salts, and is declared in accordance with Annex III.

- sum of naphthalene, acenaphthylene, acenaphthene, fluorene, phenanthrene, anthracene, fluoranthene, pyrene, benzo[a]anthracene, chrysene, benzo[b]fluoranthene, benzo[k]fluoranthene, benzo[a]pyrene, indeno[1,2,3-cd]pyrene, dibenzo[a,h]anthracene and benzo[ghi]perylene.

- sum of anthracene, benzo(a)anthracene, benzo(b)fluoranthene, benzo(k)fluoranthene, Benzo(a)pyrene, benzo(ghi)perylene, phenanthrene, fluoranthene, chrysene, indeno(1,2,3–cd)pyrene, naphthalene, and pyrene.

- WHO toxicity equivalents of PCDD/F.kg-1DM

- radioactive caesium concentration in Bq.kg-1 (MAFF, 2022a, 2012), content is measured in the raw (wet) state, only applies to Iwate prefecture, Miyagi prefecture, Akita prefecture, Yamagata prefecture, Fukushima prefecture, Ibaraki prefecture, Tochigi prefecture, Gunma prefecture, Saitama prefecture, Chiba prefecture, Tokyo, Kanagawa prefecture, Nagano prefecture, Yamanashi prefecture, Shizuoka prefecture, Niigata prefecture (MAFF, 2022b)

# Sludge thermal treatment and P-recovery technologies in use

## Czech Republic

STable 12 Technical specifications of individual (mono) thermal facilities processing sludge

|  | Municipality | Type | Drying medium | Sludge capacity (tDM.day-1) | Reason for acquisition |
| --- | --- | --- | --- | --- | --- |
| 1 | Přerov | low-temperature dryer | hot water | 6.0 | incineration |
| 2 | Karlovy Vary-Drahovice | low-temperature dryer | hot water | ≥ 3.7 | weight reduction |
| 3 | Mariánské Lázně | solar dryer | hybrid (solar+hot water) | 1.1 | weight reduction |
| 4 | Bohuslavice-Trutnov | low-temperature dryer | hot water | 3.1 |  |
|  |  | pyrolysis unit |  | 2.1 | weight reduction |
| 5 | Brno-Modřice (under preparation) | low-temperature dryer | hot water | 37.0 | incineration/cement production |
| 6 | Tábor (under preparation) | low-temperature dryer | hot water | 6.3 |  |
|  |  | pyrolysis unit |  | 0.0 | weight reduction |
| 7 | Olomouc (under construction) | low-temperature dryer | hot water | 5.8 | incineration |
| 8 | Šumperk | low-temperature dryer | hot water | 5.3 | weight reduction |
| 9 | Cheb (under construction) | solar dryer | hybrid (solar+hot water) | 2.5 | incineration |
| 10 | Tachov (under construction) | low-temperature dryer | hot water | 2.2 | weight reduction |
| 11 | Mladá Boleslav (under construction) | low-temperature dryer | hot water | 3.6 | incineration |
| 12 | České Budějovice (under preparation) | low-temperature dryer | hot water | 11.8 | incineration |

in operation

under preparation

under construction

SFigure 3 Facilities for mono thermal treatment of sewage sludge (Lacko, 2023)

In addition to the facilities mentioned above, approximately ten solar dryers are in the pipeline at various locations.

## Japan

The individual STables include the location (municipality and prefecture), unit description (number, type, and capacity), sludge production at the WWTP, and product quantity (slag, char, or ash). In some cases, the product quantity is disproportionate to the sludge production, indicating that the plant processes sludge other than its own. Due to the nature of the available data, the present results are only informative.

STable 13 Technical specifications of municipal (state) melting plants (JSWA, 2022)

|  | Municipality | Prefecture | Number of units | Furnace type | Sludge capacity (t.day-1) | Sludge production (tDM.year-1) | Slag production (tDM, FY2020) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Takaoka | Toyama | 3 | surface melting | 2 x 16; 12 | 5,498 | 933 |
| 2 | Imizu | Toyama | 2 | surface melting | 9; 12 | 3,773 | 406 |
| 3 | Ibaraki | Osaka | 2 | coke-bed melting | 2 x 110 | 1,243 | 267 |
| 4 | Hirano Ward | Osaka | 1 | swirling melting | 150 | 3,443 | 1,109 |
| 5 | Konohana Ward | Osaka | 5 | swirling melting | 5 x 150 | 18,544 | 6,871 |
| 6 | Himeji | Hyogo | 1 | coke-bed melting | 40 | 8,637 | 3,466 |
|  |  |  | 2 | pyrolysis + swirling ash melting | 33; 14 | 8,882 |  |

STable 14 Technical specifications of municipal (state) carbonization plants (JSWA, 2022)

|  | Municipality | Prefecture | Number of units | Sludge capacity (t.day-1) | Sludge production (tDM.year-1) | Char production (tDM, FY2020) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Kesennuma | Miyagi | 1 | 23 | 289 | 195 |
| 2 | Daisen | Akita | 1 | 25 | 1,344 | 102 |
| 3 | Odate | Akita | 1 | 22 | 523 | 260 |
| 4 | Maebashi | Gunma | 2 | 2 x 25 | 1,412 | 410 |
| 5 | Koto City | Tokyo | 2 | 2 x 300 | 8,126 | 2,972 |
| 6 | Tsurumi Ward | Kanagawa | 1 | 200 | 6,964 | 2,173 |
| 7 | Kanazawa Ward | Kanagawa | 1 | 150 | 8,934 | 3,016 |
| 8 | Suruga Ward | Shizuoka | 3 | 3 x 75 | 5,320 | 932 |
| 9 | Toyohashi | Aichi | 1 | x | 3,851 | 2,060 |
| 10 | Nagaokakyo | Kyoto | 1 | 40 | 2,174 | 1,149 |
| 11 | Minami Ward(1) | Kyoto | 1 | 150 | 395 | 182 |
| 12 | Hirano Ward, | Osaka | 1 | 150 | 9,189 | 6,953 |
| 13 | Nishi Ward | Hiroshima | 2 | 2 x 50 | 5,150 | 3,467 |
| 14 | Hakata Ward | Fukuoka | 1 | 100 | 5,499 | 2,700 |

- in FY2020 in trial run

carbonization plants

melting plants

SFiguer 4 State facilities for mono thermal treatment of sewage sludge (GADM, 2022)

STable 15 Technical specifications of municipal (state) incineration plants (JSWA, 2022)

|  | Municipality | Prefecture | Number of units | Boiler type | Sludge capacity (t.day-1) | Sludge production (tDM.year-1) | Ash production (tDM, FY2020) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Shiroishi Ward | Hokkaido | 2 | fluidised bed | 2 x 150 | 17,304 | 1,827 |  |
| 2 | Teine Ward | Hokkaido | 5 | grate furnace | 4 x 100; 150 | 30,565 | 14,296/13,065(1) |  |
| 3 | Otaru | Hokkaido | 2 | fluidised bed | 60; x | 2,550 | 454 |  |
| 4 | Asahkawa | Hokkaido | 2 | fluidised bed | 80; 60 | 6,028 | 889 |  |
| 5 | Hirosaki | Aomori | 1 | fluidised bed | 80 | 4,587 | 516 |  |
| 6 | Morioka | Iwate | 2 | fluidised bed | 60(2); 50 | 2,992 | 671 |  |
| 7 | Kitakami | Iwate | 1 | fluidised bed | 45 | 1,577 | 219 |  |
| 8 | Tagajō | Miyagi | 1 | fluidised bed | 110 | 3,045 | 761 |  |
| 9 | Sendai | Miyagi | 3 | fluidised bed | 2 x 200; 70 | 19,136 | 2,018 |  |
| 10 | Mukaihama | Akita | 2 | fluidised bed | 2 x 50 | 4,323 | 782 |  |
| 11 | Iwaki | Fukushima | 1 | fluidised bed | 50 | 2,465 | 278 |  |
| 12 | Tsuchiura | Ibaraki | 2 | fluidised bed | 2 x 50 | 4,974 | 857 |  |
| 13 | Tone | Ibaraki | 1 | fluidised bed | 90 | 6,440 | 667 |  |
| 14 | Hitachinaka | Ibaraki | 2 | fluidised bed | 2 x 100 | 8,170 | 1,082 |  |
| 15 | Kamisu | Ibaraki | 2 | fluidised bed | 2 x 60 | 3,527 | 880 |  |
| 16 | Utsunomiya | Tochigi | 2 | fluidised bed | 2 x 90 | 5,800 | 1,247 |  |
| 17 | Ashikaga | Tochigi | 1 | fluidised bed | 25 | 713 | 158 |  |
| 18 | Toda | Saitama | 5 | fluidised bed | 5 x 200 | 38,654 | 3,888 |  |
| 19 | Okegawa | Saitama | 2 | fluidised bed | 100; 53 | 3,440 | 553 |  |
| 20 | Wako | Saitama | 3 | fluidised bed | 3 x 210 | 21,470 | 2,637 |  |
| 21 | Misato | Saitama | 3 | fluidised bed | 140; 240; 250 | 30,361 | 3,593 |  |
| 22 | Kuki | Saitama | 1 | fluidised bed | 50 | 1,615 | 210 |  |
| 23 | Sakado | Saitama | 1 | fluidised bed | 25 | 480 | 23 |  |
| 24 | Mihama Ward | Chiba | 3 | fluidised bed | 65; 2x 100 | 15,234 | 1,624 |  |
| 25 | Mihama Ward | Chiba | 2 | fluidised bed | 2 x 150 | 9,842 | 854 |  |
| 26 | Abiko | Chiba | 3 | fluidised bed | 80; 2 x 82 | 12,675 | 956 |  |
| 27 | Chūō Ward | Chiba | 3 | fluidised bed | 50; 105; 70(3) | 9,467 | 1,416 |  |
| 28 | Ichihara | Chiba | 2 | fluidised bed | 30; 50 | 3,053 | 355 |  |
| 29 | Inagi | Tokyo | 1 | fluidised bed | 80 | 947 | 780 |  |
| 30 |  |  | 1 | gasification furnace | 110 | 7,509 |  |  |
| 31 | Fuchu | Tokyo | 3 | fluidised bed | 130; 2 x 100 | 9,464 | 867 |  |
| 32 | Akishima | Tokyo | 2 | fluidised bed | 150; 100 | 10,816 | 1,217 |  |
| 33 |  |  | 1 | grate furnace | 140 | 2,932 |  |  |
| 34 | Kunitachi | Tokyo | 2 | fluidised bed | 2 x 40 | 3,450 | 370 |  |
| 35 | Hino | Tokyo | 2 | fluidised bed | 60; 100 | 5,309 | 455 |  |
| 36 | Hachioji (Komiyamachi) | Tokyo | 2 | fluidised bed | 50; 100 | 6,925 | 774 |  |
| 37 | Kiyose | Tokyo | 2 | fluidised bed | 2 x 100 | 8,089 | 1,379 |  |
| 38 |  |  | 1 | gasification furnace | 100 | 7,617 |  |  |
| 39 | Adachi City | Tokyo | 2 | fluidised bed | 200; 300(3) | 8,411 | 1,132 |  |
| 40 | Itabashi City | Tokyo | 3 | fluidised bed | 250; 250(3); 200 | 24,734 | 3,262 |  |
| 41 | Edogawa City | Tokyo | 3 | fluidised bed | 2 x 300; 300(3) | 32,694 | 5,072 |  |
| 42 | Ota City | Tokyo | 6 | fluidised bed | 6 x 300 | 68,305 | 10,650 |
| 43 | Koto City | Tokyo | 3 | fluidised bed | 3 x 300 | 37,127 | 6,125 |  |
| 44 | Hachioji (Kitanomachi) | Tokyo | 2 | fluidised bed | 2 x 60 | 896 | 142 |  |
| 45 | Tachikawa | Tokyo | 2 | fluidised bed | 2 x 45 | 2,878 | 258 |  |
| 46 | Machida (Minaminaruse) | Tokyo | 2 | fluidised bed | 2 x 50 | 5,611 | 501 |  |
| 47 | Machida (Miwamidoriyama) | Tokyo | 2 | fluidised bed | 30; 60 | 3,219 | 349 |  |
| 48 | Hiratsuka | Kanagawa | 1 | fluidised bed | 120 | 5,002 | 1,427 |  |
| 49 |  |  | 2 | other | 2 x 100 | 8,225 |  |  |
| 50 | Chigasaki | Kanagawa | 4 | fluidised bed | 120; 3 x 180 | 30,707 | 2,206 |  |
| 51 | Odawara (Nishisakawa) | Kanagawa | 1 | fluidised bed | 60 | 4,402 | 412 |  |
| 52 | Odawara (Ogicho) | Kanagawa | 2 | fluidised bed | 2 x 30 | 1,998 | 188 |  |
| 53 | Tsurumi Ward | Kanagawa | 2 | fluidised bed | 150; 200 | 15,425 | 4,683 |  |
| 54 | Kanazawa Ward | Kanagawa | 2 | fluidised bed | 2 x 200 | 11,782 | 4,147 |  |
| 55 | Kawasaki Ward | Kanagawa | 4 | fluidised bed | 4x40 | 30,878 | 4,243 |  |
| 56 | Yokosuka | Kanagawa | 2 | fluidised bed | 40; 90 | 6,777 | 826 |  |
| 57 | Kamakura | Kanagawa | 1 | fluidised bed | 50 | 3,365 | 237 |  |
| 58 | Fujisawa | Kanagawa | 3 | fluidised bed | 2 x 60; 70 | 8,674 | 782 |  |
| 59 | Yamato | Kanagawa | 2 | fluidised bed | 60; 50 | 4,180 | 188 |  |
| 60 | Hakone | Kanagawa | 1 | fluidised bed | 15 | 186 | 19 |  |
| 61 | Yugawara | Kanagawa | 1 | fluidised bed | 15(5) | 565 | 15 |  |
| 62 | Sado | Niigata | 1 | other | - | 215 | 78 |  |
| 63 | Kanazawa | Ishikawa | 2 | fluidised bed | 80; 70 | 3,263 | 750 |  |
| 64 | Kofu | Yamanashi | 2 | fluidised bed | 50; 60(3) | 4,243 | 612 |  |
| 65 | Suwa | Nagano | 2 | fluidised bed | 2 x 35 | 2,181 | 668 |  |
| 66 | Nagano | Nagano | 2 | fluidised bed | 50 | 4,925 | 910 |  |
| 67 | Ueda City | Nagano Prefecture | 1 | fluidised bed | 25 | 878 | 106 |  |
| 68 | Gifu City (Nishinakajima) | Gifu | 2 | fluidised bed | 50(5) | 2,547 | 564 |  |
| 69 | Gifu City (Minamiuzura) | Gifu | 1 | fluidised bed | 50 | 2,064 | 213 |  |
| 70 | Takayama | Gifu | 1 | fluidised bed | 20 | 768 | 90 |  |
| 71 | Seki | Gifu | 1 | fluidised bed | 30 | 1,262 | 132 |  |
| 72 | Suruga Ward | Shizuoka | 1 | fluidised bed | 100 | 2,853 | 286 |  |
| 73 | Naka Ward | Shizuoka | 1 | fluidised bed | 60 | 1,529 | 183 |  |
| 74 | Minami Ward | Shizuoka | 2 | fluidised bed | 50; 130 | 9,344 | 1,246 |  |
| 75 | Iwata | Shizuoka | 1 | fluidised bed | 35 | 2,282 | 181 |  |
| 76 | Nishio | Aichi | 3 | fluidised bed | 35; 2 x 95 | 12,292 | 2,193 |  |
| 77 | Handa | Aichi | 2 | fluidised bed | 25; 50 | 4,588 | 604 |  |
| 78 | Komaki | Aichi | 1 | fluidised bed | 50 | 3,471 | 669 |  |
| 79 | Toyohashi | Aichi | 2 | fluidised bed | 25; 70 | 3,797 | 844 |  |
| 80 | Minami Ward (Chūji) | Aichi | 2 | fluidised bed | 2 x 160 | 11,052 | 1,474 |  |
| 81 | Minami Ward (Motoshibata Nishimachi) | Aichi | 2 | fluidised bed | 2 x 190 | 16,329 | 2,103 |  |
| 82 | Minato Ward | Aichi | 2 | fluidised bed | 2 x 200 | 17,592 | 1,929 |  |
| 83 | Ichinomiya | Aichi | 1 | fluidised bed | 45 | 1,359 | 151 |  |
| 84 | Chita | Aichi | 2 | fluidised bed | 2 x 19 | 587 | 142 |  |
| 85 | Yokkaichi | Mie | 2 | fluidised bed | 2 x 30 | 2,660 | 398 |  |
| 86 | Kusatsu | Shiga | 2 | fluidised bed | 120; 120(4) | 15,823 | 2,469 |  |
| 87 | Hikone | Shiga | 1 | fluidised bed | 110(4) | 5,025 | 827 |  |
| 88 | Nagaokakyo | Kyoto | 1 | fluidised bed | 50 | 1,857 | 474 |  |
| 89 | Minami Ward | Kyoto | 2 | fluidised bed | 2 x 150 | 10,043 | 8,256 |  |
| 90 |  |  | 2 | grate furnace | 2 x 150 | 10,124 |  |  |
| 91 | Fukuchiyama | Kyoto | 1 | fluidised bed | 50 | 1,287 | 652 |  |
| 92 | Higashiosaka (Kitakōnoikechō) | Osaka | 3 | fluidised bed | 2 x 130; 100 | 12,226 | 2,176 |  |
| 93 | Higashiosaka (Kawamata) | Osaka | 4 | fluidised bed | 3 x 90; 70 | 13,312 | 2,269 |  |
| 94 | Ibaraki | Osaka | 1 | fluidised bed | 100(3) | 6,760 | 778 |  |
| 95 | Hirakata | Osaka | 2 | fluidised bed | 2 x 95 | 7,407 | 781 |  |
| 96 | Takatsuki | Osaka | 2 | fluidised bed | 2 x 90 | 7,131 | 646 |  |
| 97 | Toyonaka | Osaka | 2 | fluidised bed | 50; 110 | 6,792 | 1,790 |  |
| 98 | Osakasayama | Osaka | 1 | fluidised bed | 70 | 3,614 | 348 |  |
| 99 | Matsubara | Osaka | 2 | fluidised bed | 85; 90 | 7,486 | 745 |  |
| 100 | Fujiidera | Osaka | 1 | fluidised bed | 65 | 3,471 | 356 |  |
| 101 | Tadaoka | Osaka | 3 | fluidised bed | 159; 182; 205 | 28,489 | 3,684 |  |
| 102 | Ikeda | Osaka | 1 | fluidised bed | 25(5) | 1,296 | 199 |  |
| 103 | Amagasaki | Hyogo | 3 | fluidised bed | 3 x 200 | 33,178 | 4,113 |  |
| 104 | Ono | Hyogo | 2 | fluidised bed | 2 x 70 | 4,689 | 648 |  |
| 105 | Kakogawa | Hyogo | 2 | fluidised bed | 2 x 90 | 5,545 | 827 |  |
| 106 | Higashinada Ward | Hyogo | 2 | fluidised bed | 2 x 150(5) | 15,573 | 3,751 |  |
| 107 | Akashi | Hyogo | 3 | fluidised bed | 2 x 45; 40 | 4,093 | 535 |  |
| 108 | Uda | Nara | 2 | fluidised bed | 2 x 100 | 7,45 | 874 |  |
| 109 |  |  | 1 | grate furnace | 90 | 289 |  |  |
| 110 | Wakayama (Shioya) | Wakayama | 1 | fluidised bed | 60 | 881 | 378 |  |
| 111 | Wakayama (Mikazura) | Wakayama | 1 | fluidised bed | 45 | 1,792 | 214 |  |
| 112 | Tottori | Tottori | 1 | fluidised bed | 40 | 286 | 389 |  |
| 113 | Matsuyama | Ehime | 1 | fluidised bed | 56 | 2,273 | 1,007 |  |
| 114 | Higashi Ward | Fukuoka | 2 | fluidised bed | 2 x 75 | 7,368 | 2,322 |  |
| 115 | Nishi Ward | Fukuoka | 2 | fluidised bed | 2 x 50 | 4,026 | 1,879 |  |
| 116 |  |  | 2 | other | 2 x 50 | 1,146 |  |  |
| 117 | Miyazaki | Miyazaki | 1 | fluidised bed | 60 | 2,135 | 260 |  |

(1)     ash production – 14,296 tDM; quantity dispatched – 13,065 tDM

- two-stage combustion circulating fluidised bed incinerator

- turbocharged fluidised bed incinerator

- circulating fluidised bed incinerator

- multi-layer burning incinerator

mono-incineration plant

SFiguer 5 State sewage sludge mono-incineration plants (GADM, 2022)

References

Act No. 156/1998 Coll. on fertilisers, supplementary soil substances, supplementary plant preparations and substrates and on agrochemical soil testing (Act on fertilisers) [Zákon č. 156/1998 Sb.o hnojivech, pomocných půdních látkách, rostlinných biostimulantech a substrátech a o agrochemickém zkoušení zemědělských půd ] (consolidated version 16 July 2022), 2022.

Act No. 542/2020 Coll. on waste (Waste Act) [Zákon č. 541/2020 Sb. o odpadech] (consolidated version 01 February 2022), 2022.

Act on Special Measures against Dioxins No. 105 of 1999 [平成十一年法律第百五号 ダイオキシン類対策特別措置法] (revised version 2022), 2022.

Council Directive of 12 June 1986 on the protection of the environment, and in particular of the soil, when sewage sludge is used in agriculture (consolidated version 01 January 2022), 2022.

EBI, 2023. Sewage Sludge as feedstock for pyrolysis to be included in the scope of the EU Fertilizing Products Regulation.

Environmental Agency Notification No. 46 August 23, 1991 [平成３年８月23日 環境庁告示第46号] (revised version 2020), 2020.

Environmental quality standards regarding water pollution No. 59 (水質汚濁に係る環境基準について 公布日：昭和 46 年 12 月 28 日 環境庁告示 59) (version 2012), 2012.

Fertilizer Control Act (Act No. 127 of 1950 [肥料取締法（昭和二十五年法律第百二十七号] (version Act No. 8 of 2007), 2007.

GADM, 2022. Download GADM data (version 4.1) – Japan – level 1 [WWW Document]. URL https://gadm.org/download_country.html

Huygens, D., Delgado Sancho, L., Saveyn, H.G.M., Tonini, D., Eder, P., European Commission, Joint Research Centre, 2019. Technical proposals for selected new fertilising materials under the Fertilising Products Regulation (Regulation (EU) 2019/1009): process and quality criteria, and assessment of environmental and market impacts for precipitated phosphate salts & derivates, thermal oxidation materials & derivates and pyrolysis & gasification materials.

JSWA, 2022. Sewer Statistics No. 77 [下 水 道 統 計 第 77 号]. JSWA, Tokio.

JSWA, 2018. Sewer Statistics No. 72 [下 水 道 統 計 第 72 号]. JSWA, Tokio.

Lacko, J., 2023. RCzechia: Spatial Objects of the Czech Republic. Journal of Open Source Software 8, 5082. https://doi.org/10.21105/joss.05082

MAFF, 2022a. Response to Radionuclide Contamination in Foods after the Nuclear Power Plant Accident : MAFF [WWW Document]. URL https://www.maff.go.jp/e/policies/food_safety/response.html (accessed 12.13.22).

MAFF, 2022b. Handling points [取扱いのポイント：農林水産省] [WWW Document]. URL https://www.maff.go.jp/j/syouan/nouan/kome/k_hiryo/caesium/point.html?fbclid=IwAR3x7jT6RdkMlYh7FA3YyRUnL20OaSCqTS8D2_srTna_E0iO0E0fm7hg1J8 (accessed 12.21.22).

MAFF, 2012. Basic knowledge about sludge fertilizer (for the general public) [汚泥肥料に関する基礎知識（一般向け） ：農林水産省 ] [WWW Document]. URL https://www.maff.go.jp/j/kanbo/joho/saigai/odei_qa.html (accessed 12.13.22).

Ministerial  Regulation Defining Judgment Criteria for Industrial Waste Containing Metals (Prime Minister’s Office Regulation No. 5 of 1973) [金属等を含む産業廃棄物に係る判定基準を定める省令（昭和四十八年総理府令第五号] (consolidated version 01 October 2017), 2017.

Regulation (EU) 2019/1009 of the European Parliament and of the Council of 5 June 2019 laying down rules on the making available on the market of EU fertilising products and amending Regulations (EC) No 1069/2009 and (EC) No 1107/2009 and repealing Regulation (EC) No 2003/2003 (consolidated version: 03 October 2022), 2022.

Regulation No. 273/2021 Coll. on details of waste management [Vyhláška č. 273/2021 Sb. o podrobnostech nakládání s odpady] (consolidated version 01 January 2023), 2023.

Regulation No. 474/2000 Coll. of the Ministry of Agriculture on the specification of requirements for fertilisers [Vyhláška Ministerstva zemědělství č. 474/2000 Sb., o stanovení požadavků na hnojiva] (consolidated version 01 November 2021), 2021.

Testing methods for polychlorobiphenyl in industrial water and wastewater [JIS K 0093:2006, 工業用水・工場排水中のポリクロロビフェニル（ＰＣＢ）試験方法] (version 2020), 2020.