SUPPLEMENTARY MATERIAL

Waste Glass Fibre Composites Valorization Using the Fluidised Bed: A Global Warming Potential and Economic Assessment

Journal of Material Cycles and Waste Management

K. Pender, L. Yang

Table 1 Filler/fibre ratio and corresponding GRP constituent weight fractions used throughout the investigation

| Filler/fibre ratio | Weight fraction (%) |  |  |
| --- | --- | --- | --- |
|  | Polymer | Fibre | Filler |
| 0.0 | 30 | 70 | 0 |
| 0.5 | 30 | 47 | 23 |
| 1.0 | 30 | 35 | 35 |
| 1.5 | 30 | 28 | 42 |
| 2.0 | 30 | 23 | 47 |
| 2.5 | 30 | 20 | 50 |

Table 2 Composition of model resin systems used for GWP analysis.

| Model resin system | GWPresin (kg CO2 eq./kg resin) | Resin component | Chemical formula | Fraction of resin |
| --- | --- | --- | --- | --- |
| Epoxy | 3.30 | Bisphenol A diglycidyl ether | C21H24O4 | 0.87 |
|  |  | Isophorone Dianmine | C10H22N2 | 0.13 |
| Polyester | 2.60 | Unsaturated polyester resin | C18H18O9 | 0.59 |
|  |  | Styrene | C8H8 | 0.40 |
|  |  | MEKP | C8H18O6 | 0.01 |
| Vinyl ester [1] | 2.95 | Bisphenol A diglycidyl ether | C21H24O4 | 0.36 |
|  |  | Bisphenol A | C15H16O2 | 0.10 |
|  |  | Methacrylic acid | C4H6O2 | 0.09 |
|  |  | Styrene | C8H8 | 0.44 |
|  |  | MEKP | C8H18O6 | 0.01 |
| Phenolic [2] | 3.18 | Phenol | C6H6O | 0.69 |
|  |  | Formaldehyde | CH2O | 0.18 |
|  |  | Hexamine | C6H12N4 | 0.13 |

Table 3 Cost and revenue input data for FBR plant financial model.

| Capital costs |  |
| --- | --- |
| Working capital | 15% of plant capital |
| Capital investment | Plant capital + working capital |
| Direct costs |  |
| Electricity (Fans and downsizing) | 0.15 $/kWh [3] (Quantity determined from energy analysis) |
| Natural gas | 0.024 $/kWh [4] (Quantity determined from energy analysis) |
| Misc. materials | 10% of maintenance |
| Maintenance | 5% of capital cost |
| Operating labour | Assumed 3 staff during operation @ 31.32 $/h [5] |
| Supervision | 15% operating labour |
| Lab charges | 10% operating labour |
| Scrubber operation and maintenance | Annual cost: 32000 $/m3s [6] |
| Indirect costs |  |
| Plant overheads | 60% operating labour |
| Insurance | 0.5% of capital cost |
| Admin | 25% of plant overheads |
| Distribution | 5% of total indirect expenses |
| R & D | 5% of total indirect expenses |
| GRP waste transport | Waste stream radius = 125 mile Transport cost = 3.97 $/mile [7] |
| Revenue |  |
| Gate fee (av. Dec 2020 UK landfill cost incl. tax) | 154 $/tonne [8] |
| Process steam | 0.009 $/kg |
| Filler (CaCO3) | 50 $/tonne |
| Recycled fibre | Resale price est. as % of vGF cost (1 $/kg vGF) |
| Exchange rates | 1.25 $/£ |
|  | 1.15 $/€ |

# References

| [1] | A. Kandelbauer , G. Tondi and S. H. Goodman , Handbook of Thermoset Plastics: 6. Unsaturated Polyesters and Vinyl Esters, William Andrew, 2013. |
| --- | --- |
| [2] | A. Pizzi and C. C. Ibeh, Handbook of Thermoset Plastics: Phenol–Formaldehydes., 2014. |
| [3] | Statista, “Electricity prices for industry in the United Kingdom (UK) from 2008 to 2019,” 2020. |
| [4] | Energy and Environmental Services, “Prices of natural gas for industry in the United Kingdom (UK) from 2008 to 2018,” Statista, 2019. |
| [5] | Eurostat Statistics Explained, “Estimated hourly labour costs,” 2019. |
| [6] | EPA, “Air pollution control technology fact sheet - Spray tower scrubber,” 2003. |
| [7] | C. Barnett, “Cost tables,” MotorTansport, 2018. |
| [8] | letsrecycle, “EfW, landfill, RDF 2020 gate fees,” 2020. [Online]. Available: https://www.letsrecycle.com/prices/efw-landfill-rdf-2/efw-landfill-rdf-2020-gate-fees/. |