Electronic Supplementary Material

Table S1. Characteristics of wildlife crossing structures for mammals on the A4 motorway in Lower Silesian Forest (section Zgorzelec-Krzyżowa) monitored from 2010 to 2013.

| Code | Type | Total width or span [m] | Functional width/span (excluding rivers or fences) [m] | Height [m] | Length [m] | Entrance slope for overpass or openness index [(width × height)/length] for underpass | Remarks | Distance to human settlements [m] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PZ23 | Overpass | 30 | 28.5 | - | 200 | 15° | Integrated with a forest gravel road (width 4.5 m). Planted with shrubs and trees of native species, spontaneously overgrown by herbaceous vegetation. Both entrances connected with mosaic of woods and fields, but crossed by narrow technical roads. | 1460 |
| PZ28 | Overpass | 30 | 29 | - | 230 | 15° | Integrated with a forest gravel road (width 5 m). Planted with shrubs and trees of native species, spontaneously overgrown by herbaceous vegetation. Both entrances connected with forests, but crossed by gravel technical roads. | 1320 |
| PZ34 | Overpass | 45 | 45 | - | 210 | 15° | Integrated with a forest gravel road (width 6 m). Planted with shrubs and trees of native species, spontaneously overgrown by herbaceous vegetation. Both entrances connected with forests, but crossed by gravel technical roads. | 2620 |
| PZ47 | Overpass | 45 | 45 | - | 210 | 15° | Integrated with a forest gravel road (width 5.7 m). Planted with shrubs and trees of native species, spontaneously overgrown by herbaceous vegetation. Both entrances connected with forests, on one side crossed by a gravel technical roads. | 590 |
| PZ51 | Overpass | 40 | 40 | - | 110 | 40° | Planted with shrubs and trees of native species, spontaneously overgrown by herbaceous vegetation. One entrance connected with large forest, second entrance with mosaic of woods and fields. | 1290 |
| PZ52 | Overpass | 40 | 40 | - | 110 | 40° | Planted with shrubs of native species, spontaneously overgrown by herbaceous vegetation. Entrances connected with mosaic of woods and fields, a large pine forest in vicinity. | 560 |
| MA16 | Large underpass: extended bridge | 33 | 29 (2×14.5m on river sides) | 4.7 | 100 | - | Embankments of a river reinforced with concrete grids, the ground on both river sides is covered with grass. Both entrances connected with mosaic of small woods and fields. | 600 |
| MA25 | Large underpass: extended bridge | 52 | 46 (2×23m on both river sides) | 5.1 | 120 | - | Embankments of a watercourse reinforced with willow spilling, the ground on both sides is spontaneously overgrown by herbaceous vegetation. Both entrances connected with mosaic of small woods, pastures and fields, crossed on one side by a local road. | 370 |
| MA31 | Large underpass: extended bridge | 114 | 103 (2×50m on river sides) | 4.6 | 70 | - | Embankments of a big river reinforced with concrete grids, the ground not covered with grass. Both entrances connected with river valley, mosaic of fields, meadows and bushes, crossed by local roads. | 260 |
| MA42 | Large underpass: extended bridge | 114 | 93 (2×46.5m on both sides of a watercourse) | 5.4 | 110 | - | Embankments of a big river reinforced with concrete grids, along the watercourse goes a gravel road, the ground covered with herbaceous vegetation in 30%. One entrance connected with pine forest, the second bordering with a village. There is a pond for rainwater in vicinity. | 260 |
| MA46 | Large underpass: extended bridge | 51 | 45.5 (2×22.75m on both sides of a watercourse) | 2.7 – 3.8 | 35 | - | Embankments of a river reinforced with concrete grids, the ground covered with herbaceous vegetation in 50%. Both entrances connected with river valley, mosaic of fields, meadows, bushes and buildings. There is a big pond for rainwater in vicinity. | 290 |
| MA15 | Small underpass: extended bridge | 15 | 10 (2×5m on both sides of a watercourse) | 7.9 | 80 | - | Embankments of a small watercourse reinforced with willow spilling, the ground on both sides is covered with grass. Both entrances connected with mosaic of small woods and fields. | 130 |
| MA35 | Small underpass: extended bridge | 16.3 | 6 | 3.7 | 100 | - | Embankments of a seasonal watercourse reinforced with concrete grids, along the watercourse goes a gravel road, soil poorly covered with vegetation. Both entrances connected with a pine forest, but on their sides two fenced ponds for rainwater. | 2050 |
| PZ38 | Small underpass | 19 | 19 | 4.5 | 70 | 1.2 | Integrated with a forest gravel road (width 5.5 m) and two drainage ditches along the road. Ground covered with grass in 40%. Both entrances connected with large pine forest, but crossed by local dirt roads. | 1940 |
| PZ40 | Small underpass | 15 | 15 | 4.7 | 70 | 1.0 | Integrated with a forest gravel road (width 6.5 m) and two drainage ditches along the road. Ground covered with grass in 40%. Both entrances connected with large pine forest, but crossed by local dirt roads. | 2250 |

Table S2. Numbers of crossings by animals from different species and by people recorded with sand-beds on 15 wildlife crossing structures on the A4 motorway (section Zgorzelec-Krzyżowa) in the Lower Silesian Forest, 2010-2013. Detailed description of wildlife crossing structures in Table S1.

| Species | Overpasses |  |  |  |  |  | Large underpasses (>20 m) |  |  |  |  | Small underpasses (<20 m) |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PZ23 | PZ28 | PZ34 | PZ47 | PZ51 | PZ52 | MA16 | MA25 | MA31 | MA42 | MA46 | MA15 | MA35 | PZ38 | PZ40 |  |
| Red deer | 562 | 1 196 | 835 | 465 | 285 | 10 | 6 | 0 | 14 | 149 | 8 | 0 | 0 | 4 | 6 | 3 540 |
| Roe deer | 555 | 440 | 241 | 968 | 834 | 1 143 | 462 | 425 | 698 | 622 | 967 | 365 | 112 | 75 | 18 | 7 925 |
| Fallow deer | 0 | 22 | 0 | 0 | 79 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 101 |
| Wild boar | 329 | 959 | 165 | 1 242 | 1 131 | 641 | 6 | 2 | 21 | 0 | 0 | 0 | 0 | 0 | 0 | 4 496 |
| European hare | 209 | 122 | 124 | 238 | 331 | 186 | 201 | 277 | 443 | 311 | 214 | 38 | 265 | 90 | 60 | 3 109 |
| Grey wolf | 51 | 29 | 38 | 20 | 47 | 0 | 0 | 1 | 8 | 5 | 0 | 6 | 10 | 8 | 2 | 225 |
| Red fox/racoon dog | 137 | 71 | 100 | 95 | 85 | 140 | 74 | 34 | 47 | 67 | 59 | 16 | 56 | 29 | 9 | 1 019 |
| Eurasian badger | 20 | 10 | 4 | 5 | 18 | 40 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 99 |
| Martens/polecat | 6 | 4 | 0 | 3 | 0 | 1 | 14 | 18 | 36 | 24 | 44 | 65 | 5 | 4 | 3 | 227 |
| Otter | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 3 | 0 | 17 | 0 | 0 | 0 | 22 |
| Wild animals total | 1 869 | 2 853 | 1 507 | 3 036 | 2 810 | 2 161 | 764 | 757 | 1 269 | 1 181 | 1 292 | 507 | 449 | 210 | 98 | 20 763 |
| Cattle | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 60 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 80 |
| Horse | 0 | 0 | 25 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 52 | 24 | 0 | 101 |
| Dog | 106 | 74 | 107 | 167 | 93 | 132 | 386 | 482 | 579 | 711 | 606 | 575 | 136 | 136 | 33 | 4 323 |
| Cat | 5 | 6 | 10 | 28 | 8 | 48 | 73 | 184 | 80 | 85 | 199 | 191 | 28 | 30 | 35 | 1 010 |
| Domestic animals total | 111 | 80 | 142 | 195 | 101 | 180 | 459 | 726 | 679 | 796 | 805 | 766 | 216 | 190 | 68 | 5 514 |
| People | 479 | 961 | 1 052 | 573 | 322 | 544 | 742 | 1 261 | 1 275 | 2 005 | 1 327 | 635 | 1 259 | 1220 | 349 | 14 004 |
| Total | 2 459 | 3 894 | 2 701 | 3 804 | 3 233 | 2 885 | 1 965 | 2 744 | 3 223 | 3 982 | 3 424 | 1 908 | 1 924 | 1620 | 515 | 40 281 |

Table S3. Numbers of crossings by animals from different species and by people recorded with camera traps on 15 wildlife crossing structures on the A4 motorway (section Zgorzelec-Krzyżowa) in the Lower Silesian Forest, 2010-2013. Detailed description of WCS in Table S1.

| Species | Overpasses |  |  |  |  |  | Large underpasses (>20 m) |  |  |  |  | Small underpasses (<20 m) |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PZ23 | PZ28 | PZ34 | PZ47 | PZ51 | PZ52 | MA16 | MA25 | MA31 | MA42 | MA46 | MA15 | MA35 | PZ38 | PZ40 |  |
| Red deer | 176 | 305 | 289 | 152 | 152 | 17 | 0 | 0 | 0 | 4 | 0 | 1 | 0 | 6 | 0 | 1 102 |
| Roe deer | 322 | 117 | 30 | 401 | 197 | 558 | 67 | 21 | 113 | 39 | 201 | 46 | 4 | 15 | 31 | 2 179 |
| Fallow deer | 0 | 0 | 4 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 |
| Wild boar | 296 | 770 | 26 | 525 | 994 | 238 | 7 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 858 |
| European hare | 119 | 25 | 34 | 66 | 143 | 63 | 1 | 36 | 118 | 5 | 2 | 2 | 2 | 18 | 17 | 651 |
| Grey wolf | 35 | 35 | 8 | 7 | 46 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 3 | 6 | 152 |
| Red fox/racoon dog | 130 | 17 | 15 | 22 | 36 | 63 | 19 | 14 | 16 | 2 | 3 | 2 | 3 | 9 | 4 | 360 |
| Eurasian badger | 4 | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 8 |
| Martens/polecat | 11 | 5 | 0 | 2 | 0 | 21 | 2 | 3 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 45 |
| Otter | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| Wild animals total | 1093 | 1274 | 406 | 1175 | 1572 | 960 | 96 | 76 | 247 | 50 | 207 | 51 | 16 | 51 | 59 | 7361 |
| Cattle | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 |
| Horse | 0 | 0 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 2 | 0 | 17 |
| Dog | 26 | 0 | 20 | 6 | 11 | 19 | 1 | 20 | 40 | 11 | 17 | 47 | 4 | 52 | 10 | 284 |
| Cat | 1 | 7 | 4 | 10 | 6 | 8 | 1 | 22 | 3 | 0 | 1 | 17 | 2 | 7 | 17 | 113 |
| Domestic animals total | 27 | 7 | 34 | 16 | 17 | 27 | 2 | 45 | 43 | 11 | 18 | 64 | 11 | 61 | 27 | 417 |
| People | 214 | 253 | 577 | 189 | 98 | 208 | 132 | 421 | 614 | 163 | 73 | 148 | 509 | 941 | 784 | 5 586 |
| Total | 1 334 | 1 534 | 1 017 | 1 380 | 1 687 | 1 195 | 230 | 542 | 904 | 224 | 298 | 263 | 536 | 1 053 | 870 | 13 364 |

Table S4. Crossing rates (mean number of crossings by animals of different species per day) ± standard deviation, and constancy (in brackets) of species using 15 wildlife crossing structures on the A4 motorway (section Zgorzelec-Krzyżowa) in the Lower Silesian Forest, 2010-2013, based on tracks registered on sand-beds.

| Wildlife crossing structure | Wild-living mammals |  |  |  |  |  |  |  |  |  | Domestic animals |  |  |  | Humans |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Red deer | Roe deer | Fallow deer | Wild boar | European hare | Grey wolf | Red fox/Racoon dog | Badger | Martens/ Polecat | Otter | Horse | Cattle | Dog | Cat |  |
| Overpasses |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PZ23 | 1.1±1.5 (42.2) | 1.5±1.5 (73.4) |  | 1.1±2.2 (32.8) | 0.5±1.0 (39.1) | 0.3±0.8 (12.5) | 0.4±0.6 (46.9) | 0.02±0.1 (3.1) | 0.01±0.1 (1.6) |  |  |  | 0.3±0.9 (21.9) | 0.03±0.3 (1.6) | 1.1±1.0 (73.4) |
| PZ28 | 2.9±2.3 (81.3) | 1.4±1.5 (64.1) | 0.1±0.6 (4.7) | 3.1±6.2 (54.7) | 0.4±0.8 (25.0) | 0.1±0.4 (6.3) | 0.2±0.4 (26.6) | + | 0.02±0.1 (3.1) |  |  |  | 0.2±0.5 (10.9) | 0.03±0.3 (1.6) | 2.7±2.7 (87.5) |
| PZ34 | 1.8±2.5 (51.6) | 0.7±1.2 (34.4) |  | 0.3±0.7 (17.2) | 0.3±0.6 (29.7) | 0.1±0.3 (17.2) | 0.4±0.6 (37.5) | 0.01±0.1 (1.6) |  |  | 0.1±0.4 (6.3) |  | 0.4±0.7 (28.1) | 0.02±0.1 (3.1) | 3.0±3.0 (84.4) |
| PZ47 | 0.6±1.4 (29.7) | 1.8±2.0 (56.3) |  | 1.5±2.9 (37.5) | 0.8±1.8 (26.6) | 0.03±0.1 (4.7) | 0.3±0.5 (34.4) | 0.01±0.1 (1.6) | 0.02±0.1 (3.1) |  |  |  | 0.6±1.1 (28.1) | 0.1±0.5 (4.7) | 1.3±2.5 (43.8) |
| PZ51 | 0.8±1.8 (25.0) | 2.2±2.1 (79.7) | 0.3±0.8 (15.6) | 2.8±3.6 (67.2) | 1.4±1.8 (60.9) | 0.2±0.7 (10.9) | 0.3±0.5 (29.7) | 0.02±0.1 (1.6) |  |  |  |  | 0.3±0.6 (26.6) | 0.03±0.2 (3.1) | 1.0±2.0 (60.9) |
| PZ52 | 0.04±0.2 (3.1) | 2.4±1.4 (89.1) |  | 1.3±2.2 (39.1) | 0.6±0.9 (43.8) |  | 0.4±0.5 (43.8) | 0.1±0.2 (15.6) | + |  |  |  | 0.4±1.0 (28.1) | 0.1±0.3 (10.9) | 1.6±1.9 (73.4) |
| Large underpasses (>20 m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MA16 | 0.01±0.1 (1.6) | 1.1±1.0 (68.8) |  | 0.01±0.1 (1.6) | 0.6±0.8 (51.6) |  | 0.1±0.3 (20.3) | + | 0.04±0.2 (6.3) |  |  |  | 1.1±1.3 (65.6) | 0.2±0.5 (15.6) | 1.5±2.1 (60.9) |
| MA25 |  | 1.1±1.5 (54.7) |  | + | 0.9±1.2 (50.0) | + | 0.2±0.3 (25.0) |  | 0.1±0.2 (10.9) |  |  | 0.1±0.4 (4.7) | 1.4±1.3 (70.3) | 0.6±0.7 (51.6) | 2.6±3.0 (75.0) |
| MA31 | 0.02±0.1 (1.6) | 1.8±1.9 (75.0) |  | + | 2.1±2.5 (64.1) | 0.005±0.04 (1.6) | 0.1±0.3 (12.5) |  | 0.1±0.3 (7.8) | + |  | 0.1±0.4 (3.1) | 1.9±2.2 (68.8) | 0.2±0.5 (21.9) | 3.0±3.0 (73.4) |
| MA42 | 0.2±0.8 (9.4) | 1.6±1.8 (64.1) |  |  | 1.3±1.7 (64.1) | 0.02±0.1 (1.6) | 0.2±0.5 (23.4) | 0.1±0.2 (9.4) | + | + |  |  | 2.4±2.3 (76.6) | 0.2±0.5 (20.3) | 4.7±4.2 (85.9) |
| MA46 | + | 2.7±2.3 (76.6) |  |  | 0.8±1.1 (54.7) |  | 0.1±0.3 (9.4) |  | 0.1±0.2 (14.1) |  |  |  | 2.2±2.0 (81.3) | 0.6±0.8 (46.9) | 2.9±3.1 (70.3) |
| Small underpasses (<20 m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MA15 |  | 0.7±0.8 (56.3) |  |  | 0.2±0.5 (15.6) | 0.02±0.1 (1.6) | 0.05±0.2 (7.8) |  | 0.1±0.2 (14.1) | 0.04±0.2 (6.3) |  |  | 1.7±1.3 (85.9) | 0.7±0.8 (53.1) | 1.5±2.1 (70.3) |
| MA35 |  | 0.1±0.4 (12.5) |  |  | 1.0±1.5 (65.6) | 0.04±0.2 (4.7) | 0.2±0.3 (29.7) | 0.01±0.1 (1.6) | + |  | 0.1±0.3 (7.8) |  | 0.2±0.5 (21.9) | 0.03±0.2 (3.1) | 3.5±3.2 (89.1) |
| PZ38 | + | 0.1±0.3 (10.9) |  |  | 0.3±0.6 (26.6) | 0.03±0.2 (4.7) | 0.1±0.3 (9.4) |  | 0.02±0.1 (3.1) |  | 0.1±0.4 (6.3) |  | 0.4±0.6 (39.1) | 0.04±0.2 (6.3) | 3.4±2.5 (89.1) |
| PZ40 | + | 0.03±0.2 (3.1) |  |  | 0.2±0.5 (20.3) | 0.01±0.1 (1.6) | 0.1±0.2 (12.5) |  | + |  |  |  | 0.1±0.3 (7.8) | 0.1±0.3 (12.5) | 1.1±2.0 (34.4) |

Table S5. Results of calculation of similarity indices for community of wild species using: (A) overpasses (Community 1) and underpasses (Community 2); (B) overpasses (Community 1), large underpasses (Community 2) and small underpasses (Community 3). Data based on tracks recorded on sand-beds, 2010-2013. Calculations performed in SpadeR online (https://chao.shinyapps.io/SpadeR/, Chao et al., 2015).

(A) Estimated similarity indices for overpasses (n1) and underpasses (n2)

BASIC DATA INFORMATION:

The loaded set includes abundance/incidence data from 2 communities and a total of 10 species.

Samples size in Community 1: n1  = 14236

Samples size in Community 2: n2  = 6527

Number of observed species in Community 1: D1  = 9

Number of observed species in Community 2: D2  = 9

Number of observed shared species in two communities: D12 = 8

Number of bootstrap replications for s.e. estimate: 100

| Similarity indices and q value | Estimate | s.e. | 95%Lower | 95%Upper |
| --- | --- | --- | --- | --- |
| Classic richness-based similarity |  |  |  |  |
| Sorensen (q=0) | 0.8889 | 0.0228 | 0.8442 | 0.9336 |
| Jaccard (q=0) | 0.8000 | 0.0349 | 0.7316 | 0.8684 |
| Measures for comparing species relative abundances |  |  |  |  |
| Horn (q=1) | 0.6916 | 0.0047 | 0.6823 | 0.7009 |
| Morisita-Horn (q=2) | 0.6104 | 0.0070 | 0.5967 | 0.6241 |
| Regional overlap (q=2) | 0.7581 | 0.0054 | 0.7475 | 0.7686 |
| ChaoJaccard-abundance | 0.9896 | 0.0000 | 0.9896 | 0.9896 |
| ChaoSorensen-abundance | 0.9948 | 0.0000 | 0.9948 | 0.9948 |
| Measures for comparing size-weighted species relative abundances |  |  |  |  |
| Horn size-weighted (q=1) | 0.7204 | 0.0047 | 0.7113 | 0.7296 |
| Measures for comparing species absolute abundances |  |  |  |  |
| Morisita-Horn (q=2) | 0.5543 | 0.0070 | 0.5406 | 0.5680 |
| Regional overlap (q=2) | 0.7132 | 0.0058 | 0.7019 | 0.7246 |
| Bray-Curtis | 0.5411 | 0.0049 | 0.5315 | 0.5507 |

(B) Estimated similarity indices for overpasses (n1), large underpasses (n2) and small underpasses (n3)

BASIC DATA INFORMATION:

The loaded set includes abundance/incidence data from 3 communities and a total of 10 species.

Sample size in each community:  n1=14236; n2=5263; n3=1264

Number of observed species in one community: D1=9; D2=9; D3=8

Number of observed shared species in two communities: D12=8; D13=7; D23=8

Number of observed shared species in three communities: D123=7

Number of bootstrap replications for s.e. estimate: 100

| Similarity indice and q value | Estimate | s.e. | 95%Lower | 95%Upper |
| --- | --- | --- | --- | --- |
| Classic richness-based similarity |  |  |  |  |
| C03 (q=0, Sorensen) | 0.9231 | 0.0175 | 0.8887 | 0.9574 |
| U03 (q=0, Jaccard) | 0.8000 | 0.0368 | 0.7280 | 0.8720 |
| Measures for comparing species relative abundances |  |  |  |  |
| C13=U13 (q=1, Horn) | 0.7821 | 0.0039 | 0.7745 | 0.7896 |
| C23 (q=2, Morisita-Horn | 0.7317 | 0.0062 | 0.7196 | 0.7438 |
| U23 (q=2, Regional overlap) | 0.8911 | 0.0031 | 0.8851 | 0.8971 |
| Measures for comparing size-weighted species relative abundances |  |  |  |  |
| Horn size-weighted (q=1 | 0.7687 | 0.0036 | 0.7617 | 0.7757 |
| Measures for comparing species absolute abundances |  |  |  |  |
| C23 (Morisita-Horn) | 0.3386 | 0.0038 | 0.3312 | 0.3460 |
| U23 (Regional overlap) | 0.6057 | 0.0040 | 0.5978 | 0.6135 |
| Bray-Curtis | 0.7308 | 0.0206 | 0.6904 | 0.7711 |
| Estimated pairwise similarity |  |  |  |  |
| n1, n2 | 0.889 | 0.033 | 0.824 | 0.954 |
| n1, n3 | 0.824 | 0.036 | 0.753 | 0.894 |
| n2, n3 | 0.941 | 0.036 | 0.870 | 1.000 |

NOTE: Any estimate greater than 1 is replaced by 1; any estimate less than 0 is replaced by 0.

Table S6. Use of 15 wildlife crossing structures by the most abundant wild animals, people and domestic animals in the Lower Silesian Forest, 2010-2013. Mean number of crossings by animals or people per day (crossing rate) (±SE) and range. Data based on sand-beds (see Table S2 and S4 for details). Statistical significance of Mann-Whitney U test for overpasses (O) versus large underpasses (LU), small underpasses (SU) and all underpasses (AU), as well as LU versus SU.

| Species | Overpasses (n=6) | Underpasses |  |  | ## Statistical significance of Mann-Whitney U test |
| --- | --- | --- | --- | --- | --- |
|  |  | large (>20 m) (n=5) | small (<20 m) (n=4) | total (n=9) |  |
| Roe deer | x=1.66 SE = 0.087 range 0 – 9.5 | x= 1.66 SE = 0.102 range 0 – 10.5 | x= 0.24 SE = 0.035 range 0 – 3.5 | x=1.03 SE = 0.066 range 0 – 10.5 | O vs LU NS O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU < 0.0001 |
| Red deer | x=1.23 SE = 0.103 range 0 – 11.0 | x= 0.05 SE = 0.022 range 0 – 6.0 | x= 0 | x=0.03 SE = 0.012 range 0 – 6.0 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU = 0.01 |
| Wild boar | x=1.68 SE = 0.181 range 0 – 45.0 | x= 0.002 SE = 0.002 range 0 – 0.5 | x= 0 | x=0.001 SE = 0.001 range 0 – 0.5 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU NS |
| Fallow deer | x= 0.07 SE = 0.021 range 0 – 3.0 | x= 0 | x= 0 | x= 0 | O vs LU < 0.001 O vs SU < 0.01 O vs AU < 0.0001 LU vs SU - |
| Ungulates total | x= 4.64 SE = 0.250 range 0 – 59.0 | x= 1.71 SE = 0.107 range 0 – 11.0 | x= 0.24 SE = 0.035 range 0 – 3.5 | x= 1.06 SE = 0.068 range 0 – 11.0 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU <0.0001 |
| European hare | x=0.67 SE = 0.066 range 0 – 8.5 | x= 1.16 SE = 0.091 range 0 – 11.5 | x= 0.43 SE = 0.059 range 0 – 10.5 | x=0.83 SE = 0.059 range 0 – 11.5 | O vs LU < 0.0001 O vs SU NS O vs AU = 0.02 LU vs SU < 0.0001 |
| Wolf | x=0.12 SE = 0.024 range 0 – 2.5 | x= 0.004 SE = 0.003 range 0 – 1.0 | x= 0.03 SE = 0.009 range 0 – 1.5 | x=0.01 SE = 0.005 range 0 – 1.5 | O vs LU < 0.0001 O vs SU < 0.01 O vs AU < 0.0001 LU vs SU = 0.02 |
| Red fox/racoon dog | x=0.33 SE = 0.027 range 0 – 3.5 | x= 0.14 SE = 0.020 range 0 – 2.5 | x= 0.09 SE = 0.015 range 0 – 1.7 | x=0.12 SE = 0.013 range 0 – 2.5 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU NS |
| Eurasian badger | x= 0.03 SE = 0.006 range 0 – 1.0 | x= 0 | x= 0.002 SE = 0.002 range 0 – 0.5 | x= 0.001 SE = 0.001 range 0 – 0.5 | O vs LU < 0.001 O vs SU < 0.01 O vs AU < 0.0001 LU vs SU NS |
| Martens/polecat | x= 0.007 SE = 0.003 range 0 – 0.5 | x= 0.07 SE = 0.013 range 0 – 1.5 | x= 0.03 SE = 0.008 range 0 – 1.0 | x= 0.05 SE = 0.008 range 0 – 1.5 | O vs LU < 0.001 O vs SU < 0.01 O vs AU < 0.0001 LU vs SU NS |
| Otter | x= 0 | x= 0.010 SE = 0.005 range 0 – 1.0 | x= 0 | x= 0.004 SE = 0.002 range 0 – 1.0 | O vs LU NS O vs SU = 0.01 O vs AU NS LU vs SU = 0.03 |
| Mesocarnivores total | x= 0.35 SE = 0.028 range 0 – 3.5 | x= 0.21 SE = 0.025 range 0 – 3.0 | x= 0.13 SE = 0.017 range 0 – 1.7 | x= 0.17 SE = 0.016 range 0 – 3.0 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU NS |
| People | x=1.77 SE = 0.122 range 0 – 18.0 | x= 2.94 SE = 0.186 range 0 – 16.5 | x= 2.38 SE = 0.170 range 0 – 15.0 | x=2.69 SE=0.128 range 0 – 16.5 | O vs LU < 0.0001 O vs SU < 0.01 O vs AU < 0.0001 LU vs SU NS |
| Domestic cat | x=0.06 SE = 0.015 range 0 – 3.0 | x= 0.38 SE = 0.037 range 0 – 3.0 | x= 0.22 SE = 0.033 range 0 – 2.5 | x=0.31 SE = 0.02 range 0 – 3.0 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU < 0.001 |
| Domestic dog | x=0.36 SE = 0.043 range 0 – 7.5 | x= 1.82 SE = 0.108 range 0 – 11.0 | x= 0.60 SE = 0.062 range 0 – 6.5 | x=1.28 SE = 0.071 range 0 – 11.0 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU < 0.0001 |
| Cattle | x= 0 | x= 0.03 SE = 0.0.013 range 0 – 2 | x= 0 | x= 0.02 SE = 0.007 range 0 – 2 | O vs LU = 0.01 O vs SU - O vs AU NS LU vs SU = 0.05 |
| Horse | x= 0.01 SE = 0.009 range 0 – 3.0 | x= 0 | x= 0.04 SE = 0.016 range 0 – 2.0 | x= 0.02 SE = 0.007 range 0 – 2.0 | O vs LU NS O vs SU NS O vs AU = 0.03 LU vs SU < 0.001 |
| Domestic animals total | x= 0.43 SE = 0.050 range 0 – 7.5 | x= 2.23 SE = 0.121 range 0 – 11.0 | x= 0.86 SE = 0.084 range 0 – 8.5 | x= 1.62 SE = 0.082 range 0 – 11.0 | O vs LU < 0.0001 O vs SU < 0.0001 O vs AU < 0.0001 LU vs SU < 0.0001 |

Table S7. Seasonal differences in crossing rates by animals from different species  on 15 wildlife crossing structures on the A4 motorway (section Zgorzelec-Krzyżowa) in the Lower Silesian Forest, 2010-2013. Based on data from sand-beds.

| Species | Spring | Summer | Autumn | Winter | p* |
| --- | --- | --- | --- | --- | --- |
| Roe deer | 1.24 ± 0.095 (0 – 10.5) | 1.45 ± 0.115 (0 – 7.0) | 1.32 ± 0.126 (0 – 10.5) | 1.16 ± 0.099 (0 – 9.5) | 0.04 |
| Red deer | 0.51 ± 0.089 (0 – 11.0) | 0.44 ± 0.083 (0 – 8.0) | 0.73 ± 0.119 (0 – 10.0) | 0.36 ± 0.072 (0 – 8.0) | 0.04 |
| Wild boar | 0.82 ± 0.141 (0 – 18.0) | 0.46 ± 0.107 (0 – 10.0) | 0.74 ± 0.137 (0 – 16.0) | 0.62 ± 0.190 (0 – 45.0) | NS |
| Grey wolf | 0.02 ± 0.013 (0 – 2.5) | 0.03 ± 0.011 (0 – 1.5) | 0.03 ± 0.014 (0 – 2.5) | 0.13 ± 0.030 (0 – 2.5) | 0.0008 |
| European hare | 0.82 ± 0.079 (0 – 7.0) | 0.64 ± 0.064 (0 – 4.0) | 0.79 ± 0.102 (0 – 9.0) | 0.80 ± 0.095 (0 – 11.5) | NS |
| Red fox | 0.16 ± 0.024 (0 – 2.5) | 0.21 ± 0.029 (0 – 2.0) | 0.21 ± 0.027 (0 – 1.5) | 0.24 ± 0.029 (0 – 3.5) | NS |
| People | 2.33 ± 0.152 (0 – 16.0) | 3.45 ± 0.259 (0 – 16.5) | 2.84 ± 0.217 (0 – 18.0) | 1.08 ± 0.083 (0 – 8.0) | < 0.00001 |
| Domestic cat | 0.21 ± 0.030 (0 – 2.5) | 0.31 ± 0.045 (0 – 3.0) | 0.25 ± 0.041 (0 – 3.0) | 0.10 ± 0.020 (0 – 2.0) | 0.002 |
| Domestic dog | 0.70 ± 0.070 (0 – 7.0) | 1.07 ± 0.126 (0 – 9.0) | 1.02 ± 0.103 (0 – 7.5) | 0.92 ± 0.091 (0 – 11.0) | NS |

Seasons: spring (March-May), summer (June-August), autumn (October-November), winter (December-February). Crossing rate=mean number of crossings by animals from particular species per day ± standard deviation (range). * significance of one-way ANOVA on ranks (Kruskal-Wallis H test).

Table S8. Crossing rates of wild animals, domestic animals and people on 15 wildlife crossing structures on the A4 motorway (section Zgorzelec-Krzyżowa) in the Lower Silesian Forest, in consecutive years of monitoring (2010-2013). Based on data from sand-beds.

| Species | 2010-2011 | 2011-2012 | 2012-2013 | p* |
| --- | --- | --- | --- | --- |
| Roe deer | 0.85 ± 0.088 (0 – 8.5) | 1.03 ± 0.075 (0 – 10.5) | 1.79 ± 0.099 (0 – 10.5) | < 0.00001 |
| Red deer | 0.47 ± 0.081 (0 – 8.0) | 0.54 ± 0.082 (0 – 11.0) | 0.50 ± 0.074 (0 – 8.0) | NS |
| Wild boar | 0.28 ± 0.070 (0 – 8.0) | 0.50 ± 0.100 (0 – 18.0) | 1.08 ± 0.166 (0 – 45.0) | < 0.00001 |
| Grey wolf | 0.01 ± 0.005 (0 – 1.0) | 0.03 ± 0.009 (0 – 1.5) | 0.12 ± 0.024 (0 – 2.5) | 0.0001 |
| European hare | 0.51 ± 0.069 (0 – 7.0) | 0.41 ± 0.038 (0 – 4.0) | 1.27 ± 0.092 (0 – 11.5) | < 0.00001 |
| Red fox | 0.18 ± 0.026 (0 – 2.5) | 0.15 ± 0.20 (0 – 3.5) | 0.27 ± 0.024 (0 – 2.5) | 0.0004 |
| People | 1.69 ± 0.163 (0 – 18.0) | 2.47 ± 0.154 (0 – 18.0) | 2.6 ± 0.154 (0 – 16.5) | < 0.00001 |
| Domestic cat | 0.10 ± 0.024 (0 – 2.5) | 0.12 ± 0.020 (0 – 2.0) | 0.35 ± 0.035 (0 – 3.0) | < 0.00001 |
| Domestic dog | 1.23 ± 0.112 (0 – 11.0) | 0.70 ± 0.064 (0 – 6.5) | 0.90 ± 0.079 (0 – 9.0) | 0.0001 |

Monitoring year from June to May. Crossing rate = mean number of crossings by animals of particular species per day ± standard deviation (range). * significance of one-way ANOVA on ranks (Kruskal-Wallis H test).

Table S9. Overlap of daily activity of animals from different species and people using 15 wildlife crossing structures on the A4 motorway (section Zgorzelec-Krzyżowa) in the Lower Silesian Forest, 2010-2013. Calculations with formulas of (A) Pianka (1973) and (B) Czechanowski (Feinsinger et al. 1981). Based on data recorded by camera traps.

(A) Pianka

| Pianka | People | Cat | Dog | Wild boar | Red deer | Roe deer | Hare | Red fox | Wolf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| People | 0 |  |  |  |  |  |  |  |  |
| Cat | 0.790 | 0 |  |  |  |  |  |  |  |
| Dog | 0.908 | 0.761 | 0 |  |  |  |  |  |  |
| Wild boar | 0.169 | 0.537 | 0.249 | 0 |  |  |  |  |  |
| Red deer | 0.258 | 0.554 | 0.312 | 0.943 | 0 |  |  |  |  |
| Roe deer | 0.424 | 0.658 | 0.436 | 0.864 | 0.917 | 0 |  |  |  |
| Hare | 0.191 | 0.550 | 0.277 | 0.941 | 0.953 | 0.909 | 0 |  |  |
| Red fox | 0.505 | 0.505 | 0.528 | 0.881 | 0.907 | 0.896 | 0.874 | 0 |  |
| Wolf | 0.214 | 0.535 | 0.265 | 0.948 | 0.938 | 0.896 | 0.921 | 0.887 | 0 |

(B) Czechanowski

| Czechanowski | People | Cat | Dog | Wild boar | Red deer | Roe deer | Hare | Red fox | Wolf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| People | 0 |  |  |  |  |  |  |  |  |
| Cat | 0.663 | 0 |  |  |  |  |  |  |  |
| Dog | 0.792 | 0.661 | 0 |  |  |  |  |  |  |
| Wild boar | 0.213 | 0.471 | 0.268 | 0 |  |  |  |  |  |
| Red deer | 0.279 | 0.548 | 0.328 | 0.833 | 0 |  |  |  |  |
| Roe deer | 0.407 | 0.619 | 0.439 | 0.709 | 0.794 | 0 |  |  |  |
| Hare | 0.220 | 0.499 | 0.295 | 0.832 | 0.841 | 0.751 | 0 |  |  |
| Red fox | 0.453 | 0.454 | 0.521 | 0.723 | 0.775 | 0.788 | 0.720 | 0 |  |
| Wolf | 0.215 | 0.479 | 0.279 | 0.839 | 0.823 | 0.772 | 0.813 | 0.887 | 0 |