Supplemental Information

Identifying soil N2O sources by combining laboratory experiments with process based-models

Zhifeng Yan a,b,*, Zhaopei Chua, Balázs Grosz c, Baoxuan Changa, Narasinha Shurpali d, Gang Liu e, Zhaolei Li f, Jinsen Zheng g, Si-liang Li a,b,Klaus Butterbach-Bahl h,i

a Institute of Surface-Earth System Science, School of Earth System Science, Tianjin University, Tianjin 300072, China

b Critical Zone Observatory of Bohai Coastal Region, Tianjin Key Laboratory of Earth Critical Zone Science and Sustainable Development in Bohai Rim, Tianjin University, Tianjin 300072, China

c Thünen Institute of Climate-Smart Agriculture, 38116 Braunschweig, Germany

d Natural Resources Institute Finland, Halolantie 31 A, 71750 Maaninka Finland

e College of Management and Economics, Tianjin University, Tianjin, 300072, China

f Key Laboratory of Low-carbon Green Agriculture in Southwestern China, Ministry of Agriculture and Rural Affairs,Interdisciplinary Research Center for Agriculture Green Development in Yangtze River Basin, College of Resources and Environment, Southwest University, Chongqing, China

g Crop, Livestock and Environment Division, Japan International Research Center for Agricultural Sciences, Ohwashi 1-1, Tsukuba, Ibaraki, 305-8686 Japan

h Institute of Meteorology and Climate Research, Atmospheric Environmental Research, Karlsruhe Institute of Technology, Garmisch-Partenkirchen, Germany

i Pioneer Center Land-CRAFT, Agroecology, Aarhus University, Aarhus C, Denmark

∗ Corresponding author:

Zhifeng Yan, Institute of Surface-Earth System Science, School of Earth System Science, Tianjin University, Tianjin 300072, China Email: yanzf17@tju.edu.cn

This Supplementary Materials provide expanded methodological details, supporting analyses, and additional tables and figures that substantiate the core findings presented in the main manuscript.

1. Calculation of NH4+ and DOC concentrations

The adsorbed and dissolved NH4+ are assumed to reach equilibrium and follow the Langmuir model as below (Sieczka & Koda 2016):

| qNH4+= qmaxKLCNH4+1+KLCNH4+ | (S1) |
| --- | --- |

where qNH4+ is the concentration of adsorbed NH4+,  qmax is the maximum adsorption capacity of NH4+, KL is the Langmuir constant, and CNH4+ is the concentration of dissolved NH4+. Moreover, dissolved NH4+ and dissolved NH3 are assumed to reach equilibrium as below (Maggi et al. 2008):

| CNH3,aq= CNH4+10-pHKNH3-NH4+ | (S2) |
| --- | --- |

where CNH3,aq is the concentration of dissolved NH3, and KNH3-NH4+ is the equilibrium constant.

The exchange rate between dissolved organic carbon (DOC) and soil adsorbed organic carbon (SOC), rSOC-DOC, was estimated as below (Yan et al. 2018):

| rSOC-DOC= αθ(CSOC-KSOCCDOC)Kθ+θ | (S3) |
| --- | --- |

where α is the mass transfer coefficient, KSOC is the adsorption and desorption equilibrium constant of DOC, Kθ is the moisture constant, CSOC is the SOC content, and CDOC is the DOC concentration.

2. Calculation of nitrogen flows

During the simulations, the N flows from initial NH4+ to NO2− (FN) and from initial NO3−to NO2− (FD) can be calculated by the following equations,

| FN= j=1Nti=1NzrNH4+-NO2-,i,jViθi∆t | (S4) |
| --- | --- |
| FD= j=1Nti=1NzrNO3--NO2-,D,i,jViθi∆tCNO3,0θV0CNO3,0θV0+j=1Nti=1NzrNO2--NO3-,i,jViθi∆t∙j=1Nti=1NzrNH4+-NO2-,i,jj=1Nti=1Nz((rNH4+-NO2-,i,j+rNO3--NO2-,D,i,j) | (S5) |

where Vi is the volume of ith voxel, V0 is the total volume of soil, θi is the moisture content of ith voxel, θ is the averaged moisture, Nt is the number of time step, Nz is the number of numerical voxels, and ∆t is the time step.

3. Procedures of model parameterization

The values of  μNH4+-NO2-, μNO2--NO,N, μNO-N2O,N, μNO2--NO3-, μNO3--NO2-, μNO2--NO,D, μNO-N2O,D, and μN2O-N2were initially determined by comparing simulated concentrations of NH4+ and NO3−, and N2O production as well as its attributing ratio from denitrification (CD) with measured ones. The value of μNH4+-NO2- was determined by NH4+ concentrations under different moisture conditions. The values of  μNO2--NO3- and μNO3--NO2- were determined by NO3− concentrations under different moisture conditions. The values of μNO2--NO,N and μNO-N2O,N were mainly determined by N2O fluxes under 40% and 60% WFPS.  The values of μNO2--NO,D and μNO-N2O,D were mainly determined by N2O fluxes under 70-90% WFPS. The value of μN2O-N2 was mainly determined by N2O flux under 100% WFPS. Afterwards, the values of μNO2--NO3- and μNO3--NO2- were further adjusted based on the values of CD under different soil moisture conditions. Finally, all the parameters were optimized using Markov chain (MC) approach by allowing the parameter values to change randomly 1000 times from half to double of the initially estimated ones.

Table S1. Parameter values given by literature or assumed

| Symbols | Descriptions | Values | Units | Sources and notes |
| --- | --- | --- | --- | --- |
| KNH4+ | Half-saturated concentration of NH4+ | 1.71×10-4 | Mol/l | (Chang et al. 2022) |
| KNO2-,NO2--NO,N | Half-saturated Concentration of NO2− from NO2− to N2O in the nitrification process | 1.00×10-5 | Mol/l | (Chang et al. 2022) |
| KNO,NO-N2O,N | Half-saturated concentration of NO from NO to N2O in the nitrification process | 8.33×10-4 | Mol/l | (Chang et al. 2022) |
| KNO2-,NO2--NO3- | Half-saturated Concentration of NO2− from NO2− to NO3− | 3.93×10-4 | Mol/l | (Chang et al. 2022) |
| KNO3-,NO3--NO2- | Half-saturated Concentration of NO3− from NO3− to NO2− | 8.33×10-4 | Mol/l | (Chang et al. 2022) |
| KNO2-,NO2--NO,D | Half-saturated concentration of NO2− from NO2− to NO in the denitrification process | 8.33×10-4 | Mol/l | (Chang et al. 2022) |
| KNO,NO-N2O,D | Half-saturated concentration of NO from NO to N2O in the denitrification process | 8.33×10-4 | Mol/l | (Chang et al. 2022) |
| KN2O,N2O-N2 | Half-saturated concentration of N2O | 8.33×10-4 | Mol/l | (Chang et al. 2022) |
| KDOC,NO3--NO2- | Half-saturated Concentration of DOC from NO3− to NO2− | 4.17×10-4 | Mol/l | (Chang et al. 2022) |
| KDOC,NO2--NO,D | Half-saturated Concentration of DOC from NO2− to NO in the HD process | 4.17×10-4 | Mol/l | (Chang et al. 2022) |
| KDOC,NO-N2O,D | Half-saturated Concentration of DOC from NO to N2O in the HD process | 4.17×10-4 | Mol/l | (Chang et al. 2022) |
| KDOC,N2O-N2 | Half-saturated Concentration of DOC from N2O to N2 | 8.34×10-4 | Mol/l | (Chang et al. 2022) |
| KDOC,NO2--NH4+ | Half-saturated Concentration of DOC from NO2− to NH4+ | 4.17×10-4 | Mol/l | (Chang et al. 2022) |
| KDOC,DOC-CO2 | Half-saturated Concentration of DOC from DOC to CO2 | 8.33×10-4 | Mol/l | (Chang et al. 2022) |
| KO2,NH4+-NO2- | Half-saturated Concentration of O2 from NH4+to NO2− | 1.25×10-5 | Mol/l | (Chang et al. 2022) |
| KO2,NO2--NO3- | Half-saturated Concentration of O2 from NO2− to NO3− | 6.88×10-5 | Mol/l | (Chang et al. 2022) |
| KO2,DOC-CO2 | Half-saturated Concentration of O2 from DOC to CO2 | 1.13×10-3 | Mol/l | (Chang et al. 2022) |
| IO2,NO3--NO2-,D | Inhibition constant of O2 from NO3− to NO2− in the denitrification process | 1.49×10-4 | Mol/l | Assumed |
| IO2,NO2--NO | Inhibition constant of O2 from NO2− to NO | 1.49×10-4 | Mol/l | Assumed |
| IO2,NO-N2O | Inhibition constant of O2 from NO to N2O | 1.49×10-4 | Mol/l | Assumed |
| IO2,N2O-N2 | Inhibition constant of O2 from N2O to N2 | 5.95×10-7 | Mol/l | Assumed |
| μDOC-CO2 | Maximum reaction rate of CO2 from DOC | 2.80×10-4 | mol/g/s | Assumed |
| KNH3,eqi | Henry’s law constant of NH3 | 1.46×103 | - | (Sander 2015) |
| KNO,eqi | Henry’s law constant of NO | 0.047 | - | (Sander 2015) |
| KN2O,eqi | Henry’s law constant of N2O | 0.6 | - | (Sander 2015) |
| KN2,eqi | Henry’s law constant of N2 | 0.016 | - | (Sander 2015) |
| KO2,eqi | Henry’s law constant of O2 | 0.032 | - | (Sander 2015) |
| KCO2,eqi | Henry’s law constant of CO2 | 0.82 | - | (Sander 2015) |
| KNH3-NH4+ | Equilibrium constant between NH4 and NH3 | 1.00×109.24 | - | (Maggi et al. 2008) |
| DNH3,0 | Diffusion coefficient of gaseous NH3 in air | 2.28×10-5 | m2/s | (Spiller 1989) |
| DNO,0 | Diffusion coefficient of gaseous NO in air | 2.11×10-5 | m2/s | (Massman 1998) |
| DN2O,0 | Diffusion coefficient of gaseous N2O in air | 1.68×10-5 | m2/s | (Massman 1998) |
| DN2,0 | Diffusion coefficient of gaseous N2 in air | 2.12×10-5 | m2/s | (Cadogan et al. 2014) |
| DO2,0 | Diffusion coefficient of gaseous O2 in air | 2.10×10-5 | m2/s | (Weast 1997) |
| DCO2,0 | Diffusion coefficient of gaseous CO2 in air | 1.92×10-5 | m2/s | (Weast 1997) |
| qmax | Maximum adsorption capacity of NH4+ | 5.00×10-6 | Mole/l | (Sieczka & Koda 2016) |
| KL | Langmuir constant of NH4+ | 7.20×103 | l/Mole | (Sieczka & Koda 2016) |
| α | Mass transfer coefficient of SOC desorption | 2.00×10-6 | 1/s | (Yan et al. 2018) |
| KSOC | Adsorption/desorption equilibrium constant between SOC and DOC | 0.039 | l/g | (Yan et al. 2018) |
| Kθ | Moisture constant | 0.10 | m3/m | (Yan et al. 2018) |
| XHET,0 | Initial biomass concentration of heterotroph | 5.3×10-3 | g/l | (Maggi et al. 2008) |
| XAOB,0 | Initial biomass concentration of ammonia-oxidizing bacteria | 2.57×10-2 | g/l | (Maggi et al. 2008) |
| XNOB,0 | Initial biomass concentration of nitrite-oxidizing bacteria | 5.0×10-4 | g/l | (Maggi et al. 2008) |
| XDEN,0 | Initial biomass concentration of denitrifiers | 6.0×10-3 | g/l | (Maggi et al. 2008) |
| ρb | Bulk density of incubated soil | 1.0×103 | kg/m3 | (Wang et al. 2023) |
| hsoil | Depth of incubated soil | 8×10-3 | m | (Wang et al. 2023) |
| V0 | Total volume of incubated soil | 2×10-5 | m3 | (Wang et al. 2023) |
| ma | cementation exponent for dissolved species | 1.5 | - | (Hamamoto et al. 2010) |
| mg | cementation exponent for gaseous species | 1.5 | - | (Hamamoto et al. 2010) |
| na | saturation exponent for dissolved species | 2.5 | - | (Hamamoto et al. 2010) |
| ng | saturation exponent for gaseous species | 2.5 | - | (Hamamoto et al. 2010) |

Table S2. Parameter values determined by Markov chain (MC) optimization

| Symbols | Descriptions | Units | Well-mixed model | PBM-T-NF model |
| --- | --- | --- | --- | --- |
| μNH4+-NO2- | Maximum reaction rate of NO2− from NH4+ | mol/g/s | 8.63×10-6 | 8.72×10-6 |
| μNO2--NO,N | Maximum reaction rate of NO from NO2− in the nitrification process | mol/g/s | 8.84×10-8 | 6.11×10-8 |
| μNO-N2O,N | Maximum reaction rate of N2O from NO in the nitrification process | mol/g/s | 2.10×10-4 | 3.21×10-5 |
| μNO2--NO3- | Maximum reaction rate of NO3− from NO2− | mol/g/s | 5.10×10-3 | 5.97×10-2 |
| μNO3--NO2- | Maximum reaction rate of NO2− from NO3− | mol/g/s | 2.47×10-5 | 2.19×10-5 |
| μNO2--NO,D | Maximum reaction rate of NO from NO2− in the denitrification process | mol/g/s | 4.43×10-7 | 1.38×10-7 |
| μNO-N2O,D | Maximum reaction rate of N2O from NO in the denitrification process | mol/g/s | 1.80×10-1 | 5.70×10-3 |
| μN2O-N2 | Maximum reaction rate of N2 from N2O | mol/g/s | 1.20×10-4 | 1.22×10-5 |

Table S3 Initial concentrations of NH4+ and NO3 under different soil moisture contents

| Soils | Concentrations mg-N/kg-soil | 40% WFPS | 60% WFPS | 70% WFPS | 80% WFPS | 90% WFPS | 100% WFPS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LC | NH4+ | 25.83 | 26.23 | 26.22 | 27.08 | 26.53 | 28.36 |
|  | NO3− | 125.02 | 132.37 | 127.50 | 117.76 | 107.83 | 64.91 |
| SZ | NH4+ | 24.80 | 24.19 | 24.25 | 23.99 | 23.33 | 24.33 |
|  | NO3− | 100.67 | 94.83 | 94.08 | 93.42 | 80.92 | 64.92 |

Fig. S1 Simulated N2O fluxes by the PBM-T-NF model as (a) μNH4+-NO2-, (b) μNO2--NO,N, (c) μNO-N2O,N, (d) μNO2--NO3-, (e) μNO3--NO2-, (f) μNO2--NO,D, (g) μNO-N2O,D, or (h) μN2O-N2was increased (5.0 ×) or decreased (0.2 ×) by five times. LC soil was used to evaluate the effects of parameters.

Fig. S2 Simulated N2O fluxes by the PBM-T-NF model as (a) NH4+ concentration, (b) NO3− concentration, (c) bulk density, or (d) soil depth was doubled (5.0 ×) or halved (0.2 ×).  Virtual soil core was used in the simulations.

References

Cadogan SP, Maitland GC, Trusler JPM (2014) Diffusion Coefficients of CO2 and N2 in Water at Temperatures between 298.15 K and 423.15 K at Pressures up to 45 MPa. Journal of Chemical & Engineering Data 59(2): 519-525

Chang B, Yan Z, Ju X, Song X, Li Y, Li S, Fu P, Zhu-Barker X (2022) Quantifying biological processes producing nitrous oxide in soil using a mechanistic model. Biogeochemistry 159: 1-14

Hamamoto S, Moldrup P, Kawamoto K, Komatsu T (2010) Excluded-volume expansion of Archie's law for gas and solute diffusivities and electrical and thermal conductivities in variably saturated porous media. Water Resources Research 46(6): WR008424

Maggi F, Gu C, Riley WJ, Hornberger GM, Venterea RT, Xu T, Spycher N, Steefel C, Miller NL, Oldenburg CM (2008) A mechanistic treatment of the dominant soil nitrogen cycling processes: Model development, testing, and application. Journal of Geophysical Research Biogeosciences 113(G2): G02016

Massman WJ (1998) A review of the molecular diffusivities of H2O, CO2, CH4, CO, O3, SO2, NH3, N2O, NO, and NO2 in air, O2 and N2 near STP. Atmospheric Environment 32(6): 1111-1127

Sander R (2015) Compilation of Henry’s law constants (version 4.0) for water as solvent. Atmospheric Chemistry and Physics 15(8): 4399-4981

Sieczka A, Koda E (2016) Kinetic and equilibrium studies of sorption of ammonium in the soil-water environment in agricultural areas of Central Poland. Applied Sciences 6(10): 269

Spiller LL (1989) Determination of Ammonia/Air Diffusion Coefficient Using Nafion Lined Tube.  22(11-12): 2561-2573

Wang H, Yan Z, Ju X, Song X, Zhang J, Li S, Zhu-Barker X (2023) Quantifying nitrous oxide production rates from nitrification and denitrification under various moisture conditions in agricultural soils: Laboratory study and literature synthesis. Frontiers in Microbiology 13: 1110151

Weast RC (1997) CRC Handbook of Chemistry and Physics. CRC Press.

Yan Z, Bond-Lamberty B, Todd-Brown KE, Bailey VL, Li S, Liu C, Liu C (2018) A moisture function of soil heterotrophic respiration that incorporates microscale processes. Nature Communications 9(1): 2562