Supplementary material

# RHF macro for Image Pro Plus, v. 5 (Pavlova et al. 2010)

# Option Explicit

Sub nuclei quantification()         Determine which parameters are static (string) and which are dynamic (integer)

Dim filename3 As String*100

Dim imname As String

Dim imdir As String

Dim lenname As Integer

Dim outlinename As String

Dim i As Integer

Dim infname As String*255

Dim iname2 As String

Dim more As Integer

Dim imdir2 As String

Dim file As Integer

Beginning:

ret= ipappcloseall()

file = IpStGetName("Find the file you want to process","c:\my documents\nuclei images", "*.tif",  Filename3)                       A folder with images was chosen

If file = 0 ThenMsgBox ("please select a file to analyze")

GoTo Beginning

End If

imdir = CurDir(filename3)

imdir2 = imdir

i=0

retry:

more = ipstsearchdir(imdir2, "*.tif",i, infname)

Debug.Print more

If more = 2 Then

i=i+1

GoTo retry

End If

ret = IpAppRun("C:\Program Files\Microsoft Office\excel.exe", RUN_MINIMIZED, RUN_AUTOCLOSE) MS

Excel is supposed present and will be opened automatically for output data

ret = IpDde (DDE_OPEN, "excel", "sheet1")

ret = IpDde(DDE_SET, "row", "1")

ret = IpDde(DDE_SET, "col", "1")

ret = IpBlbSaveData("c:\Documents and Settings\nuclei images\",S_DATA+ S_APPEND+S_Y_AXIS

+S_DDE)

Do While more = 1

imname = Dir (infname)

imdir = "c:\Documents and Settings\nuclei images\"

lenname =Len(imname)

Debug.Print lenname

outlinename = Left(imname,lenname-4)

ret = IpWsLoad(INFNAME,"tif") Image in TIFF format is converted into 8 bit grey scale (Figure 1a) and then duplicated

ret = IpWsConvertImage(IMC_GRAY, CONV_SCALE, 0, 0, 0, 0)

ret = IpWsDuplicate()

ret = IpHstEqualize(EQ_BESTFIT) Mask the brightly fluorescing chromocenters with the Equalize command (Figure 1b, c)

ret = IpLutApply()

The Best Fit option was chosen to enhance the contrast and the dynamic range of the active image, by stretching the histogram between the minimal value black (0) and maximal value white (255)

The Apply LUT (Lookup Table) command will reveal regions of enhanced contrast Blur the chromocenters with a median filter (7 x 7 kernels)
Use threshold and segmentation to find the Region Of Interest (is here the whole nucleus)

ret = IpFltMedian(7, 5)

ret = IpBlbShow(1)

ret = IpBlbSetAttr(BLOB_AUTORANGE, 0)

ret = IpBlbEnableMeas(BLBM_ALL, 0)

ret = IpBlbSetAttr(BLOB_CLEANBORDER,1)

Measure of following parameters:

ret = IpBlbEnableMeas(BLBM_AREA, 1)          	Area (area of object)

ret = IpBlbEnableMeas(BLBM_AREAPOLY, 1)       	Area polygon (area included in the polygon defining the    perimeter of the object’s outline)

ret = IpBlbEnableMeas(BLBM_SIZECOUNT, 1)

ret = IpBlbEnableMeas(BLBM_DENSITY, 1)         	Density mean (average intensity of the object)

ret = IpBlbEnableMeas(BLBM_DENSSUM, 1)     	Density sum (sum of intensity inside the object)

ret = IpBlbEnableMeas(BLBM_HETEROGENEITY, 1)

ret = IpBlbEnableMeas(BLBM_MARGINATION, 1)

ret = IpBlbEnableMeas(BLBM_PERIMETER, 1)     	Perimeter (length of the object’s outline)

ret = IpBlbEnableMeas(BLBM_ROUNDNESS, 1)

ret = IpBlbEnableMeas(BLBM_SRANGE, 1)

ret = IpTemplateMode(1)

ret = IpSegShow(1)

ret = IpSegSetRange(0,100, 255)

ret = IpTemplateMode(0)

ret = IpBlbCount()

ret = IpBlbUpdate(0)

ret = IpTemplateMode(0)

ret = IpBlbHideObject(0,0,0)

ret = IpTemplateMode(0)

ret = IpBlbUpdate(4)

ret = IpBlbSaveOutline(imdir2 & "\" & outlinename & "nuclei.scl")

Outline the ROI (Figure 1e) and output to MS Excel, and save the data

ret = IpBlbSaveOutline(imdir2 & "\" & outlinename & "nuclei.out")

ret = IpSegCreateMask(2,0,1)

ret = IpSegShow(0)

ret = IpBlbLoadOutline(imdir2 & "\" & outlinename & "nuclei.scl")

ret = IpAppSelectDoc(1)

ret = IpOpImageLogic(3, OPL_AND, 1)

ret = IpBlbLoadOutline(imdir2 & "\" & outlinename & "nuclei.scl")

ret = IpBlbUpdate(4)

ret = IpBlbMeasure()

ret = IpDde (DDE_OPEN, "excel", "sheet1")

ret = IpBlbSaveData ("", S_APPEND+S_HEADER+S_Y_AXIS+S_DDE)

ret = IpAppSelectDoc(4)

ret = IpWsDuplicate()

ret = ipfltsobel()                                  Threshold and find the edges of the chromocenters (CCs)

ret = IpFltShow(0)

ret = IpFltClose(MORPHO_2x2SQUARE, 10)

ret = IpFltErode(MORPHO_2x2SQUARE, 3)

ret = IpBlbShow(1)

ret = IpBlbSetAttr(BLOB_AUTORANGE, 1)

ret = IpBlbSetAttr(BLOB_BRIGHTOBJ, 1)

ret = IpAppSelectDoc(3)

ret = IpBlbCount()

ret = IpTemplateMode(0)               Perform a segmentation step and draw the outlines around the chromocenters

ret = IpBlbHideObject(0,0,0)

ret = IpTemplateMode(0)               Number the outlined CCs from the top left corner to the bottom right corner and save      the outline data of the CCs

ret = IpBlbUpdate(4)

ret = ipappmenuselect(1, 5 , "", DLG_Menu_coord)

ret = IpBlbUpdate(4)

ret = IpBlbSaveOutline(imdir2 & "\" & outlinename & "chromocentre.scl")

ret = IpBlbSaveOutline(imdir2 & "\" & outlinename & "chromocentre.out")

ret = IpBlbMeasure()             		Measure the same parameters as for the nucleus on the CCs

ret = IpDde (DDE_OPEN, "excel", "sheet1")     Automatically export the data to Microsoft excel (or to the Clipboard)

ret = IpBlbSaveData ("",S_APPEND+S_Y_AXIS+S_DDE)

ret = IpDde(DDE_SET, "row_inc", "1")

ret = IpDde(DDE_PUT, "r[-4]c", Chr$(34) & INFname & Chr$(34)) Place the names of the analysed file 2 rows above the exported data

ret = IpDde(DDE_SET, "append", "1")

i=i+1

more = ipstsearchdir(imdir2, "*.tif",i, infname)

ret= ipappcloseall()

Loop

ret = IpAppRestore()

End Sub