Article title: Halving global GHG emissions by 2050 without depending on nuclear and CCS

Journal name:Climatic Change

Authors: Osamu Akashia,*, Tatsuya Hanaokab, Toshihiko Masuib, Mikiko Kainumab

a Musashino University, 3-3-3 Ariake Koto-ku Tokyo 135-8181, Japan

b National Institute for Environmental Studies, 16-2 Onogawa, Tsukuba, Ibaraki 305-8506, Japan

* Corresponding author. Email:  o_akashi@musashino-u.ac.jp

List of technologies used in AIM/Enduse[Global]*

| Sector | Category | Technology options |
| --- | --- | --- |
| Power generation | Coal power plants | Pulverized coal combustion (PCC), Supercritical (SC) PCC, Ultra supercritical (USC) PCC, Integrated gasification combined cycle (IGCC), SC PCC with carbon capture and storage (CCS), USC PCC with CCS, IGCC with CCS |
|  | Gas power plants | Combined cycle, Advanced combined cycle (ACC), ACC with CCS |
|  | Renewables | Wind power, Photovoltaics, Biomass power plant, Biomass IGCC, Biomass IGCC with CCS |
| Hydrogen production |  | Coal, Coal with CCS, Natural gas, Natural gas with CCS, Biomass, Biomass with CCS |
| Industry | Steel | Large size coke oven, Coke gas recovery, Automatic combustion, Coke dry type quenching, Coal wet adjustment, COG latent heat recovery, Next generation coke oven, Automatic igniter, Cooler waste heat recovery, Mainly waste heat recovery, High-efficiency igniter, Blast furnace gas recovery, Blast furnace gas recovery with CCS, Wet top pressure recovery turbine, Dry top pressure recovery turbine, Heat recovery of hot blast stove, Coal injection, Dry top pressure gas recovery, LDG recovery, LDG latent heat recovery, Continuous caster, Hot charge rolling, Hot direct rolling, High-efficiency heating furnace, Heat furnace with regenerative burner, Continuous annealing lines, DC electric furnace, Scrap pre-heat |
|  | Cement | Ball mill, Tube mill, Vertical mill, Wet kiln, Semi wet/dry kiln, Dry long kiln, Vertical (Shaft) kiln, SP/NSP, Advanced kiln with CCS |
|  | Other industries | High-efficiency boiler (coal, oil, gas), boiler with combustion control (coal, oil, gas), cogeneration (coal, oil, gas), Regenerative gas boiler, High-efficiency industrial furnace (oil, gas), Motor with inverter control, High-efficiency motor |
| Residential and Commercial | Cooling | High-efficiency cooler (Sold average in developed countries in 2000, Top Runner, Highest performance) |
|  | Warming | High-efficiency kerosene stove, LPG stove, Gas stove, High-efficiency air conditioner (Sold average in developed countries in 2000, Top Runner, Highest performance), Wall insulation for detached house, Wall insulation, Double-glazed glass with low-e |
|  | Hot water | High-efficiency kerosene water heater, High-efficiency LPG water heater, Latent heat recovery LPG water heater, Latent heat recovery gas water heater, CO2 refrigerant water heater, Solar thermal water heater, Fuel cell |
|  | Cooking | High-efficiency gas cooking stove (LPG, Natural gas) |
|  | Lighting | Incandescent, Fluorescent of incandescent type, Fluorescent with energy saving stabilizer, Inverter type fluorescent, Hf Inverter type fluorescent |
|  | Refrigerators | High-efficiency refrigerator (Sold average in developed countries in 2000, Top Runner, Highest performance) |
|  | TV | High-efficiency TV (Sold average in developed countries in 2000, Top Runner, Highest performance), TV (Liquid crystal display) |
| Transport | Passenger cars | High-efficiency passenger car, Weight reduction, Engine friction reduction, Aerodynamic drag reduction, Rolling resistance reduction, Brake drag reduction, Continuously variable transmission, VVLT & cylinder reactivation, GDI engine, Hybrid electric vehicle (HEV), Electric vehicle (EV), Fuel cell vehicle (FCV), Biofuel |
|  | Trucks | High-efficiency truck, Engine improvement, Weight reduction, Aerodynamic drag reduction, Rolling resistance reduction, Natural gas vehicle, Hybrid vehicle, Fuel cell vehicle, Biofuel |
|  | Passenger buses | High-efficiency bus, Rolling resistance reduction for bus, Hybrid engine, Biofuel |
|  | Ships | High-efficiency ship, Biofuel |
|  | Aircraft | High-efficiency aircraft, Biofuel |
|  | Railways | High-efficiency train, Regenerative braking system with VVVF |
| Agriculture | Rice cultivation | Water management (Midseason drainage, Shallow flooding, Alternative flooding/Drainage), Fertilizer management (ammonium sulphate), Upland rice, Addition of phosphogypsum, Direct wet seeding, Off-season straw, Rice straw compost |
|  | Cropland | Reduced fertilization, Nitrogen inhibitor, Fertilizer free zone, Optimized distribution geometry, fertilization management (Spreader maintenance, Split fertilization, Sub-optimal fertilizer application), Conversion of fertilization tillage to no-till |
|  | Mature management | Anaerobic digestion (Centralized plant, Farmscale plant), Covered digester, Covered lagoon, daily spread of manure, Complete mix digester, Plug flow digester, Slowing down of anaerobic decomposition |
|  | Livestock rumination | Administration of chemical substances (Propionate precursors, Prebiotics), Feed management (High fat diet, Improved feed intake and genetics, Replacement of roughage with concentrates) |
| Waste | Municipal solid waste | Biological treatment, Improved oxidation through improved capping and restoration, Direct use of landfill gas, Electricity and heat generation from landfill gas, Flaring of landfill gas, Upgrading of natural gas, Anaerobic digestion, Composting (Windrow plant, Tunnel plant, Hall plant), Incineration, Paper recycling |
| Fluorinated gas emissions | By-product emissions | Thermal oxidation |
|  | Refrigerants | Alternative systems (CO2, Hydrocarbons, Hydrocarbons & NH3), Leakage reduction, Recovery, Decomposition |
|  | Aerosols | Alternative aerosols (Hydrocarbon aerosol propellants, Not-in-kind alternatives), 50% reduction (for Medical applications, General aerosol propellants) |
|  | Foam blowing agents | Recovery, Decomposition, Alternative systems (Water-blown CO2 systems, Liquid CO2 foam blowing, Hydrocarbon foam blowing) |
|  | Solvents | Alternative solvents (NIK aqueous, NIK semi-aqueous), Retrofit options, 50% reduction |
|  | Manufacturing | Cleaning facility (NF3 in situ clean, NF3 remote clean), Recapture/Destroy, Plasma abatement, Catalytic destruction, Thermal oxidation, Retrofitting (PFPB, SWPB, CWPB, VSS, HSS), SO2 replacement |
|  | Electrical equipment | Leakage reduction, Device recycling |

Note: *Only extant and promising technologies were used in the model.