# Online Appendix

Tab. 1 Leakage rates li are determined by fitting the cumulative leaked fraction (LF) value of the exponential leakage model with L(t)/R0 =lexp(-lt) against the Alcalde et al. (2018) LF values: titi+1L(t)/R0 dt=exp-li+1ti-exp-li+1ti+1=!LFti+1+LF(ti), i∈[0,2] and t0=0. The P50 (P05) values indicate that 50% (5%) of the calculated cumulative leaked fraction or leakage rate values are larger than the displayed value. Worst-case leakage estimates in this article utilize the P50 leakage rate lwc.

|  | Alcalde et al. Leaked Fraction (LF) Values |  |  | Leakage Rates li - Exponential Leakage |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | Time | P50 value LF | P05 value LF | Time | P50 value li | P05 value li |
|  | (yr) | (%) | (%) | (yr) | (%/yr) | (%/yr) |
| 'Well-Managed' Sites | t1=100 | 0.156 | 0.358 | 1-100 | l1=0.00156 | l1=0.00359 |
|  | t2=1,000 | 0.888 | 2.96 | 100-1,000 | l2=0.000817 | l2=0.00294 |
|  | t3=10,000 | 8.18 | 25.71 | 1,000-10,000 | l3=0.000850 | l3=0.00296 |
| 'Poorly-Managed' Sites | t0=1 | 0.202 | 0.521 | 0-1 | lwc=0.202 | lwc=0.522 |
|  | t1=100 | 6.41 | 16.5 | 1-100 | l1=0.0662 | l1=0.180 |
|  | t2=1,000 | 8.05 | 20.0 | 100-1,000 | l2=0.00184 | l2=0.00398 |
|  | t3=10,000 | 22 | 32.6 | 1,000-10,000 | l3=0.00171 | l3=0.00153 |

Let tI be the time of a CO2 injection pulse and Δt the time at which CO2 prices hit the backstop marginal cost level. The sequestration pulse is given by R(t)=R0(tI)δ(t-tI), the leakage flux entailed after the sequestration pulse is given by table 1 with Lt=l1R0tIexp(-l1t-tI) for tI≤t≤tI+100yr, Lt= l2R0(tI)exp(-l2t-tI) for tI+100yr<t≤tI+1000yr and Lt= l3R0tIexp-l3t-tI for t>tI+1000yr, the carbon price is assumed as pt=pt0exp(r (t-t_0)) for t 0≤t≤Δt and as p(t)=p(Δt) for t>Δt.

Then the analytical solution of the effectiveness of temporary CO2 storage η is:

For tI≤Δt,  η(l, tI)=1-(1-exp-l1Δt- tI ) –

-l1l1+rexp(r(Δt-tI))(exp-l1+rΔt-tI-exp-l1+r100yr-

-l2l2+rexp(r(Δt-tI))(exp-l2+r100yr-exp-l2+r1000yr-

-l3l3+rexp(r(Δt-tI))(exp-l3+r1000yr

For tI>Δt,  η(l, tI)=1-

-l1l1+r1-exp-l1+r100yr-

-l2l2+rexp-l2+r100yr-exp-l2+r1000yr-

-l3l3+rexp-l3+r1000yr

Furthermore, the differential equation describing the injection pulse response of the compensation payment fund F(t) of figure 1 is given by:

dF(t)dt=rFt-ptLt.

Thus, for any tI≤Δt and for t>tI+1000yr, the fund evolves as:

Ft,tI=ptIR0tIexp⁡(r(t-tI))(FtIptIR0tI-(1-exp-l1Δt-tI-

-l1l1+rexp(r(Δt-tI))(exp-l1+rΔt-tI-exp-l1+r100yr-

-l2l2+rexp(r(Δt-tI))(exp-l2+r100yr-exp-l2+r1000yr-

-l3l3+rexp(r(Δt-tI))(exp-l3+r1000yr-exp-l3+rt)).

If t≤Δt, the last three lines equate zero and the upper integration bound of the first line (the term exp-l1Δt-tI) is substituted by exp-l1t-tI. For t>Δt, the lines are adjusted according to the same logic.	
Note that the term FtIptIR0tI equates (1-η) for figures 3 & 4 (a) & (b) and 3% for (c) and (d).