Supplementary files for mapping China’s township-level potential for biogas production from agricultural crop residues for electricity generation

- Summary of key equations, simplifications, parameters, and their potential impacts on model accuracy in Simplified ADM1 (ADM1-R4) model.

Table S1. Summary of key equations, simplifications, parameters, and their potential impacts on model accuracy in Simplified ADM1 (ADM1-R4) model

| Aspect | Simplifications and Key Assumptions | Equations/Descriptions |
| --- | --- | --- |
| Reaction Pathways | Aggregation of biochemical pathways (carbohydrates, proteins, lipids degradation). | Xi,hydrolysis=Xi,0·e-khyd,i·t(i represents carbohydrates, proteins, lipids). |
| Microbial Growth Dynamics | Assumes steady-state biomass; explicit microbial growth omitted. | Biomass assumed constant; microbial decay implicitly included in simplified kinetics. |
| Intermediate Reactions | Consolidation of acidogenesis and acetogenesis into simplified kinetic expressions. | Single-step first-order kinetic equations. r=kdeg·S |
| Hydrolysis Kinetics | Simplified to first-order kinetics. | rhyd=khyd·Xsubstrate |
| Key Parameters | Hydrolysis rate constants （khyd); Degradation rate constants (kdeg). | Hydrolysis: khyd, carb= 0.25 d-1, khyd, prot=0.2 d-1, khyd,lip= 0.1 d-1; Degradation: adjusted empirical constants from literature (Weinrich et al., 2021). |

- Parameters, Test Methods, Detection Limits, and Quality Control Procedures

Table S2. Parameters, Test Methods, Detection Limits, and Quality Control Procedures

| Parameter | Standard/Reference | Equipment | Detection Limit | Quality Control Procedures |
| --- | --- | --- | --- | --- |
| O (%) | Poggio et al., 2016 | Elementar Vario EL cube | 0.1% | Instrument calibrated daily; standard reference samples used |
| C (%) | Poggio et al., 2016 | Elementar Vario EL cube | 0.1% | Replicate analysis and certified reference material |
| H (%) | Poggio et al., 2016 | Elementar Vario EL cube | 0.1% | Daily blank and standard verification |
| TS (%) | Standard Methods | Oven (105°C) | 0.1% | Triplicate analysis until weight stability |
| TN (%) | Standard Methods | Elementar Vario EL cube | 0.1% | Internal standard and recovery checks |
| TKN (g/kg) | Kjeldahl nitrogen method | KN680 Kjeldahl Detector | 0.01 g/kg | Calibration curve, blank correction |
| TAN (g/kg) | UV-Vis spectrophotometry | T6 UV-Vis Spectrophotometer | 0.01 g/kg | Calibration curve, blank correction |
| Moisture Content (%) | Dry weight loss | Oven (105°C) | 0.1% | Repeated until constant weight achieved |
| OM (%) | Ignition loss | Oven, Muffle Furnace | ±1.0% | Consistent ignition temperature; duplicate |
| Total Solid (%) | APHA Standard Methods | Muffle Furnace | ±0.1% | Validated against standard procedure |
| Volatile Solid (%) | APHA Standard Methods | Muffle Furnace | ±0.1% | Blank and spike control samples |
| pH | pH meter method | SX751 Multifunction Meter | ±0.1 pH | pH buffer calibration |
| Hemicellulose (%) | Li et al., 2016 | SH2201 Furnace, SHP-III Pump | ±0.1% | Standard cellulose spike and blank |
| Cellulose (%) | Li et al., 2016 | SH2201 Furnace, SHP-III Pump | ±0.1% | Parallel sample validation |
| Lignin (%) | Li et al., 2016 | SH2201 Furnace, SHP-III Pump | ±0.1% | Mass balance check, duplicates |
| VFA (mol/kg) | Li et al., 2016 | Titration | 0.1 mol/kg | Acid/base titrant standardization |
| Crude Lipids (g/kg TS) | Soxhlet extraction | Soxhlet apparatus | 0.1 g/kg | Solvent blank and recovery efficiency |
| Crude Fibres (g/kg TS) | Acid-base digestion | CXC-06 Fiber Tester | 0.1 g/kg | Recovery vs standard cellulose |
| Inorganic Nitrogen (g/L) | Colorimetric method | KN680 Kjeldahl Detector | 0.1 g/L | Duplicate and standard sample control |
| Density (g/L) | Manufacturer specs | Anton Paar Analyzer | ±0.01 g/L | Temperature-controlled repeat readings |

- The characteristics of the substrates collected from the data source

Table S3. The characteristics of the substrates collected from the data source based on CH4 yields

|  | TS | VS | Carbohydrate | Protein | Lipid | Lignin | VFA | CH4 yield |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum | 7.1 | 5.7 | 28.3 | 2.9 | 0.0 | 1.3 | 0.0 | 51.0 |
| Maximal | 92.5 | 87.5 | 88.2 | 33.1 | 24.2 | 26.7 | 15.8 | 528.0 |
| Units | %FM | % TS | % VS | % VS | % VS | % VS | % VS | mL CH4 gVS |

Note: TS: Total solids, VS：Volatile solids, S/I: Substrate to inoculum

- Relevant parameters from the literature

Table S4.1 Elemental and physical analysis results of agricultural residues from the literature and BMP modeling result

| Parameter |  |  |  | Substrates’ value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RS (Li, Ke, et al., 2016) | RS (Li, Tan, et al., 2016) | CP (Marín et al., 2007) | SB (Bilba et al., 2003) | VW-Tomato (Elkhalifa et al., 2022) | VW- Cucumber (Elkhalifa et al., 2022) | VW- Carrot (Elkhalifa et al., 2022) | Units |
| C | 39.7 | 36.0 | 46.4 | 45.5 | 49.68 | 56.69 | 41.54 | % TS |
| H | 5.4 | 6.6 | 5.7 | 5.6 | 7.35 | 6.52 | 5.39 | % TS |
| N | 0.9 | 1.1 | 1.5 | 0.3 | 3.72 | 3.91 | 1.37 | % TS |
| O (by difference) | - | 0.4 | 0.1 | - | 0 | 0 | 0 | % TS |
| S(%TS) | 38.2 | 55.8 | 46.3 | 45.2 | 39.25 | 32.88 | 51.7 | % TS |
| Empirical formula | C52H84O37N1 | C1225H1953O983N29S | C2477H2530O1860N70S | C177H260O132N | C16H27O9N | C17H23O7N | C35H55O33N |  |
| BMP | 439.0 | 296.6 | 350.0 | 358.9 | 389.1 | 515.6 | 322.1 | mL CH4 g VS |
| Note: TS: Total solids, VS：Volatile solids, RS：Rice straw, CP： Citrus peel, SB：Sugarcane bagasse, VW: Vegetable waste |  |  |  |  |  |  |  |  |

| Table S4.2 Input parameters for the predictions obtained from the literature and the results of the model calculations |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter |  |  | Substrates’ value |  |  |  |  |  |
|  | RS (Li, Ke, et al., 2016) | RS (Li, Tan, et al., 2016) |  | CP (Ajieh et al., 2021) | CP (Feumba Dibanda Romelle, 2016) | SB (Suryaningrum et al., 2021) | WL (Wang et al., 2022) | Units |
| TS | 92.5 | 92.9 |  | 59.3 | 63.7 | 69.3 | 58.12 | % |
| VS | 77.4 | 81.6 |  | 60.7 | 37.2 | 56.8 | 43.78 | % |
| Carbohydrate | 58.53 | 83.6 |  | 71.6 | 90.9 | 92.4 | 87.27 | % VS |
| Protein | 28.7 | 5.6 |  | 5.91 | 1.1 | 1.57 | 8.65 | % VS |
| Lipid | 5.23 | 0 |  | 14.1 | 0.5 | 0.9 | 0.12 | % VS |
| Lignin | 7.54 | 10.8 |  | 8.2 | 7.5 | 5.2 | 3.96 | % VS |
| Volatile fatty acids | BD | 0 |  | 0.2 | - | - | - | % VS |
| S/I |  |  |  |  |  |  |  |  |
| ML-GB | 178.7 | 310.5 |  | 256 | 245 | 375.4 | 321.3 | mL CH4 /gVS |
| ML-RF | 215.4 | 241.1 |  | 286.9 | 393.2 | 464.4 | 443.8 | mL CH4 /gVS |
| Note: RS：Rice straw, CP： Citrus peel, SB：Sugarcane bagasse, WL: Water bamboo leaves, VW: Vegetable waste, S/I: Substrate to inoculum, ML-GB: Gadient Bosting model；ML-RF: Random Forest model；Protein content was determined by multiplying the difference between TKN and TAN by a factor of 6.25 (Li, Ke, et al., 2016). |  |  |  |  |  |  |  |  |

- Range of variation of elements

Table S5. Range of variation of elements from the crop residues

| Residues | C | H | N | O | S |
| --- | --- | --- | --- | --- | --- |
| RS | 35-45%(Saad et al., 2019) | 5-7%(Parikh et al., 2007) | 0.5%-1.5% (Dhillon et al., 2018) | 35%-45%(Saad et al., 2019) | 0-0.3%(Chen et al., 2019) |
| CP | 40-50%(Foo & Hameed, 2012) | 5-7%(Parikh et al., 2007) | 0.8-1.2% (Alva et al., 2006) | 40-50%(M’hiri et al., 2017) | 0-0.3%(M’hiri et al., 2017) |
| SB | 40-50%(Villamar et al., 2016) | 5-7%(Parikh et al., 2007) | 0.8-1.2% (Santana et al., 2020) | 40-50%(Kumar et al., 2022) | 0-0.3%(Kumar et al., 2022) |
| WL | 35-45% (Wang et al., 2023) | 5-7%(Parikh et al., 2007) | 0.8-1.2% (Wang et al., 2023) | 40-50%(Wang et al., 2023) | 0-0.3%(Wang et al., 2023) |
| VW | 30-50%(Zhang et al., 2020) | 4-10%(Moreno Cardenas et al., 2015) | 1.5-2.5% (Chen et al., 2019) | 20-30%(Gupta et al., 2023) | 0-0.3%(Gupta et al., 2023) |

Note: RS：Rice straw, CP： Citrus peel,  SB：Sugarcane bagasse, WL: Water bamboo leaves, VW: Vegetable waste

| Model | Most Influential Parameters | Direction of Impact | Notes |
| --- | --- | --- | --- |
| BMP | Carbon (C), Hydrogen (H), Oxygen (O) | Positive (C, H); Negative (O) | Higher C and H promote methane yield, high O content reduces it |
| S-ADM1 | Xch (Carbohydrates) | Positive | Xch variations had a greater effect than Xpr or Xli |
| ML-GB | Lignin, VFA | Negative | Lignin and volatile fatty acids reduce biogas prediction |
| ML-RF | VFA, Lignin | Negative | Similar to GB, with slightly different ranking |

- Summary of sensitivity analysis results

Table S6 Summary of sensitivity analysis results of different models

Reference

Ajieh, M. U., Isagba, E. S., Ihoeghian, N., Edosa, V. I. O., Amenaghawon, A., Oshoma, C. E., Erhunmwunse, N., Obuekwe, I. S., Tongo, I., Emokaro, C., & Ezemonye, L. I. N. (2021). Assessment of sociocultural acceptability of biogas from faecal waste as an alternative energy source in selected areas of Benin City, Edo State, Nigeria. Environment, Development and Sustainability, 23(9), 13182–13199. https://doi.org/10.1007/s10668-020-01205-y

Alva, A. K., Paramasivam, S., Obreza, T. A., & Schumann, A. W. (2006). Nitrogen best management practice for citrus trees. Scientia Horticulturae, 107(3), 233–244. https://doi.org/10.1016/j.scienta.2005.05.017

Bilba, K., Arsene, M.-A., & Ouensanga, A. (2003). Sugar cane bagasse fibre reinforced cement composites. Part I. Influence of the botanical components of bagasse on the setting of bagasse/cement composite. Cement and Concrete Composites, 25(1), 91–96. https://doi.org/10.1016/S0958-9465(02)00003-3

Cheah, S., Malone, S. C., & Feik, C. J. (2014). Speciation of sulfur in biochar produced from pyrolysis and gasification of oak and corn stover. Environmental Science & Technology, 48(15), 8474–8480. https://doi.org/10.1021/es500073r

Chen, M., Wang, C., Wang, B., Bai, X., Gao, H., & Huang, Y. (2019). Enzymatic mechanism of organic nitrogen conversion and ammonia formation during vegetable waste composting using two amendments. Waste Management (New York, N.Y.), 95, 306–315. https://doi.org/10.1016/j.wasman.2019.06.027

Dhillon, A. K., Sharma, N., Dosanjh, N. K., Goyal, M., & Mahajan, G. (2018). Variation in the nutritional quality of rice straw and grain in response to different nitrogen levels. Journal of Plant Nutrition, 41(15), 1946–1956. https://doi.org/10.1080/01904167.2018.1482915

Elkhalifa, S., Mariyam, S., Mackey, H. R., Al-Ansari, T., McKay, G., & Parthasarathy, P. (2022). Pyrolysis Valorization of Vegetable Wastes: Thermal, Kinetic, Thermodynamics, and Pyrogas Analyses. Energies, 15(17), 6277. https://doi.org/10.3390/en15176277

Feumba Dibanda Romelle. (2016). CHEMICAL COMPOSITION OF SOME SELECTED FRUIT PEELS. European Journal of Food Science and Technology, 4(4), 12–21.

Foo, K. Y., & Hameed, B. H. (2012). Preparation, characterization and evaluation of adsorptive properties of orange peel based activated carbon via microwave induced K2CO3 activation. Bioresource Technology, 104, 679–686. https://doi.org/10.1016/j.biortech.2011.10.005

Gupta, P. J., Trivedi, M. J., & Soni, H. P. (2023). Proximate Analysis of Vegetable and Fruit Waste Collected from the Kadi Vegetable Market, Gujarat. Biosciences Biotechnology Research Asia, 20(2), 673–679. https://doi.org/10.13005/bbra/3121

Kumar, M., Upadhyay, S. N., & Mishra, P. K. (2022). Pyrolysis of Sugarcane (Saccharum officinarum L.) Leaves and Characterization of Products. ACS Omega, 7(32), 28052–28064. https://doi.org/10.1021/acsomega.2c02076

Li, H., Ke, L., Chen, Z., Feng, G., Xia, D., Wang, Y., Zheng, Y., & Li, Q. (2016). Estimating the Fates of C and N in Various Anaerobic Codigestions of Manure and Lignocellulosic Biomass Based on Artificial Neural Networks. Energy & Fuels, 30(11), 9490–9501. https://doi.org/10.1021/acs.energyfuels.6b01883

Li, H., Tan, F., Ke, L., Xia, D., Wang, Y., He, N., Zheng, Y., & Li, Q. (2016). Mass balances and distributions of C, N, and P in the anaerobic digestion of different substrates and relationships between products and substrates. Chemical Engineering Journal, 287, 329–336. https://doi.org/10.1016/j.cej.2015.11.003

Marín, F. R., Soler-Rivas, C., Benavente-García, O., Castillo, J., & Pérez-Alvarez, J. A. (2007). By-products from different citrus processes as a source of customized functional fibres. Food Chemistry, 100(2), 736–741. https://doi.org/10.1016/j.foodchem.2005.04.040

M’hiri, N., Ioannou, I., Ghoul, M., & Mihoubi Boudhrioua, N. (2017). Phytochemical characteristics of citrus peel and effect of conventional and nonconventional processing on phenolic compounds: A review. Food Reviews International, 33(6), 587–619. https://doi.org/10.1080/87559129.2016.1196489

Moreno Cardenas, E. L., Zapata Zapata, A. D., & Alvarez Mejia, F. (2015). Analysis of hydrogen production by anaerobic fermentation from urban organic waste. DYNA, 82(189), 127–133. https://doi.org/10.15446/dyna.v82n189.42461

Parikh, J., Channiwala, S. A., & Ghosal, G. K. (2007). A correlation for calculating elemental composition from proximate analysis of biomass materials. Fuel, 86(12–13), 1710–1719. https://doi.org/10.1016/j.fuel.2006.12.029

Poggio, D., Walker, M., Nimmo, W., Ma, L., & Pourkashanian, M. (2016). Modelling the anaerobic digestion of solid organic waste—Substrate characterisation method for ADM1 using a combined biochemical and kinetic parameter estimation approach. Waste Management (New York, N.Y.), 53, 40–54. https://doi.org/10.1016/j.wasman.2016.04.024

Saad, M. J., Chia, C. H., Zakaria, S., Sajab, M. S., Misran, S., Abdul Rahman, M. H., & Chin, S. X. (2019). Physical and Chemical Properties of the Rice Straw Activated Carbon Produced from Carbonization and KOH Activation Processes. Sains Malaysiana, 48(2), 385–391. https://doi.org/10.17576/jsm-2019-4802-16

Santana, A. C. de A., Oliveira, E. C. A., Da Silva, V. S. G., Santos, R. L. dos, Da Silva, M. A., & Freire, F. J. (2020). Critical nitrogen dilution curves and productivity assessments for plant cane. Revista Brasileira de Engenharia Agrícola e Ambiental, 24(4), 244–251. https://doi.org/10.1590/1807-1929/agriambi.v24n4p244-251

Suryaningrum, L. H., Samsudin, R., Mulyasari, & Saputra, A. (2021). The digestibility of improved sugar cane bagasse on Barbonymus schwanenfeldii. IOP Conference Series: Earth and Environmental Science, 674(1), 012004. https://doi.org/10.1088/1755-1315/674/1/012004

Villamar, C. A., Rivera, D., & Aguayo, M. (2016). Anaerobic co-digestion plants for the revaluation of agricultural waste: Sustainable location sites from a GIS analysis. Waste Management & Research : The Journal of the International Solid Wastes and Public Cleansing Association, ISWA, 34(4), 316–326. https://doi.org/10.1177/0734242X16628979

Wang, R., Cai, W., Le Yu, Li, W., Zhu, L., Cao, B., Li, J., Shen, J., Zhang, S., Nie, Y., & Wang, C. (2023). A high spatial resolution dataset of China’s biomass resource potential. Scientific Data, 10(1), 384. https://doi.org/10.1038/s41597-023-02227-7

Wang, Y., Zhuo, Y., & Liu, T. (2022). Population Dynamics in China’s Urbanizing Megaregion: A Township-Level Analysis of the Beijing–Tianjin–Hebei Region. Land, 11(9), 1394. https://doi.org/10.3390/land11091394

Zhang, H., Wu, K., Qiu, Y., Chan, G., Wang, S., Zhou, D., & Ren, X. (2020). Solar photovoltaic interventions have reduced rural poverty in China. Nature Communications, 11(1), 1969. https://doi.org/10.1038/s41467-020-15826-4