Supplementary material of the article titled “Synergism of life cycle assessment (LCA) and sustainable development goals (SDGs) framework for best selection of sludge-based concrete scenarios”

Table S1. Geographic location for the material (cement, gravel, sand, etc.) collected and used to prepare different concrete mixtures

| Material | Location | Latitude | Longitude |
| --- | --- | --- | --- |
| Sand | El Salheya | 30.539748847336515 | 31.67697478085796 |
| Aggregate | El Galala | 29.656071045759248 | 32.22831975362354 |
| Cement | El Katameya | 29.920872225860865 | 31.537071674024432 |
| Superplasticizer | El Oubour | 30.189009697863852 | 31.4531490575549 |

Table S2. Description of midpoint indicators used in life cycle assessment (LCA) for determining the environmental impacts of implementing the different concrete mixtures (i) ordinary Portland cement (OPC) mixture and sludge disposal, representing 100% of OPC in concrete while transporting sludge to landfills (ii) raw sludge (RS) mixture, evaluating concrete with 5% RS as a partial cement replacement + 95% OPC, and (iii) thermally activated alum sludge (TAAS) mixture, denoting concrete with 15% TAAS as a partial cement replacement + 85% OPC

| Indicator | Definition | Reference |
| --- | --- | --- |
| Carcinogens | Carcinogens are affected by the hazardous substances produced through raw mill process of cement production. Carcinogens are estimated in kg C₂H₃Cl eq. | (Rosyid et al., 2020) |
| Non-carcinogens | Non-carcinogens are caused by substances emitted during fuel consumption and raw material, not causing cancer. Non-carcinogens are estimated in kg C₂H₃Cl eq. | (Ige et al., 2022) |
| Respiratory inorganics | Respiratory inorganics caused by the emissions of particulate matter (PM) and its precursors (e.g., NOx, SOx, and NH3) emitted during cement production (i.e., extraction, transportation, and crushing). Respiratory inorganics are estimated in kg PM2.5 eq. | (Acharya & Patro, 2023) |
| Ionizing radiation | The destruction in ionizing radiation is caused by damaging in cellular that may not survive or be modified to become cancerous. Ionizing radiation is estimated in Bq C-14 eq. | (Little, 2003) |
| Ozone layer depletion | Destructing the ozone layer is caused by anthropogenic emissions of recalcitrant chemicals containing chlorine and/or bromine atoms associated with the fossil fuels usage in cement production. Ozone layer depletion is estimated in kg CFC-11 eq. | (Çankaya & Pekey, 2019) |
| Respiratory organics | Respiratory organics provide a health risk to humans due to the emission of precursors and particulate matter. Respiratory organics are estimated in kg C₂H4 eq. | (Acharya & Patro, 2023) |
| Aquatic eco-toxicity | Aquatic ecotoxicity is caused by fuel use and air emissions from the clinkering process. In addition, the electricity and consumables required to operate the cement plant affect aquatic ecotoxicity. Aquatic eco-toxicity is estimated in kg Triethylene Glycol (TEG) water. | (Hong & Li, 2011) |
| Terrestrial eco-toxicity | Destructing in terrestrial ecotoxicity caused by substances (e.g., aluminum) in soil, the main component of sludge. Terrestrial ecotoxicity is estimated in kg TEG soil. | (Hong & Li, 2011) |
| Terrestrial acid/nutria | The increase in terrestrial acid/nutria is caused by airborne inorganic material deposition and the emission of NH3, SO2, and NOx. Terrestrial acid/nutria is estimated in kg SO₂ eq. | (Mirkarimi et al., 2021) |
| Land occupation | The damage in land occupation is caused by dumping sludge into landfills, further deteriorating the topsoil. Land occupation is estimated in m2 org·arable. | (Taelman et al., 2016) |
| Aquatic acidification | The primary cause of aquatic acidification is CO2, further reducing the pH levels of the ocean. It can be estimated in kg SO₂ eq. | (van Zelm et al., 2015) |
| Aquatic eutrophication | Aquatic eutrophication is a major problem with water quality, leading to the increase in algae growth. Aquatic eutrophication is estimated in kg PO₄ P-lim | (Payen & Ledgard, 2017) |
| Global warming | The amount of CO2 generated in the concrete manufacturing processes, including the production and transportation of cement and supplementary cementitious materials. Global warming is estimated in kg CO2 eq. | (Rashid et al., 2020) |
| Non-renewable energy | Non-renewable energy impact can increase due to the usage of fossil fuel, heat energy, and electricity consumption. It is measured in MJ primary. | (Çankaya & Pekey, 2019) |
| Mineral extraction | Mineral extraction can cause resources depletion and soil erosion. It can be expressed in MJ surplus. | (Yellishetty et al., 2012) |

Table S3. Description of endpoint indicators used in LCA for determining the environmental impacts of implementing the different concrete mixtures (i) ordinary Portland cement (OPC) mixture and sludge disposal, representing 100% of OPC in concrete while transporting sludge to landfills (ii) raw sludge (RS) mixture, evaluating concrete with 5% RS as a partial cement replacement + 95% OPC, and (iii) thermally activated alum sludge (TAAS) mixture, denoting concrete with 15% TAAS as a partial cement replacement + 85% OPC

| Indicator | Definition | Reference |
| --- | --- | --- |
| Human health | Human health category reflects the environmental impact on human health through different indicators (e.g., respiratory inorganics, carcinogens, and noncarcinogens). Its characteristics include the production of photochemical oxidants and air pollution. Human health is estimated in Disability-adjusted life years (DALY). | (Ismaeel, 2018; Shi et al., 2022) |
| Climate Change | Climate change is expressed by global warming. It can be calculated by the amount of CO2 produced during cement manufacturing. Climate change is estimated in kg CO2 eq. | (Vieira et al., 2016; Rashid et al., 2020) |
| Ecosystem quality | Ecosystem quality is the degree to which a particular area has been impacted by environmental load, in terms of land occupation and terrestrial ecotoxicity. Ecosystem quality can be estimated in PDF·m2·yr, with PDF as the potentially disappeared fraction of species | (Van den Heede & De Belie, 2012) |
| Resource | Resource damage is evaluated by the economic loss resulting from the increase in marginal cost due to resource extraction. Resource is estimated in MJ. | (Dong & Ng, 2014) |

Table S4. Midpoint environmental impacts of OPC mixture and sludge landfilling (scenario_1), RS mixture (scenario_2), and TAAS mixture (scenario_3) with the reduction percentage compared to scenario_1

| Impact category | Unit | Scenario_1 | Scenario_2 |  | Scenario_3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Value | Reduction (%) | Value | Reduction (%) |
| Carcinogens | kg C₂H₃Cl eq | 414 | 95.8 | 76.86 | 215 | 48.07 |
| Non-carcinogens | kg C₂H₃Cl eq | 395 | 92.5 | 76.58 | 206 | 47.85 |
| Respiratory inorganics | kg PM2.5 eq | 15.2 | 3.75 | 75.33 | 8.01 | 47.30 |
| Ionizing radiation | Bq C-14 eq | 230000 | 55600 | 75.83 | 121000 | 47.39 |
| Ozone layer depletion | kg CFC-11 eq | 0.00105 | 0.000362 | 65.52 | 0.00059 | 43.81 |
| Respiratory organics | kg C₂H₄ eq | 5.23 | 1.43 | 72.66 | 2.81 | 46.27 |
| Aquatic eco-toxicity | kg TEG water | 934000 | 294000 | 68.52 | 512000 | 45.18 |
| Terrestrial eco-toxicity | kg TEG soil | 313000 | 76700 | 75.50 | 164000 | 47.60 |
| Terrestrial acid/nutria | kg SO₂ eq | 287 | 73.6 | 74.36 | 152 | 47.04 |
| Land occupation | m2 org.arable | 5890 | 1360 | 76.91 | 3070 | 47.88 |
| Aquatic acidification | kg SO₂ eq | 54.5 | 13.9 | 74.50 | 28.8 | 47.16 |
| Aquatic eutrophication | kg PO₄ P-lim | 0.31 | 0.0742 | 76.06 | 0.162 | 47.74 |
| Global warming | kg CO₂ eq | 17000 | 4160 | 75.53 | 8930 | 47.47 |
| Non-renewable energy | MJ primary | 168000 | 42200 | 74.88 | 88700 | 47.20 |
| Mineral extraction | MJ surplus | 2320 | 537 | 76.85 | 1210 | 47.84 |

Table S5. Endpoint environmental impacts of OPC mixture and sludge landfilling (scenario_1), RS mixture (scenario_2), and TAAS mixture (scenario_3) with the reduction percentage compared to scenario_1

| Damage category | Unit | Scenario_1 | Scenario_2 |  | Scenario_3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Value | Reduction (%) | Value | Reduction (%) |
| Resources | MJ primary | 171000 | 42700 | 75.03 | 89900 | 47.43 |
| Climate Change | kg CO2 eq | 17000 | 4160 | 75.53 | 8930 | 47.47 |
| Ecosystem quality | PDF·m2·yr | 9240 | 2180 | 76.41 | 4830 | 47.73 |
| Human health | DALY | 0.013 | 0.00317 | 75.62 | 0.00682 | 47.54 |

Table S6. Questionnaire sheet for finding the correlation between the application of sludge and biochar in concrete manufacturing and the filfillment of SDG#1 “End poverty”, SDG#7 “Access to sustainable energy”, SDG#11 “Sustainable cities”, and SDG#17 “Partnership for sustainable development”. The responses are analyzed to determine the sustainable development goals (SDGs) target indicator matrix

| Expert Profile | Field / Question | Response / Rating | Notes/Comment |
| --- | --- | --- | --- |
|  | Full Name (optional) | ……. | ……. |
|  | Highest Academic Degree | ☐ BSc ☐ MSc ☐ PhD ☐ Other: _____ | ……. |
|  | Field of Expertise | ……. | ……. |
|  | Years of Experience | ☐ <5 ☐ 5–10 ☐ 10–15 ☐ >15 | ……. |
|  | Current Affiliation | ……. | ……. |
| SDG#1 “End poverty” | Proportion of the cement workers living below the international poverty line Response:.................................................................................. |  |  |
|  | Proportion of solid waste collectors covered by social protection systems Response:.................................................................................. |  |  |
|  | Proportion of sludge managers living in households with access to basic services Response:.................................................................................. |  |  |
|  | Proportion of total government spending on education, and health services for cement lab technician Response:.................................................................................. |  |  |
| SDG#7 “Access to sustainable energy” | Renewable energy share in the total final energy consumption in cement industry Response:.................................................................................. |  |  |
|  | Energy intensity and effeciency for sludge dewatering Response:.................................................................................. |  |  |
|  | Improving infrastructure for saving energy in concrete producing facilities Response:.................................................................................. |  |  |
|  | Upgrade technology for enhancing energy effeciency in sludge pyrolysis Response:.................................................................................. |  |  |
| SDG#11 “Sustainable cities” | Proportion of urban population living near sludge dewatering units Response:.................................................................................. |  |  |
|  | Proportion of population that has convenient access to public transport influenced by sludge transfer using trucks Response:.................................................................................. |  |  |
|  | Does concrete manufacturing systems affect the cultural and natural heritage of the city Response:.................................................................................. |  |  |
|  | Does concrete manufacturing systems affect the air quality parameters (e.g., PM2.5 and PM10) of the city Response:.................................................................................. |  |  |
|  | Average share of the built-up area of cities that is open space for public use before and after the construction of cement industries Response:.................................................................................. |  |  |
|  | Number of cities that adopt and implement disaster risk reduction strategies from sludge disposal into landfills Response:.................................................................................. |  |  |
| SDG#17 “Partnership for sustainable development” | International support to developing countries for enhancing concrete infrastructure Response:.................................................................................. |  |  |
|  | Additional financial resources mobilized for developing countries from multiple sources to implement sludge management systems Response:.................................................................................. |  |  |
|  | Debt service as a proportion of export of cement Response:.................................................................................. |  |  |
|  | Describe development cooperation related to biochar production Response:.................................................................................. |  |  |
|  | Transfer of environmentally sound technologies for concrete production to developing countries Response:.................................................................................. |  |  |
|  | Increasing the ability of a company to produce concrete more efficiently, sustainably, and effectively (capacity building in concrete production) Response:.................................................................................. |  |  |
|  | Does concrete production has significant macroeconomic impacts, acting as a major driver of economic activity and employment, especially within the construction industry Response:.................................................................................. |  |  |

Table S7. Sustainable development goals (SDGs) target indicator matrix, showing correlation with endpoint environmental impacts

| SDG | Human health | Ecosystem quality | Climate change | Resources |
| --- | --- | --- | --- | --- |
| SDG 1 | 0.142857143 | 0.14 | 0 | 0.14285714 |
| SDG 3 | 0.153846154 | 0 | 0 | 0 |
| SDG 6 | 0.25 | 0.375 | 0 | 0.125 |
| SDG 7 | 0 | 0.2 | 0 | 0.2 |
| SDG 8 | 0 | 0.0833333 | 0 | 0 |
| SDG 9 | 0.125 | 0 | 0 | 0.375 |
| SDG 11 | 0 | 0.1 | 0.1 | 0.1 |
| SDG 12 | 0.181818182 | 0.0909091 | 0.0909091 | 0.09090909 |
| SDG 13 | 0.2 | 0 | 0.4 | 0 |
| SDG 15 | 0 | 0.0833333 | 0 | 0 |
| SDG 17 | 0.052631579 | 0.0526316 | 0.0526316 | 0.05263158 |

N=aa2+b2+c2								Equation S1

where, N is the normalized value, and a, b, and c are the real indicator value for each alternative, enabling values to be compared on a common scale

Table S8. Normalized matrix for the endpoint LCA indicators, regarding OPC mixture and sludge landfilling (scenario_1), RS mixture (scenario_2), and TAAS mixture (scenario_3) with the reduction percentage compared to scenario_1

| Endpoint environmental impacts | Scenario_1 | Scenario_2 | Scenario_3 |
| --- | --- | --- | --- |
| Human health | 0.1344 | 0.7889 | 0.5459 |
| Ecosystem quality | 0.1325 | 0.7953 | 0.5466 |
| Climate change | 0.1348 | 0.7883 | 0.5455 |
| Resources | 0.1357 | 0.7842 | 0.5456 |

Table S9. Sustainable development goals (SDGs)-alternative matrix, regarding OPC mixture and sludge landfilling (scenario_1), RS mixture (scenario_2), and TAAS mixture (scenario_3) with the reduction percentage compared to scenario_1

| SDG | Scenario_1 | Scenario_2 | Scenario_3 | Dimension of sustainability |
| --- | --- | --- | --- | --- |
| SDG 1 | 0.058 | 0.338 | 0.234 | Social |
| SDG 3 | 0.021 | 0.121 | 0.083985 | Social |
| SDG 6 | 0.100 | 0.594 | 0.409635 | Environment |
| SDG 7 | 0.054 | 0.316 | 0.218435 | Environment |
| SDG 8 | 0.011 | 0.066 | 0.045546 | Economic |
| SDG 9 | 0.068 | 0.393 | 0.272847 | Economic |
| SDG 11 | 0.040 | 0.237 | 0.163768 | Environment |
| SDG 12 | 0.061 | 0.359 | 0.248135 | Economic |
| SDG 13 | 0.081 | 0.473 | 0.327382 | Environment |
| SDG 15 | 0.011 | 0.066 | 0.045546 | Environment |
| SDG 17 | 0.028 | 0.166 | 0.114925 | Social |

Figure S1. Methodological framework of linking life cycle assessment (LCA) indicators with sustainable developments goal (SDG) scores for best selection of sludge-based concrete scenarios

Sensitivity analysis for life cycle assessment (LCA)/sustainable development goals (SDGs) reliability

Sensitivity analysis was performed to evaluate the stability of scenarios' contribution to the pillars of sustainability. A total number of 30 simulations, i.e., ten for each scenario, were generated to determine the sensitivity of the proposed sustainable development goal (SDG) outputs toward varying the life cycle assessment (LCA) indicators. Fig. S1 shows the score on each sustainability pillar estimated by excluding one to two endpoint indicators followed by recalculating the contribution of each scenario to the sustainability pillars. The ranges of standard deviations were 0.2 – 0.8, 1.2 – 4.7, and 0.8 – 3.3 for scenarios 1, 2, and 3, respectively. Having a scenario's standard deviation below five was the main criterion used to maintain reliable sensitivity results, as previously reported (Heiba et al., 2023). These standard deviation values demonstrated the stability of the scenarios' contributions to the three sustainability pillars based on the chosen endpoint indicators. In addition, there is no change in the final decision, revealing that the RS mix scenario enjoyed the highest overall sustainability score. These observations demonstrate that RS concrete production could maintain a better sustainable industrial sector compared with concrete manufacturing by either ordinary Portland cement (OPC) or thermally activated alum sludge (TAAS). This finding was also supported by the results from LCA, where RS concrete had the least environmental impact (midpoint and endpoint categories). Accordingly, it’s suggested that the SDG framework could also be used to evaluate the implementation of RS in concrete production, giving a comparable assessment approach to the LCA technique.

Figure S2. Sensitivity analysis results showing the effect of removing one to two indicators on the proposed life cycle assessment (LCA)/sustainable development goal (SDG) model response

References

Acharya, P. K., & Patro, S. K. (2023). Evaluation of environmental disturbance indicator using functional performance and life cycle assessment of ferrochrome waste concrete. Journal of Building Engineering, 65, 105788.

Çankaya, S., & Pekey, B. (2019). A comparative life cycle assessment for sustainable cement production in Turkey. Journal of environmental management, 249, 109362.

Dong, Y. H., & Ng, S. T. (2014). Comparing the midpoint and endpoint approaches based on ReCiPe—a study of commercial buildings in Hong Kong. The International Journal of Life Cycle Assessment, 19, 1409-1423.

Heiba, Y., Ibrahim, M. G., Mohamed, A. E., Fujii, M., & Nasr, M. (2023). Developing smart sustainable irrigation matrix (SIM)-based model for selection of best irrigation techniques: A framework to achieve SDGs. Journal of Cleaner Production, 420, 138404. https://doi.org/10.1016/j.jclepro.2023.138404

Hong, J., & Li, X. (2011). Environmental assessment of sewage sludge as secondary raw material in cement production–a case study in China. Waste Management, 31(6), 1364-1371.

Ige, O. E., Olanrewaju, O. A., Duffy, K. J., & Collins, O. C. (2022). Environmental impact analysis of Portland cement (CEM1) using the midpoint method. Energies, 15(7), 2708.

Ismaeel, W. S. (2018). Midpoint and endpoint impact categories in Green building rating systems. Journal of Cleaner Production, 182, 783-793.

Little, M. P. (2003). Risks associated with ionizing radiation: Environmental pollution and health. British medical bulletin, 68(1), 259-275.

Mirkarimi, S. R., Ardakani, Z., & Rostamian, R. (2021). Economic and environmental assessment of tobacco production in Northern Iran. Industrial Crops and Products, 161, 113171.

Payen, S., & Ledgard, S. F. (2017). Aquatic eutrophication indicators in LCA: Methodological challenges illustrated using a case study in New Zealand. Journal of Cleaner Production, 168, 1463-1472.

Rashid, K., Farooq, S., Mahmood, A., Iftikhar, S., & Ahmad, A. (2020). Moving towards resource conservation by automated prioritization of concrete mix design. Construction and Building Materials, 236, 117586.

Rosyid, A., Boedisantoso, R., & Iswara, A. P. (2020). Environmental impact studied using life cycle assessment on cement industry. IOP Conference Series: Earth and Environmental Science, 506(1), 012024.

Shi, S., Huang, B., Ren, F., Duan, L., Lei, J., Wang, Y., & Nakatani, J. (2022). Life cycle assessment of embodied human health effects of building materials in China. Journal of Cleaner Production, 350, 131484.

Taelman, S. E., Schaubroeck, T., De Meester, S., Boone, L., & Dewulf, J. (2016). Accounting for land use in life cycle assessment: the value of NPP as a proxy indicator to assess land use impacts on ecosystems. Science of the Total Environment, 550, 143-165.

Van den Heede, P., & De Belie, N. (2012). Environmental impact and life cycle assessment (LCA) of traditional and ‘green’concretes: Literature review and theoretical calculations. Cement and concrete composites, 34(4), 431-442.

van Zelm, R., Roy, P. O., Hauschild, M. Z., & Huijbregts, M. A. (2015). Acidification. In Life cycle impact assessment (ble. 163-176).

Vieira, D. R., Calmon, J. L., & Coelho, F. Z. (2016). Life cycle assessment (LCA) applied to the manufacturing of common and ecological concrete: A review. Construction and Building Materials, 124, 656-666.

Yellishetty, M., Haque, N., & Dubreuil, A. (2012). Issues and challenges in life cycle assessment in the minerals and metals sector: a chance to improve raw materials efficiency. Non-renewable resource issues: geoscientific and societal challenges, 229-24