# Appendix 1

Table A1: Information of important characteristics for EVs fire experiments

| Exp. ID | Initial mass (kg) | Mass loss % | Peak heat release rate, Qmax (kW) | Time to reach peak heat release rate, tmax (min) | Informed total energy released (kJ) | Estimated total energy released (kJ) | Growth coefficient, αpeak (kW-min2) | Decay coefficient, β (min-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EV1 | 1587 | 15 | 6989 | 16.1 | 5200 | 4995 | 29.8 | -0.08 |
| EV2 | 1540 | 25 | 5203 | 13.0 | 6700 | 6713 | 25.5 | -0.07 |
| EV3 | 1475 | 20 | 7859 | 5.4 | N/A | 6150 | 266.79 | -0.04 |
| EV4 | 1448 | 23 | 8098 | 7.0 | 4910 | 5795 | 166.84 | -0.04 |
| EV5 | 1650 | 23 | 8675 | 10.0 | 4660 | 4993 | 87.32 | -0.04 |
| EV6 | - | - | 7990 | 7.3 | 4630 | 5400 | 150.62 | -0.05 |
| EV7 | - | - | 9722 | 8.1 | 5850 | 6399 | 148.33 | -0.04 |
| EV8 | 1122 | 19 | 4235 | 23.9 | 6314 | 5818 | 4.1 | -0.05 |
| EV9 | 1502 | 18 | 4715 | 27.3 | 8540 | 8003 | 3.4 | -0.07 |
| EV10 | 1520 | 29 | 6189 | 40.9 | 6400 | 6061 | 1.3 | -0.07 |
| EV11 | 1865 | 34 | 6990 | 4.4 | 2800 | 2819 | 295.9 | -0.28 |
| EV12 | 1564 | 17 | 6060 | 5.3 | N/A | 4616 | 152.1 | -0.09 |
| EV13 | 1951 | 35 | 10730 | 15.6 | 4454 | 4273 | 22.4 | -0.31 |
| EV14 | - | - | 5992 | 35.2 | 11800 | 11192 | 4.83 | -0.04 |
| EV15 | 1540 | 18 | 6503 | 17.9 | 8450 | 6792 | 21.3 | -0.05 |
| EV16 | 1685 | 17 | 7199 | 14.7 | 9030 | 7776 | 33.3 | -0.05 |

Table A2: Information of important characteristics for ICE vehicls fire experiments

| Exp. ID | Initial mass (kg) | Mass loss % | Peak heat release rate, Qmax (kW) | Time to reach peak heat release rate, tmax (min) | Informed total energy released (kJ) | Estimated total energy released (kJ) | Growth coefficient, αpeak (kW-min2) | Decay coefficient, β (min-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ICE1 | 1326 | 19 | 5707 | 13.29 | 5900 | 5978 | 36.5 | -0,08 |
| ICE2 | 1096 | 25 | 7100 | 6.00 | 3290 | 4869 | 197.22 | -0,06 |
| ICE3 | 1344 | 25 | 10800 | 8.00 | 4950 | 5982 | 168.75 | -0,06 |
| ICE4 | 1128 | 17 | 4800 | 15.09 | N/A | 7067 | 26.4 | -0,06 |
| ICE5 | 1404 | 19 | 6100 | 18.28 | N/A | 9991 | 12.7 | -0,06 |
| ICE6 | 1275 | 15 | 2107 | 23.38 | N/A | 3992 | 1.3 | -0,05 |
| ICE7 | - | - | 4930 | 17.50 | 3792 | 3615 | 9.6 | -0,33 |
| ICE8 | - | - | 2300 | 12.83 | 1540 | 1415 | 14.1 | -0,20 |
| ICE9 | 1320 | - | 7660 | 20.34 | 8080 | 6943 | 24.2 | -0,07 |
| ICE10 | 2200 | - | 3388 | 27.50 | N/A | 1812 | 1.0 | -0,01 |
| ICE11 | 1440 | - | 3620 | 62.01 | N/A | 5358 | 0.8 | -0,07 |
| ICE12 | 1440 | - | 3155 | 52.50 | N/A | 4860 | 0.8 | -0,06 |
| ICE13 | 1440 | - | 4103 | 20.36 | N/A | 5228 | 6.8 | -0,05 |
| ICE14 | - | - | 3193 | 24.45 | N/A | 667 | 3.8 | -0,02 |
| ICE15 | 1200 | - | 3743 | 25.67 | N/A | 4756 | 3.3 | -0,06 |
| ICE16 | 1580 | 19 | 5845 | 18.64 | N/A | 6430 | 14.0 | -0,07 |
| ICE17 | 1540 | 19 | 4396 | 28.04 | N/A | 5801 | 4.4 | -0,08 |