A life cycle assessment of cover crop ensiling and co-digestion with cattle manure: Appendix A.

Maja Rydgård1*, Lars Stoumann Jensen1, Magdalena Vinkler Schwartzkopff1, Susanna Pinna1, Peter Sørensen2, Sander Bruun1

1Copenhagen University, Bülowsvej 21, 1870 Frederiksberg C, Copenhagen, Denmark

2 Aarhus University, Denmark

*Corresponding author. Email: maja.rydgaard@plen.ku.dk

# Farm description

Data for the definition of the farm system are presented in Tables A.1-A.4, and composition of the cattle manure, cover crops and digestates are defined in Table A.5.

Table A.1. Feed and straw requirements per cow. These data were used to calculate the number of cows that could be supported on the farm.

|  | kg/cow | Tonnes (560 cows) |
| --- | --- | --- |
| Barley/oat grains | 2400 | 1344 |
| Straw for feed/bedding | 600 | 336 |

Table A.2. Area requirements for roughage per cow. These data were used to calculate the number of cows that could be supported on the farm.

|  | Hectares per cow | Hectares (560 cows) |
| --- | --- | --- |
| Grass-clover grazing | 0.347 | 194 |
| Grass-clover silage | 0.429 | 240 |
| Pea-barley silage | 0.113 | 63.3 |
| In total for roughage | 0.888 | 497.3 |

Table A.3. Calculation of the amount of manure available for field application after grazing. Grazing time 6 hours per day from 15 April to 1 November, assuming 10 days exception due to poor weather.

|  | Value | Unit |
| --- | --- | --- |
| Manure produced | 32.06 | Mg/cow |
| Manure total | 17,953.6 | Mg |
| Grazing days | 189 | Days |
| Grazing time | 47.25 | Days |
| Manure for grazing field | 2,324 | Mg |
| Manure field applied | 15,630 | Mg |

Table A.4. Manure distribution on the two rotations. Rotation 1 was included in the LCA system, while rotation 2 was excluded as this rotation was assumed to be unaffected by the changes investigated in the scenarios.

| Rotation 1 | Area (ha) | Manure (Mg ha-1) | Manure N (kg N ha-1) |
| --- | --- | --- | --- |
| Spring barley | 125 | 28 | 115 |
| Spring barley | 125 | 28 | 115 |
| Spring barley | 125 | 28 | 115 |
| Spring barley | 125 | 28 | 115 |
| Rotation 2 |  |  |  |
| Pea-barley silage | 63 | 28 | 60 |
| Grass-clover silage | 240 | 0 | 0 |
| Grass-clover grazing | 97 | 12 | 46 |
| Grass-clover grazing | 97 | 12 | 46 |

Table A.5. Composition of cattle manure (CM), cover crop (CC) and straw silage (co-ensiling ratio 3:1), and digestates of scenarios CCD (digested CM+silage) and CMD (digested cattle manure).

|  | VS(%DM) | DM(%FM) | g N kg-1 DM | NH4-N/total N | TAN (g N kg-1 DM) | TAN (g N Mg-1) | total C (g kg-1 DM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CM | 82.1 | 6.5 | 49.1 | 0.51 | 25.0 | 1.63 | 415 |
| Silage CC:straw 3:1 | 95.2 | 31 | 14.6 | 0.08 | 1.17 | 0.36 | 457 |
| Digested CM+silage | 85.3 | 6.1 | 40.8 | 0.52 | 21.2 | 1.29 | 330 |
| Digested CM | 76.2 | 6.1 | 69.4 | 0.61 | 42.3 | 2.58 | 308 |

# Storage emissions

Methane

Methane emissions (kg CH4) from slurry storage and from anaerobic digestion and digestate storage were estimated (DCE, 2021):

CH4=VS∙B0∙MCF100∙0.67

The equation was based on the IPCC Tier 2 method, equation 10.23  (Gavrilova et al., 2019). VS is the volatile solids (kg/ton), B0 is the maximum methane producing capacity (m3 CH4 per kg VS), MCF is the methane conversion factor, and 0.67 is a conversion factor from m3 CH4 to kg CH4.

The VS content in manure and digestates were obtained from Feng et al. (2020). The maximum methane producing capacity B0 of dairy cattle manure in Europe was taken from IPCC (Gavrilova et al., 2019) as 0.24 m3 CH4 kg VS-1, and for the digestates this was obtained from Feng et al. (2020) as the residual methane potential (0.111 m3 CH4 (kg VS)-1 for CMD and 0.162 for CCD).

The methane conversion factor MCF represents the degree to which B0 is achieved, and is affected by the manure management system, such as the temperature, storage cover, and the retention time of the organic material in the digester and in the storage (Gavrilova et al., 2019). Emissions are highly sensitive to the temperature, they increase exponentially with increasing temperature (Gavrilova et al., 2019). National MCF values for raw and digested cattle slurry in Denmark were estimated by Mikkelsen et al. (2016) to 12.4 % for raw slurry and 7.48 % for digested slurry (DCE, 2021). The MCF of CCD was assumed to be the same as for CMD, as no MCF value for co-digested slurry and clover or straw was available.

Nitrous oxide

- The formation of N2O during storage is dependent on whether a surface crust is formed, as nitrifying and denitrifying bacteria grow in the crust (Olesen et al., 2021). When high-fiber biomass is used in anaerobic digestion, this can contribute to formation of a surface crust on the digestate during storage. The IPCC suggests an emission factor for N2O from storage tanks with surface crust of 0.005 kg N2O-N per kg N in manure, and zero for when there is no surface crust (Gavrilova et al., 2019). Olesen et al. (2021) suggest there is no significant difference between slurry and digestate in the formation of surface crust and the emissions of N2O. The emission factors for both are thus expected to vary between 0 and 0.5 % of total N, and were inventoried as 0 %, as differences between scenarios were assumed negligible. Indirect emissions of N2O from volatilization of N were estimated as 1.4 % of the ammonia emissions (Gavrilova et al., 2019; Hergoualc’h et al., 2019).

The N2O emission factors for slurry and digestate storage are expected to vary between 0 and 0.5% of total N. Direct emissions of N2O were calculated:

N2Odirect=EF∙N∙4428

where EF is the emission factor (kg N2O-N per kg N), N is the total N in the manure, and 44/28 is the conversion from N2O-N to N2O.

Volatilization of N contributes to indirect emissions of N2O by deposition of N in the environment, which was estimated as 1.4% of the ammonia emissions (Gavrilova et al., 2019; Hergoualc’h et al., 2019):

N2Oindirect, volatilization=NH3∙0.014∙4428

Table A.6. Storage emission factors for cattle manure (CM), cattle manure digestate (CMD), and cover crop silage and cattle manure digestate (CCD).

|  | CH4 (kg CH4 ton-1) | NH3 (kg NH3 ton-1) | N2O, direct (0.5% of total N), (g N2O ton-1) | N2O, indirect, volatilization (g N2O ton-1) |
| --- | --- | --- | --- | --- |
| CM | 1.06 | 0.034 | 25.07 | 0.61 |
| CMD | 0.26 | 0.081 | 33.25 | 1.48 |
| CCD | 0.42 | 0.041 | 19.55 | 0.74 |

Indirect N2O emissions were subtracted from the ammonia emissions, resulting in 0.033, 0.080 and 0.040 kg NH3 ton-1 for cattle manure, CMD and CCD.

## Carbon dioxide

Emissions of CO2 from digested slurry are similar to those from untreated slurry, according to a review on emissions from slurry storage facilities (Kupper et al., 2020). Carbon dioxide emissions were modelled as biogenic carbon, meaning that they do not contribute to global warming.

# Daisy model set-up

## Soil profiles

The properties of the soil profiles modelled in Daisy are given in Table A.7.

Table A.7. Properties and van Genuchten-Mualem hydraulic parameters of the soil profile for the two soil types used in the simulations.

| Sandy loam, Foulum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizon (cm) | Clay (%) |  |  | Silt (%) | Organic matter (%) |  |  | Bulk density (g/cm3) |  |  | θsat | α |  |  | n |  | Ksat (cm/h) |  | l |  |  | C:N |  |  |
| A1 (0 - 36) | 6.7 |  |  | 9.4 | 2.9 |  |  | 1.54 |  |  | 36.45 | 0.02765 |  |  | 1.28838 |  | 0.34 |  | 0.5 |  |  | 11.2 |  |  |
| A2 (36-57) | 6.7 |  |  | 8.6 | 1.6 |  |  | 1.42 |  |  | 44.64 | 0.04571 |  |  | 1.36577 |  | 1.35 |  | 0.5 |  |  | 11.2 |  |  |
| B (60-98) | 9.2 |  |  | 6.8 | 0.2 |  |  | 1.75 |  |  | 29.38 | 0.02956 |  |  | 1.25506 |  | 0.95 |  | 0.5 |  |  | 9.1 |  |  |
| C (98-132) | 9.4 |  |  | 8.2 | 0.1 |  |  | 1.86 |  |  | 26.82 | 0.01057 |  |  | 1.2434 |  | 0.95 |  | 0.5 |  |  | 9.1 |  |  |
| Sandy soil, Holstebro |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizon (cm) |  |  | Clay (%) |  |  |  | Silt (%) |  |  | Very fine sand (%) |  |  | Fine sand (%) |  |  | Medium sand (%) |  |  |  | Coarse sand (%) |  |  |  | Very coarse sand (%) |  |  |
| A (0-30) |  |  | 6.6 |  |  |  | 6.2 |  |  | 6.9 |  |  | 16.2 |  |  | 25.7 |  |  |  | 25.7 |  |  |  | 5.8 |  |  |
| B1 (30-47) |  |  | 3.6 |  |  |  | 0.6 |  |  | 1.4 |  |  | 34.5 |  |  | 16.7 |  |  |  | 17.6 |  |  |  | 14.9 |  |  |
| B2 (47-90) |  |  | 1.8 |  |  |  | 0.2 |  |  | 0.7 |  |  | 4.1 |  |  | 14.6 |  |  |  | 35.1 |  |  |  | 23.9 |  |  |
| C (90-300) |  |  | 2.9 |  |  |  | 0 |  |  | 0 |  |  | 3.7 |  |  | 20 |  |  |  | 47.8 |  |  |  | 20 |  |  |
| Horizon (cm) |  | Organic matter (%) |  |  |  | Bulk density (g/cm3) |  |  | θsat |  |  | α |  | n |  |  |  | Ksat (cm/h) |  |  | l |  | C:N |  |  |
| A (0-30) |  | 7.8 |  |  |  | 1.48 |  |  | 0.455 |  |  | 0.04 |  | 1.455 |  |  |  | 57.8 |  |  | 0.018 |  | 26.59 |  |  |
| B (30-47) |  | 1.7 |  |  |  | 1.65 |  |  | 0.349 |  |  | 0.0828 |  | 2.271 |  |  |  | 535.7 |  |  | 0.525 |  | 16.18 |  |  |
| B (47-90) |  | 0.7 |  |  |  | 1.65 |  |  | 0.307 |  |  | 0.1408 |  | 3.774 |  |  |  | 3142.1 |  |  | 2.071 |  | 13.77 |  |  |
| C (90-300) |  | 0.5 |  |  |  | 1.76 |  |  | 0.365 |  |  | 0.1714 |  | 2.139 |  |  |  | 8294.4 |  |  | 3.459 |  | 10.37 |  |  |

## Weather

For the scenario modelling with Daisy, 100 years of generated weather data from eastern Denmark was used (Rasmussen et al., 2018). Rasmussen et al. (2018) generated 3000 years of weather data for based on 30 years of historical weather data (1983–2012) from eastern Denmark (Copenhagen Airport weather station). The data, which was used as a control by Rasmussen et al., was used for the Daisy simulations in the present study.  Of the generated weather data from Rasmussen et al., years 2010-2109 were used in the simulations.

Because precipitation in eastern Denmark (675.9 mm yr-1 annual average) is low relative to other regions in the country, the precipitation was up-scaled to the precipitation level of Foulum (for the sandy loam soil) and Holstebro (for the sandy soil) based on the average annual precipitation of years 2011-2023 from DMI weather archive (dmi.dk) (Table A.8).

Table A.8. Annual average precipitation in Aarhus and Holstebro, which was used for calculation of the scaling factors for scaling of precipitation.

|  | Annual average precipitation 2011-2023 (mm) | Scaling factor |
| --- | --- | --- |
| Foulum, Aarhus | 712.24 | 1.05 |
| Holstebro | 956.2 | 1.41 |

## Cover crop modelling

The data used for calibration of the ryegrass and clover crop modules comes from an ongoing long-term field trial at Foulumgaard, Denmark, which has been running since 1987 (Eriksen et al., 1999; 2004; 2008; 2015; 2014; Jensen et al. 2022). The soil is a sandy loam (Mollic Luvisol) and the site has an average annual temperature of 8.8°C and 832 mm precipitation. The field trial initially consisted of one crop rotation (spring barley, grass clover ley, grass clover ley, winter wheat, oats, and fodder beats or maize). In 2006 the rotation was split into two, one with a two-year ley (spring barley, grass clover ley, grass clover ley, spring barley, oats, and barley-pea mixture) and one with a four-year ley (spring barley, grass clover ley, grass clover ley, grass clover ley, grass clover ley, spring barley). The trial has six replicates with the rotations permutated, such that each crop in the rotation occurs in each experimental year. The aim of the calibration is to i.) improve the simulated yield dynamics such that the model responds to the observed species compositions and yield levels in response to ley age and to cuts within the season, and ii.) accurately simulate the observed soil organic carbon increase as a function of ley proportion in the crop rotation by increasing simulated belowground biomass. The measurements used for crop model parameter calibration includes ley dry-matter- and N-yields per cut from 2006-2022 for ryegrass and clover separately, dry-matter yield of main crops from 2006-2010 and 2017-2021, root and stubble biomass and nitrogen content in ley after the third cut (August) in 2020, soil water nitrate concentrations collected by the suction cup method during 2006-2010 and 2017-2021, and soil carbon content at 0-25 cm depth measured in 1987, 1992, 2005, 2009, 2019, and 2022. The data for model evaluation is from a short-term field trial at Esbjerg, Denmark, run from 2017-2019 (Fontaine et al., 2022; Kristensen et al., 2022). The measurements include dry-matter- and N-yield per cut for 2018 and 2019, and soil water nitrate concentrations collected by the suction cup method from April 2018 to March 2020.

## Soil carbon initialization

For initialization of the soil carbon content in the model, an initialization simulation was used, which replicated a field experiment of Fontaine et al. (2020a), in which a mixture of clover, ryegrass and chicory were under-sown in spring barley. Field operations in the initialization simulation were modelled on the same dates as they took place in the field experiment. In the simulation, clover and ryegrass were under-sown on May 22nd in spring barley (sown April 27th), and cattle manure was applied before barley sowing. Barley was harvested at maturity or latest Sept. 25th with 8 cm stubble. In the following year, rotary cultivation was conducted on March 20th, and cover crops were incorporated by ploughing on April 1st, as it is the current common practice at farms in Denmark where cover crops are used. The initialization simulations were run for 30 years using weather data from Foulum, on the two soils. The observed annual carbon input was used as the initial organic matter in the different scenarios, including the reference, CCD and CMD (Bruun & Jensen, 2002) (Table A.9).

Table A.9. Annual carbon input to the soil, used for initialization of soil carbon. Inputs from roots are part of the total inputs.

|  | Sandy loam | Sandy soil |
| --- | --- | --- |
| Total carbon input (kg C/ha/yr) | 1967 | 1828 |
| Input from roots (kg C/ha/yr) | 1086 | 1000 |

## Calibrations

C and N mineralisation of digestate and cattle manure were calibrated based on soil incubation data from Fontaine et al. (2020b) for N, from Cavalli et al. (2017), de la Fuente et al. (2013), Johansen et al. (2013), Sørensen (1998) and Sørensen et al. (2003) for cattle manure C, and from Alburquerque et al. (2012), Cavalli et al. (2017), Johansen et al. (2013), Müller-Stöver et al. (2016) and Viaene et al. (2017) for digestate.

The calibrated parameters are listed in Table A.10 and calibration curves are shown in Figs. A.1-A6. FAOM1 and FAOM2 are the fractions of C partitioned to the added organic matter (AOM) pools: the slow pool AOM1 and the fast pool AOM2. The remaining C fraction that is not partitioned to AOM1 or AOM2 is partitioned to the slow soil organic matter (SOM) pool. CNAOM1 is the C/N ratio of AOM1. kAOM1 and kAOM2 are the daily mineralization rates of each pool (Bruun et al., 2003).

Table A.10. Calibrated parameters for C and N mineralization of cattle manure, cattle manure digestate (CMD) and cover crop, straw and cattle manure co-digestate.

|  | Cattle manure | CMD | CCD |
| --- | --- | --- | --- |
| FAOM1 | 0.70 | 0.76 | 0.77 |
| FAOM2 | 0.22 | 0.15 | 0.14 |
| kAOM1 (d-1) | 8.00E-04 | 8.30E-04 | 8.30E-04 |
| kAOM2 (d-1) | 1.00E-01 | 9.98E-02 | 6.72E-02 |
| CNaom1 | 20.13 | 10.63 | 16.04 |
| FAOM1+FAOM2 | 0.92 | 0.91 | 0.91 |
| R2 C | 0.56 | 0.75 | 0.73 |
| R2 N | 0.69 | 0.05 | 0.09 |
| R2 total | 0.60 | 0.63 | 0.75 |

Cattle manure

| Figure A.1. Calibration of carbon mineralisation of cattle manure. The line is the calibrated curve and the dots are measured values. | Figure A.2. Calibration of nitrogen mineralisation of cattle manure. The line is the calibrated curve and the dots are measured values (Fontaine et al., 2020). |
| --- | --- |

Cattle manure digestate

| Figure A.3. Calibration of carbon mineralisation of CM digestate. The line is the calibrated curve and the dots are measured values. | Figure A.4. Calibration of nitrogen mineralisation of CM digestate. The line is the calibrated curve and the dots are measured values (Fontaine et al., 2020). |
| --- | --- |

Cover crop and cattle manure co-digestate

| Figure A.5. Calibration of carbon mineralisation of CCD digestate. The line is the calibrated curve and the dots are measured values. | Figure A.6. Calibration of nitrogen mineralisation of CCD digestate. The line is the calibrated curve and the dots are measured values (Fontaine et al., 2020). |
| --- | --- |

## Daisy simulation results

In the Ecoinvent process for barley production, the moisture content of barley is assumed to be 14%, which was used to calculate the barley substitution, presented in Table A.11 as the yield increase or decrease relative to the reference.

Table A.11. Nitrate leaching and effects on soil carbon and barley yield of the three scenarios modelled with Daisy. The values are annual averages based on 100-year simulations.

| Scenario | Leaching (kg N ha-1) | Leaching & runoff (kg NO3-ha-1) | Soil C year 100 (Mg C ha-1) | Soil C annual change relative to reference (kg CO2 ha-1) | Yield (Mg dm ha-1) | Yield relative to reference (kg FM ha-1) |
| --- | --- | --- | --- | --- | --- | --- |
| Sandy loam |  |  |  |  |  |  |
| Reference | 2.22 | 9.88 | 152.2 |  | 5.33 |  |
| CCD | 3.06 | 14.35 | 144.0 | -298.2 | 5.44 | 122.7 |
| CMD | 2.46 | 10.93 | 147.5 | -169.1 | 5.40 | 80.7 |
| Sandy soil |  |  |  |  |  |  |
| Reference | 22.5 | 99.8 | 253.5 |  | 5.59 |  |
| CCD | 27.5 | 122.2 | 242.9 | -390.7 | 5.63 | 50.8 |
| CMD | 23.9 | 105.9 | 248.6 | -179.6 | 5.61 | 22.7 |

# Field emissions

Emissions of N2O are calculated:

N2Odirect=EF∙N∙4428

where EF is the emission factor 0.006 kg N2O-N per kg total N, N is the total N in the manure or crop residues, and 44/28 is the conversion from N2O-N to N2O. Volatilization of N contributes to indirect emissions of N2O, which was estimated as 1.4% of the ammonia emissions (Gavrilova et al., 2019; Hergoualc’h et al., 2019):

N2Oindirect, volatilization=NH3∙0.014∙4428

Moreover, indirect emissions also come from losses of nitrogen to surface waters (Gavrilova et al., 2019; Hergoualc’h et al., 2019):

N2Oindirect, leaching/runoff=Nleaching∙0.011∙4428

Table A.12. Emission factors for emissions from the field, for cattle manure (CM), cattle manure digestate (CM digestate), and cover crop silage and cattle manure digestate (CC+CM digestate).

| Sandy loam | NH3 (kg NH3 ha-1) | N2O, direct (kg N2O ha-1) | N2O, indirect, volatilization (g N2O ha-1) | N2O, indirect, leaching (kg N2O ha-1) |
| --- | --- | --- | --- | --- |
| CM | 0.86 | 0.82 | 11.16 | 0.04 |
| CCD digestate | 2.17 | 1.26 | 28.38 | 0.07 |
| CM digestate | 1.65 | 0.83 | 21.53 | 0.05 |
| CC incorporation |  | 0.38 |  |  |
| Sandy soil |  |  |  |  |
| CM | 0.86 | 0.82 | 11.16 | 0.42 |
| CCD digestate | 2.18 | 1.26 | 28.46 | 0.64 |
| CM digestate | 1.65 | 0.83 | 21.53 | 0.52 |
| CC incorporation |  | 0.40 |  |  |

Variations in N2O emissions were estimated by varying the emission factors of manure, digestates, and cover crop residue incorporation to the lower and upper end of the uncertainty ranges given by the IPCC. The emission factors of cattle manure and digestates were varied to 0.1% of total N (low value), and 2% of total N (high value). The emission factor for cover crop residue incorporation was varied between 0.1 and 1.1% of total N. The variations that resulted in the highest differences from the standard scenario were applied in the sensitivity analysis.

Table A.13. Nitrous oxide emissions (kg N2O ha-1) of scenarios CCD and CMD relative to the reference scenario, with varying emission factors (EFs) for cattle manure, digestates and cover crop residue soil incorporation. The variations that resulted in the highest differences from the standard scenario were applied in the sensitivity analysis, marked in bold.

|  | All EFs low | All EFs high | manure EF high, residue standard EF | manure EF high, residue EF low | manure EF low, residue EF high |
| --- | --- | --- | --- | --- | --- |
| CCD | 0.03 | 0.18 | 0.41 | 0.64 | -0.43 |
| CMD | -0.02 | -0.44 | -0.21 | 0.02 | -0.48 |

# Cover crop growth and residue incorporation

Cover crop growth and harvest (scenario CCD) are included in Table A.14. Harvested straw yields in scenario CCD were 2.77 Mg ha-1 on the sandy loam and 3.12 Mg ha-1 on the sandy soil.

Table A.14. Cover crop growth in the three scenarios as estimated with the Daisy model (unit: Mg dm ha-1).

| Scenario | Clover | Ryegrass | Total |
| --- | --- | --- | --- |
| Sandy loam |  |  |  |
| Reference | 0.95 | 1.05 | 2.00 |
| CCD | 0.66 | 1.55 | 2.21 |
| CMD | 0.93 | 1.10 | 2.04 |
| Sandy soil |  |  |  |
| Reference | 1.01 | 1.17 | 2.18 |
| CCD | 0.72 | 1.66 | 2.38 |
| CMD | 0.99 | 1.00 | 1.99 |

Table A.15. Overview of N inputs in the three scenarios, as modelled in Daisy (unit: kg N ha-1). Residues N input includes N from above ground plant residuals, including both exfoliation during crop growth, and any above ground residuals left after harvest. Fertilizer N includes N added with fertilizer (cattle manure or digestate), in mineral and organic form.

| Scenario | Aboveground residues N input | Fertilizer mineral-N | Fertilizer organic N | Fertilizer total N | Residue + fertilizer N |
| --- | --- | --- | --- | --- | --- |
| Sandy loam |  |  |  |  |  |
| Ref (CC incorp) | 111.8 | 45.0 | 43.3 | 88.3 | 200 |
| CCD | 70.4 | 70.8 | 65.4 | 136.2 | 207 |
| CMD | 114.1 | 53.8 | 34.4 | 88.3 | 202 |
| Sandy soil |  |  |  |  |  |
| Ref | 110.4 | 44.7 | 43.3 | 88.3 | 199 |
| CCD | 66.8 | 70.1 | 65.5 | 136.6 | 203 |
| CMD | 111.1 | 53.0 | 34.4 | 88.3 | 199 |

A balance over N in harvested cover crops and straw is included in Table A.16.

Table A.16. Harvested N in scenario CCD (all values are in kg N/ha) in cover crops (CC) and straw. ‘Straw N to AD’ is the N in straw for ensiling after deduction of straw for cow stable bedding. ‘CC+straw N to AD’ is the total N in cover crop- and straw silage for anaerobic digestion after 6 % N loss during ensiling. ‘Total N to AD’ includes N in silage and 88.3 kg N in cattle manure for anaerobic digestion. N left in digestate is after 0.06 % N loss in AD.

|  | CC-N harvested | Straw-N harvested | Straw N to silage | CC+straw N to AD | Total N to AD | N in digestate |
| --- | --- | --- | --- | --- | --- | --- |
| Sandy loam | 34.2 | 20.6 | 15.6 | 48.5 | 136.8 | 136.7 |
| Sandy soil | 36.1 | 17.7 | 13.9 | 48.6 | 136.9 | 136.9 |

# Ensiling

Baling and wrapping of bales in PE-film plastic was modelled using the Ecoinvent process for baling. One bale in the Ecoinvent process is 1.4 m3 and 700 kg, i.e. the silage density is 500 kg m-3. For 20 Mg silage per hectare, this results in 29 bales per hectare. The diesel requirement for baling was 0.743 kg per bale, i.e. 21.6 kg per hectare. The plastic use was 1 kg PE plastic and 1 kg extrusion per bale, resulting in 31 kg PE plastic and 31 kg extrusion per hectare.

Loading of silage bales onto trailer with bale gripper was modelled using the ecoinvent process for bale loading, using the European market process for diesel. The diesel consumption was 10.3 kg diesel per hectare.

Transportation of silage from the field to the farm storage was estimated to 1.6 km, assuming that the farm storage would be located close to the field (500 hectares). The transportation was modelled using the ecoinvent process for transportation with tractor and trailer, which has a diesel requirement of 0.0436 kg (Mg*km)-1. This results in 1.4 kg diesel per hectare for transportation of silage bales. In total, the combined diesel requirement of baling, bale loading, and transportation of bales amounted to 33.3 kg ha-1 (or 1.64 kg Mg-1).

## Sensitivity analysis of ensiling diesel use

In an LCA on ensiling technologies, Strid and Flysjö (2007) estimated the diesel requirements for ensiling operations, including turning of organic material (forage) on the field (0.057 kg Mg TS-1), bale pressing (0.33 kg Mg TS-1), tractor loading and transport to storage (0.09 kg Mg TS-1), and ensiling in plastic bags (0.33 kg Mg TS-1). In total, the total diesel requirement was estimated to 0.8 kg diesel Mg TS-1. For silage with 13% TS (Fontaine, Eriksen, et al., 2020), this corresponds to 6.3 kg diesel Mg FM-1. The energy density of diesel is approximately 38 MJ L-1, and the mass density is 0.85 kg L-1 (Speight, 2011). Thus, this corresponds to 5.35 L (Mg FM)-1, or 0.20 MJ (Mg FM)-1. This is similar to the estimate by Møller et al (2020), who estimated the energy use for ensiling to 5 liters diesel per ton FM straw (42% TS). As the diesel requirements estimated by Strid and Flysjö (2007) and Møller et al. (2020) were higher than the estimates based on the Ecoinvent processes, this was investigated in a sensitivity analysis, by altering the diesel requirement to 6.3 kg diesel (Mg FM)-1 (as in Strid and Flysjö (2007)).

# Anaerobic digestion: Mass balance

The carbon and nitrogen in the feedstock to the anaerobic digester were balanced against the output biogas, digestate, and losses:

CCM+CCC:straw=CH4, biogas+CO2, biogas+Cdigestate+Closses

NCM+NCC:straw=Ndigestate+Nloss

## Carbon

The given C and N balance is for the CCD scenario on sandy loam soil.

Input

| CC:straw 3:1 silage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g C/kg dm | DM(%FM) | g C/kg FM | Silage mass (t fm/ha) | Silage dm | Silage kg dm/ha | kg C in silage per ha | % in AD | VS (%dm) | kg VS/ha |
| 457 | 31 | 142 | 20.51 | 0.127 | 2605 | 1190.6 | 20 | 95.2 | 2480.2 |
| CM |  |  |  |  |  |  |  |  |  |
| g C/kg dm | DM (%FM) | g C/kg FM | CM mass (ton fm/ha) | CM dm | CM kg dm/ha | kg C in CM per ha | % in AD | VS (%dm) | kg VS/ha |
| 415 | 6.5 | 27 | 27.67 | 0.065 | 1798 | 746.4 | 80 | 82.1 | 1476.5 |

Output

| Digestate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g C/kg dm | DM (%FM) | g C/kg FM | Digestate fm (t/ha) | Digestate kg dm/ha | kg C/ha | VS(%) | VS (%dm) | kg VS/ha | kg VS/ha (21%VS red.) |
| 330 | 6.1 | 20.13 | 54.922 | 3350.3 | 1105.6 | 5.56 | 85.3 | 2857.8 | 2793.4 |
| Biogas |  |  |  |  |  |  |  |  |  |
| CH4 yield (mL/g VS) | L/ha | m3/ha | CH4 density (kg/m³) | kg CH4/ha |  | kg CH4-C/ha | CH4 molar mass (g/mol) | M CH4 (mol/ha) | M/TS (mol/kg) |
| 276.24 | 1,092,975 | 1093.0 | 0.657 | 718.08 |  | 537.7 | 16.04 | 44,768 | 13.36 |

Nitrogen

Loss of N was 0.06 % of total N (Fontaine et al., 2020).

Input

| CC:straw 3:1 silage |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| g N/kg dm | g N/kg FM | kg N in silage per ha | NH4-N/total N | g NH4-N/kg dm | g NH4-N/kg ww | kg NH4-N/ha |
| 18.605 | 5.77 | 48.47 | 0.08 | 1.49 | 0.46 | 3.88 |
| CM |  |  |  |  |  |  |
| g N/kg dm | g N/kg FM | kg N in CM per ha | NH4-N/total N | g NH4-N/kg dm | g NH4-N/kg ww | kg NH4-N/ha |
| 49.1 | 3.192 | 88.30 | 0.51 | 25.04 | 1.63 | 45.03 |

Output

| Digestate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| g N/kg dm | g N/kg FM | kg N/ha | NH4-N/total N |  |  | kg NH4-N/ha |
| 40.8 | 2.49 | 136.7 | 0.52 |  |  | 71.08 |

Heat requirements

The heat requirements for heating of biomass were calculated:

- Q=mc∆T

Where Q is energy, c is specific heat capacity, and T is temperature.

The following assumptions were made:

The specific heat capacity of cattle manure is similar to that of water, as the moisture content is high (91.5 % FM). The specific heat of water is 4.184 kJ/kg, K.

The specific heat capacity of silage is 1.89 kJ/kg, K per dry matter (Borreani & Tabacco, 2010). The silage has 31 % DM (Fontaine, Feng, et al., 2020).

The specific heat of silage was estimated as 0.31*1.89+(1-0.31)*4.184 = 3.47 kJ/kg, K

Biomass T1=15 °C (Olesen et al., 2021); T2=52°C (thermophilic operation)

Biomass temp leaving reactor T3=25°C; temp leaving biogas plant T3=20°C (Olesen et al., 2021); outside temp T0= 8°C.

- The energy requirements were calculated to 7930 MJ/ha (CCD), and 4283 MJ/ha (CMD) for heating of biomass.

- Heat exchange

Using heat exchange for utilization of heat in the digestate leaving the biogas reactor, the temperature in the digestate was reduced from 52°C to 25°C. The energy from the heat exchange was calculated to reduce the energy requirement by 6180.6 MJ/ha (CCD), and 2355 (CMD).

- Heat loss

- Heat loss through the insulation was calculated:

- Q=αA∆T

α is the heat transfer rate (kW), which for an exposed, insulated tank is approximately 0.35 W m-2 K-1 (The Engineering ToolBox, 2003). This is a conservative value for a biogas tank, meaning that heat loss may be lower (Henrik Møller, personal communication).

A is the surface area of the tank, estimated to 894 m2 for a 3372.6 m3 tank (diameter D=15.09 m, height H=18.86 m, to reach a diameter to height relation of H=1.25·D). A=π·D·H

∆T is the temperature difference between the biomass in the tank and the outside temperature.

Q=0.35 W m-2 K-1 * 894 m2 * (52-8) °C = 13.8 kW (i.e. 0.0138 MJ/s), which is 190.34 MJ/24 hours, and in one year that is 69.5 GJ/500 ha, or 138.9 MJ/ha in scenario CCD. In scenario CMD, the heat loss was 88.2 MJ/ha.

- In total (per hectare), heating of biomass + maintaining the temperature (compensation for the heat loss) – heat recovered from heat exchange is 1888 MJ/ha in scenario CCD. The net heat requirement in scenario CMD was 2016 MJ ha-1.

Biogas yield

Biogas yields vary considerably between studies and are associated with uncertainty (Olesen et al., 2021). The methane yield of cover crop silage and cattle manure co-digestion measured by Feng et al. (2020) was 238.4 mL g VS-1, and for cattle manure mono-digestion it was 212.2 mL g VS-1, after 25 days retention time (HRT). The following methane yields after 45 days were considered as typical yields, based on experience from the Foulum biogas plant (Olesen et al., 2021): 230 L kg VS-1 for cattle slurry, 263 L kg VS-1 for cattle deep litter, 278 L kg VS-1 for wheat straw and 324 L kg VS-1 for grass silage. The ultimate methane yields of the same substrates were 325 mL g VS-1 for grass silage, 290 mL g VS-1 for wheat straw, and 275 mL g VS-1 for cattle slurry and cattle deep litter. With approximately 60% cattle manure and 40% grass silage in the reactor, of which 55 % straw and 45 % clover-grass silage, the theoretical ultimate methane yield was estimated to approximately 287 mL g VS-1 in scenario CCD. In scenario CMD, the ultimate methane yield was assumed to be 275 mL g VS-1, as reported for cattle slurry and cattle deep litter by Olesen et al. (2021).

The cumulative methane production can be estimated (Olesen et al., 2021):

Mt=B0(1-e-kt) 	(2)

Where M is the methane yield (mL g VS-1), B0 is the theoretical methane yield (mL g VS-1), k is a kinetic rate constant, and t is time. Using the methane yield measured by Feng et al. (2020) after 25 days, k was calculated to 0.0724 d-1 for scenario CCD and 0.059 for scenario CMD, using ultimate methane yields (B0) estimated based on Olesen et al. (2021) (287 for CCD and 275 for CMD). After that, the methane yield for an average retention time of 45 days was calculated to 276.2 mL g VS-1 for scenario CCD-sandy loam, 276.6 mL g VS-1for CCD-sandy soil and 256 mL g VS-1 for scenario CMD.  As the theoretical methane yield for specific substrates and mixtures, as well as the rate at which methane is produced, are sources of uncertainty (Olesen et al., 2021), the sensitivity to the methane yield was tested in the robustness analysis.

Biogas upgrading

The scrubbing was modelled using the Ecoinvent process for biogas purification to biomethane by amino washing. The upgraded gas leaves the process with a pressure of 5 bar, with more than 96 vol. % of methane (ecoinvent). The upgraded biomethane is modelled to replace fossil gas at the market. The common distribution of natural gas for vehicle fuel in Denmark is in high-pressure compressed natural gas (CNG). The market for high pressure natural gas was modelled using the Ecoinvent process. The volume of natural gas replaced was calculated, assuming the density of natural gas was 0.735 kg m-3 (ecoinvent).

Life cycle impact assessment results

Figure A.7. Contribution of individual processes to the total impact of scenario CCD (cover crop co-digestion).

Figure A.8. Contribution of individual processes to the total impact of scenario CMD (cattle manure digestion).

References

Alburquerque, J. A., de la Fuente, C., & Bernal, M. P. (2012). Chemical properties of anaerobic digestates affecting C and N dynamics in amended soils. Agriculture, Ecosystems & Environment, 160, 15–22. https://doi.org/10.1016/J.AGEE.2011.03.007

Borreani, G., & Tabacco, E. (2010). The relationship of silage temperature with the microbiological status of the face of corn silage bunkers. Journal of Dairy Science, 93(6), 2620–2629. https://doi.org/10.3168/JDS.2009-2919

Bruun, S., Christensen, B. T., Hansen, E. M., Magid, J., & Jensen, L. S. (2003). Calibration and validation of the soil organic matter dynamics of the Daisy model with data from the Askov long-term experiments. Soil Biology and Biochemistry, 35(1), 67–76. https://doi.org/10.1016/S0038-0717(02)00237-7

Cavalli, D., Corti, M., Baronchelli, D., Bechini, L., & Marino Gallina, P. (2017). CO2 emissions and mineral nitrogen dynamics following application to soil of undigested liquid cattle manure and digestates. Geoderma, 308, 26–35. https://doi.org/10.1016/J.GEODERMA.2017.08.027

DCE. (2021). Danish emission inventories for agriculture. https://dce2.au.dk/pub/SR443.pdf

de la Fuente, C., Alburquerque, J. A., Clemente, R., & Bernal, M. P. (2013). Soil C and N mineralisation and agricultural value of the products of an anaerobic digestion system. Biology and Fertility of Soils, 49(3), 313–322. https://doi.org/10.1007/S00374-012-0719-9

DMI. 2023. Vejrarkiv. Vejrarkiv (dmi.dk) (Accessed 22-11-2023).

Eriksen, J., Askegaard, M., & Kristensen, K. (1999). Nitrate leaching in an organic dairy/crop rotation as affected by organic manure type, livestock density and crop. Soil Use and Management, (15), 176-182. https://doi.org/10.1111/j.1475-2743.1999.tb00085.x

Eriksen, J., Askegaard, M., & Kristensen, K. (2004). Nitrate leaching from an organic dairy crop rotation: the effects of manure type, nitrogen input and improved crop. Soil Use and Management, 20 (1), 48-54. https://doi.org/10.1111/j.1475-2743.2004.tb00336.x

Eriksen, J., Askegaard, M., Rasmussen, J., Søegaard, K. (2015). Nitrate leaching and residual effect in dairy crop rotations with grass–clover leys as influenced by sward age, grazing, cutting and fertilizer regimes. Agriculture, Ecosystems & Environment, 212, 75-84. https://doi.org/10.1016/j.agee.2015.07.001

Eriksen, J., Askegaard, M., Søegaard, K. (2014). Complementary effects of red clover inclusion in ryegrass-white clover swards for grazing and cutting. Grass and Forage Science, 69 (2), 241-250. https://doi.org/10.1111/gfs.12025

Eriksen, J., Askegaard, M., Søegaard, K. (2008). Residual effect and nitrate leaching in grass-arable rotations: effect of grassland proportion, sward type and fertilizer history. Soil Use and Management, 24 (4), 373-382. https://doi.org/10.1111/j.1475-2743.2008.00178.x

Feng, L., Perschke, Y. M. L., Fontaine, D., Nikolausz, M., Ward, A. J., da Rocha, U. N., Corrêa, F. B., Eriksen, J., Sørensen, P., & Møller, H. B. (2020). Anaerobic digestion of co-ensiled cover crop and barley straw: Effect of co-ensiling ratios, manure addition and impact on microbial community structure. Industrial Crops and Products, 144, 112025. https://doi.org/10.1016/J.INDCROP.2019.112025

Fontaine, D., Eriksen, J., & Sørensen, P. (2020a). Cover crop and cereal straw management influence the residual nitrogen effect. European Journal of Agronomy, 118, 126100. https://doi.org/10.1016/J.EJA.2020.126100

Fontaine, D., Feng, L., Labouriau, R., Møller, H. B., Eriksen, J., & Sørensen, P. (2020b). Nitrogen and Sulfur Availability in Digestates from Anaerobic Co-digestion of Cover Crops, Straw and Cattle Manure. Journal of Soil Science and Plant Nutrition, 20(2), 621–636. https://doi.org/10.1007/S42729-019-00151-7

Fontaine, D., Eriksen, J., & Sørensen, P. (2020). Cover crop and cereal straw management influence the residual nitrogen effect. European Journal of Agronomy, 118, 126100. https://doi.org/10.1016/J.EJA.2020.126100

Fontaine, D., Feng, L., Labouriau, R., Møller, H. B., Eriksen, J., & Sørensen, P. (2020). Nitrogen and Sulfur Availability in Digestates from Anaerobic Co-digestion of Cover Crops, Straw and Cattle Manure. Journal of Soil Science and Plant Nutrition, 20(2), 621–636. https://doi.org/10.1007/S42729-019-00151-7

Fontaine, D., Kristensen, R. K., Rasmussen, J., Eriksen, J. (2022). Data on growth, uptake and N2 fixation of grass-clover leys fertilized with mineral N fertilizer and cattle slurry. Data in Brief, 42, 107998. https://doi.org/10.1016/j.dib.2022.107998

Gavrilova, O., Leip, A., Dong, H., Douglas MacDonald, J., Alfredo Gomez Bravo, C., Amon, B., Barahona Rosales, R., del Prado, A., Aparecida de Lima, M., Oyhantçabal, W., John van der Weerden, T., Widiawati, Y., Bannink, A., Beauchemin, K., & Clark, H. (2019). 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. In Chapter 10: Agriculture, Forestry and Other Land Use (Vol. 4). IPCC.

Hergoualc’h, K., Akiyama, H., Bernoux, M., Chirinda, N., del Prado, A., Kasimir, Å., MacDonald, J. D., Ogle, S. M., Regina, K., & van der Weerden, T. J. (2019). 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. In Chapter 11: N2O Emissions from Managed Soils, and CO2 Emissions from Lime and Urea Application (Vol. 4).

Jensen, J. L., Beucher, A. M., & Eriksen, J. (2022). Soil organic C and N stock changes in grass-clover leys: Effect of grassland proportion and organic fertilizer. Geoderma, 424. https://doi.org/10.1016/j.geoderma.2022.116022

Johansen, A., Carter, M. S., Jensen, E. S., Hauggard-Nielsen, H., & Ambus, P. (2013). Effects of digestate from anaerobically digested cattle slurry and plant materials on soil microbial community and emission of CO2 and N2O. Applied Soil Ecology, 63, 36–44. https://doi.org/10.1016/J.APSOIL.2012.09.003

Kristensen, R. K., Fontaine, D., Rasmussen, J., Eriksen, J. (2022). Contrasting effects of slurry and mineral fertilizer on N2-fixation in grass-clover mixtures. European Journal of Agronomy, 133, 126431. https://doi.org/10.1016/j.eja.2021.126431

Kupper, T., Häni, C., Neftel, A., Kincaid, C., Bühler, M., Amon, B., & VanderZaag, A. (2020). Ammonia and greenhouse gas emissions from slurry storage - A review. Agriculture, Ecosystems & Environment, 300, 106963. https://doi.org/10.1016/J.AGEE.2020.106963

Müller-Stöver, D. S., Sun, G., Kroff, P., Thomsen, S. T., & Hauggaard-Nielsen, H. (2016). Anaerobic co-digestion of perennials: Methane potential and digestate nitrogen fertilizer value. Journal of Plant Nutrition and Soil Science, 179(6), 696–704. https://doi.org/10.1002/JPLN.201500599

Møller, H.B., Romio, C., Rasmussen, M.D., Jensen, L.S., Schwartzkopff, M., Holm-Nielsen, J.B., Agdal, E.B.L., Larsen, S.U., Hinge, J. (2020). HALM TIL BIOGAS – Erfaringer og klimaaspekter. INBIOM – Innovationsnetværket for Bioressourcer.

Olesen, J. E., Møller, H. B., Petersen, S. O., Sørensen, P., Nyord, T., & Sommer, S. G. (2021). Sustainable biogas - climate and environmental effects of biogas production. In DCA report  nr. 181 (p. 85). Aarhus Universitet, DCA – Danish Centre for Food and Agriculture.

Speight, J. G. (2011). Production, properties and environmental impact of hydrocarbon fuel conversion. Advances in Clean Hydrocarbon Fuel Processing: Science and Technology, 54–82. https://doi.org/10.1533/9780857093783.1.54

Strid, I., & Flysjö, A. (2007). Livscykelanalys (LCA) av ensilage.

Sørensen, P. (1998). Effects of storage time and straw content of cattle slurry on the mineralization of nitrogen and carbon in soil. Biology and Fertility of Soils, 27(1), 85–91. https://doi.org/10.1007/S003740050404

Sørensen, P., Weisbjerg, M. R., & Lund, P. (2003). Dietary effects on the composition and plant utilization of nitrogen in dairy cattle manure. The Journal of Agricultural Science, 141(1), 79–91. https://doi.org/10.1017/S0021859603003368

The Engineering ToolBox. (2003). Heat loss from oil filled tanks and pipe lines. The Engineering ToolBox

Viaene, J., Agneessens, L., Capito, C., Ameloot, N., Reubens, B., Willekens, K., Vandecasteele, B., & De Neve, S. (2017). Co-ensiling, co-composting and anaerobic co-digestion of vegetable crop residues: Product stability and effect on soil carbon and nitrogen dynamics. Scientia Horticulturae, 220, 214–225. https://doi.org/10.1016/J.SCIENTA.2017.03.015