## Supplementary information

Figure S1: Depth XPS analysis with atomic concentrations of relevant components. In regions near the surface of as-grown MoS2 (depth < 5 nm), the oxygen and carbon contents are comparingly high, resulting from exposition to air and therefore adsorption of humidity and hydrocarbons. The seemingly sub stoichiometric sulfur-to-molybdenum-ratio may be consequence of preferential sulfur-etching of the applied method.

Figure S2: a) STEM-EDS-maps of H2O 1d and b) of 100°C 1d. c) EDS spectra of the encapsulated MoSx, the H2O 1d and the 100°C 1d, all normalized to the Mo L peak reveals the lowest oxygen content for the encapsulated MoSx. d) Background subtracted EEL spectrum of encapsulated MoSx shows S L edge at the expected 165 eV, whereas mapping of S L for the H2O 1d sample was not possible due to the absence of signal. The EEL spectra of the MoSx layer of the H2O 1d and the encapsulated MoSx did not show any O K edge around 530 eV. This discrepancy between EDS and EELS of the O K edge may indicate that all samples have a very low oxygen content and that the EDS O K edge may originate from spurious signals.