Application of SPI-Guided Analgesia in Laparoscopic Gynecologic Surgery: a Randomized Controlled Trial Evaluating Remifentanil-Sparing Effect and Predictive Value of Time-Weighted SPI

Wei Liu1, Ying Li2, Ran Lin3, KE Min1, Qin Lin1, Wei Lin1, Shu Zheng4, Xiangfeng Wang1

1.Department of Anesthesiology, The Affiliated People's Hospital of Fujian University of Traditional Chinese Medicine, Fuzhou, Fujian 350004, China

Corresponding author:Xiangfeng Wang,yy2022002@fjtcm.edu.cn ORCID: 0000-0001-8994-9228

Detailed Calculation Method for Tb-TW-SPI

S1. Detailed Calculation Method for Tb-TW-SPI

S1.1 Data Preparation

Extract the SPI value sequence (e.g., every 5 minutes) from the anesthesia record system for each patient, from anesthesia induction (T₀) to the end of surgery (T_end): SPI(t₁), SPI(t₂), ..., SPI(t_n).

Data Acquisition and Recording SPI values were continuously monitored using a GE Datex-Ohmeda S/5™ Anesthesia Workstation (GE Healthcare, Helsinki, Finland). A finger-cuff photoplethysmographic probe was placed on the index finger of the non-blood-pressure-monitoring arm. SPI data were automatically logged at 5-minute intervals by the GE S/5 Collect software and exported as comma-separated values (.csv) files for post-hoc analysis.

Computational Software All calculations were performed using R Statistical Software (Version 4.3.0). The following R packages were used: tidyverse (Version 2.0.0): Data import and manipulation pracma (Version 2.4.4): Trapezoidal numerical integration

S1.2 Generation of Excess Sequence

Calculate the value exceeding the threshold (43) at each time point:SPI_excess(ti)=max{SPI(ti)−43,0}. When SPI(t_i) ≤ 43, SPI_excess(ti)=0; When SPI(t_i) > 43,SPI_excess(ti)=SPI(ti)−43.

S1.3 Tb-TW-SPI Calculation Formula

Integrate the SPI_excess sequence using trapezoidal numerical integration to compute the area under the excess threshold curve:

Tb−TW−SPI=i=1n−1[SPIexcess(ti)+SPIexcess(ti+1)]2×(ti+1−ti).

The result is expressed in SPI units × minutes, where SPI is a dimensionless index ranging from 0 to 100,representing the total cumulative load of noxious stimuli exceeding the set threshold throughout the entire surgical period.

S1.4 Calculation Example

Assume a patient's intraoperative SPI recordings are: SPI = 40 at t₁ = 0 min; SPI = 60 at t₂ = 5 min; SPI = 55 at t₃ = 10 min.

The excess-threshold sequence SPI_excess is: 0, 17, 12.

Tb-TW-SPI = (0+17)/2 × 5 + (17+12)/2 × 5 = 42.5 + 72.5 = 115.0 (SPI units·min)

S1.5 Graphical Illustration

Figure S1. Schematic representation of Tb-TW-SPI calculation. The shaded area above the SPI threshold of 43 represents the cumulative nociceptive load, integrated using the trapezoidal rule.