Supplemental Material #1

PROTOCOL: miRNA isolation from HM samples

using miRNeasy mini kit (Cat. No. 217004) and RNeasy MinElute kit

- If samples are frozen, thaw milk sample overnight at 4°C

  - For smaller volumes such as 250ul to 1mL, thaw on ice for 30 minutes prior to extractions

- Turn on centrifuge and set to 4°C.

- Transfer 250 µL milk sample to a fresh 1.5mL tube

  - If sample contains RNAlater (250 uL milk to 500 uL RNAlater), split into 3 250uL aliquots (all 3 will go into one spin column in step 17. )

STEPS BELOW MUST BE CONDUCTED IN CHEMICAL FUME HOOD DUE TO QIAZOL AND CHLOROFORM

- Add 800 µL QIAzol Lysis Reagent

- Vortex

- Incubate at RT for 5 min

- Add 200 μL chloroform

- Vortex

- Incubate at RT for 5 min

- Centrifuge at 13,000×g, 4°C, 15 min

- Set centrifuge to 22°C

- Transfer the upper aqueous phase to a fresh 1.5mL tube

- Add 1 volume of 70% ethanol to upper aqueous phase

- Discard lower layers in HAZARDOUS WASTE

  - Troubleshooting Tips

    - Small pink layer on top of middle clear aqueous layer- vortex and spin for an additional 15 min

    - Complete inversion of layer (clear layer on bottom)- salt concentration too high (likely due to RNAlater) repeat Qiazol and chloroform incubations with 100 ul of Qiazol and 50ul of chloroform and spin for additional 15 min

- Discard tubes in tips container in hood

- Vortex sample thoroughly

- Pipet 700 µL of sample into RNeasy Mini spin column placed in 2mL collection tube

- Centrifuge at 8000 rcf for 15s (if centrifuge does not go this low set for 30s and stop after 15s)

- Pipet the flow-through (contains miRNA) into a new 1.5mL reaction tube

- Repeat steps 17-19 until all of aqueous mixture has been passed though the column. Repeats will be needed for RNAlater-preserved samples

- DO NOT DISCARD SPIN COLUMN. PLACE COLUMNS ON ICE OR STORE AT 4°C. IT WILL BE USED LATER TO EXTRACT TOTAL RNA

miRNA ENRICHED FRACTION EXTRACTION STEPS

- Add 0.65 volumes of 100% ethanol to flow-through from step 19

- Vortex

- Pipet 700 µL of sample into RNeasy MinElute spin column placed in 2mL collection tube

- Centrifuge at 8000 rcf/g for 15s

- Discard flow-through in HAZARDOUS WASTE

- Repeat steps 28-30 as needed. Repeats will be needed for RNAlater preserved samples

- Pipet 500 µL of Buffer RPE into RNeasy MinElute spin column

- Centrifuge at 8000 rcf/g for 15s

- Discard flow-through in normal waste

- Add 500 µL of 80% ethanol into RNeasy MinElute spin column

- Centrifuge at 8000 rcf/g for 2 min

- Remove spin-column from collection tube immediately after centrifugation and place in new dry collection tube

- Open spin-column lid and centrifuge at 8000rcf/g for 5 min

- Place RNeasy MinElute spin column in 1.5ml Eppendorf tube

- Add 14 µL of RNase-free water to spin column and close lid

- Centrifuge at 8000rcf/g for 1 min to elute miRNA-enriched fraction

- Discard column

- Freeze samples at -80°C until provided to core for Fragment Analyzer

TOTAL RNA EXTRACTION

- Pipet 700 µL of Buffer RWT to RNeasy Mini Spin Column from step 21

- Centrifuge at 8000 rcf/g for 15s

- Discard flow-through in HAZARDOUS WASTE

- Add 500ul of Buffer RPE to the RNeasy spin column

- Centrifuge at 8000 rcf/g for 15s

- Discard flow-through in normal waste

- Repeat steps 44-48

- Discard collection tube immediately after centrifugation

- Place spin column in new 2ml collection tube

- Open lid of spin column and centrifuge at full speed for 1 min

- Discard collection tube  in tips container

- Place spin column in new 1.5 ml Eppendorf tube

- Pipet 30 µL of RNase-free water to spin column

- Centrifuge at 8000 rcg/g for 1 min to elute total RNA

- Discard column in tips container

- Check RNA quality with TapeStation and RNA ScreenTape

- Freeze samples at -80°C

- Let waste sit in tips container in hood for minimum of 48hrs to let QIAzol and chloroform evaporate

- After 48hrs discard in large sharps container

| Supplementary Table 1. Summary Statistics for RINe Scores |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Timepoint | Storage Temperature | Treatment |  |  |  |
| IMD | RT |  | 9.07 ¬± 0.4; Median 9.08 (8.86-9.43) |  |  |
| 4hr | 4C | RNAlater | 7.95 ¬± 0.81; Median 8.17 (7.67-8.42) |  |  |
| 24hr | 4C | RNAlater | 6.94 ¬± 1.6; Median 7.23 (6.27-8.13) |  |  |
| 24hr | -20C | RNAlater | 7.56 ¬± 1.09; Median 7.87 (7.22-8.47) |  |  |
| 24hr | -80C | RNAlater | 7.76 ¬± 0.86; Median 7.83 (7.5-8.33) |  |  |
| 1wk | 4C | RNAlater | 6.12 ¬± 1.74; Median 6.62 (5.44-7.12) |  |  |
| 1wk | -20C | RNAlater | 7.63 ¬± 0.82; Median 7.68 (7.34-8.17) |  |  |
| 1wk | -80C | RNAlater | 8.01 ¬± 0.65; Median 8.13 (7.81-8.38) |  |  |
| 4wk | -20C | RNAlater | 7.29 ¬± 1.24; Median 7.17 (6.62-8.33) |  |  |
| 4wk | -80C | RNAlater | 7.7 ¬± 1.01; Median 7.9 (7.33-8.47) |  |  |
| 12wk | -20C | RNAlater | 7.39 ¬± 0.73; Median 7.28 (7.03-7.95) |  |  |
| 12wk | -80C | RNAlater | 7.92 ¬± 0.65; Median 8.05 (7.62-8.27) |  |  |
| 24wk | -20C | RNAlater | 7.1 ¬± 1.32; Median 7.17 (6.27-8.1) |  |  |
| 24wk | -80C | RNALater | 7.99 ¬± 0.81; Median 8.07 (7.7-8.6) |  |  |
| 4hr | 4C | No Treatment | 7.84 ¬± 1.25; Median 8.25 (6.86-8.76) |  |  |
| 24hr | 4C | No Treatment | 5.51 ¬± 1.96; Median 5.6 (4.12-6.78) |  |  |
| 24hr | -20C | No Treatment | 2.88 ¬± 1; Median 3.03 (2.15-3.3) |  |  |
| 24hr | -80C | No Treatment | 2.89 ¬± 0.87; Median 2.95 (2.07-3.47) |  |  |
| 1wk | 4C | No Treatment | 3.4 ¬± 0.81; Median 3.13 (2.92-3.85) |  |  |
| 1wk | -20C | No Treatment | 2.51 ¬± 0.94; Median 2.28 (1.74-3.38) |  |  |
| 1wk | -80C | No Treatment | 2.81 ¬± 1.21; Median 2.52 (1.7-3.73) |  |  |
| 4wk | -20C | No Treatment | 2.51 ¬± 0.83; Median 2.56 (1.9-3) |  |  |
| 4wk | -80C | No Treatment | 2.84 ¬± 0.93; Median 2.77 (2.37-3.67) |  |  |
| 12wk | -20C | No Treatment | 2.78 ¬± 1.21; Median 1.93 (1.7-3.9) |  |  |
| 12wk | -80C | No Treatment | 3.14 ¬± 1.03; Median 3.33 (2.07-3.77) |  |  |
| 24wk | -20C | No Treatment | 2.55 ¬± 1.21; Median 2 (1.47-3.57) |  |  |
| 24wk | -80C | No Treatment | 3.15 ¬± 0.92; Median 3.1 (2.42-3.88) |  |  |

| Supplementary Table 2. Summary Statistics RNA Concentration |  |  |  |
| --- | --- | --- | --- |
| Timepoint | Storage Temperature | Treatment |  |
| IMD | RT |  | 76.99 ¬± 58.15; Median 60.07 (38.67-119.35) |
| 4hr | 4C | RNAlater | 26.9 ¬± 19.87; Median 21.67 (12.52-37.58) |
| 24hr | 4C | RNAlater | 22.33 ¬± 12.64; Median 20.73 (11.08-31.38) |
| 24hr | -20C | RNAlater | 26.67 ¬± 19.56; Median 20.86 (13.7-36.96) |
| 24hr | -80C | RNAlater | 28.84 ¬± 16.09; Median 26.15 (16.42-37.27) |
| 1wk | 4C | RNAlater | 16.35 ¬± 10.46; Median 17.48 (7.64-24.18) |
| 1wk | -20C | RNAlater | 20.12 ¬± 15.17; Median 16.83 (8.39-29.09) |
| 1wk | -80C | RNAlater | 25.69 ¬± 20.47; Median 18.34 (12.86-35.19) |
| 4wk | -20C | RNAlater | 21.97 ¬± 12.06; Median 18.45 (15.16-30.52) |
| 4wk | -80C | RNAlater | 30.66 ¬± 17.77; Median 26.27 (15.43-39.95) |
| 12wk | -20C | RNAlater | 21.92 ¬± 13.97; Median 18.42 (10.39-28.08) |
| 12wk | -80C | RNAlater | 28.69 ¬± 22.62; Median 21.27 (12.16-36.98) |
| 24wk | -20C | RNAlater | 16.17 ¬± 11.52; Median 14.12 (7-23.83) |
| 24wk | -80C | RNAlater | 29.25 ¬± 23.14; Median 20.17 (15.01-34.4) |
| 4hr | 4C | No Treatment | 79.25 ¬± 62; Median 65.32 (38.25-103.49) |
| 24hr | 4C | No Treatment | 70.71 ¬± 49.75; Median 48.23 (36.87-101.65) |
| 24hr | -20C | No Treatment | 80.16 ¬± 58.76; Median 57.57 (40.02-117.47) |
| 24hr | -80C | No Treatment | 86.78 ¬± 61.21; Median 65.82 (50.48-126.47) |
| 1wk | 4C | No Treatment | 42.11 ¬± 30.8; Median 38.63 (23.66-51.86) |
| 1wk | -20C | No Treatment | 66.13 ¬± 56.08; Median 46.51 (26.34-104.67) |
| 1wk | -80C | No Treatment | 71.15 ¬± 56.66; Median 61.6 (32.39-87.03) |
| 4wk | -20C | No Treatment | 74.33 ¬± 47.38; Median 61.33 (45.4-120.67) |
| 4wk | -80C | No Treatment | 76.78 ¬± 55.86; Median 70.93 (44.97-99.47) |
| 12wk | -20C | No Treatment | 73.61 ¬± 61; Median 53.07 (33.65-101.03) |
| 12wk | -80C | No Treatment | 80.11 ¬± 61.45; Median 52.1 (39.77-105.27) |
| 24wk | -20C | No Treatment | 66.38 ¬± 46.31; Median 58.73 (33.93-78.63) |
| 24wk | -80C | No Treatment | 84.11 ¬± 60.4; Median 68.88 (43.19-121.72) |

| Supplementary Table 3. EEF1A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| timepoint | treatment | mean_sd | median_iqr | range_ct | temp | p_value |
| 24wk | RNA | 24.26 ± 1.41 | 24.05 (1.51) | 22.05–28.94 | -20C | 2.91E-08 |
| 24wk | RNA | 24.26 ± 1.41 | 24.05 (1.51) | 22.05–28.94 | -80C | 2.91E-08 |
| 24wk | RNARL | 19.56 ± 0.65 | 19.48 (0.82) | 18.47–21.29 | -20C | 2.91E-08 |
| 24wk | RNARL | 19.56 ± 0.65 | 19.48 (0.82) | 18.47–21.29 | -80C | 2.91E-08 |
| IMD | RNA | 21.08 ± 0.67 | 20.93 (0.61) | 20.09–22.73 | RT | NA |