Probabilistic numerical simulation of morphology from non-isothermal calorimetric crystallization curves

Alíz Lelik1, Lilla Bubenkó1, Medárd Koncz1, János Molnár2, Örs Sepsi3, Alfréd Menyhárd1

1Laboratory of Plastics and Rubber Technology, Department of Physical Chemistry and Materials Science at Budapest University of Technology and Economics, H-1111 Budapest, Műegyetem rkp. 3., Hungary

2Borealis Polyolefine GmbH, St.-Peter-Strasse 25, A-4021 Linz, Austria

3 Semilab Semiconductor Physics Laboratory Co. Ltd., Prielle Kornélia u. 2. H-1117 Budapest, Hungary

Supplementary Information

Determination of first and second derivatives of Hoffmann-Lauritzen equation

The following constants and variables are used in this chapter:

- 𝑇: Temperature [K]

- 𝑇0: starting temperature of cooling run [K]

- 𝑡: time [s]

- β: cooling rate [K/s]

- 𝐺: growth rate of crystal front [m/s]

- 𝐺0, 𝜅𝑔: iterative parameters

- 𝑅: universal gas constant JmolK

- 𝑈∗: activation energy of transport processes [J/mol]

𝑚

𝑇0 : equilibrium melting point [K]

- 𝑇𝑔: glass transition temperature [K]

- 𝑇∞: refence temperature used in WLF equation [K]

- 𝑇1, 𝑇2, 𝑇3: combined temperature constants [K]

- 𝐴1, 𝐴2, 𝐴3, 𝐴4: combined parameters

2

𝑓1(𝑡), 𝑓2(𝑡), 𝑓′(𝑡), 𝑓3(𝑡): combined functions using that 𝑇 = 𝑇0 – β ∙ 𝑡

The Hoffmann-Lauritzen equation is written as:

GT=G0∙e-U*R∙T-Tg+T∞∙eκG∙Tm02∙Tm0+T2∙T2∙Tm0-T	 (s1)

The actual temperature can be given as:

T=T0-β∙t	(s2)

The time dependence of temperature at a constant cooling rate can be implemented in Eq. s1 using Eq. s2. For simplicity we provide here the exponential parts of modified Eq. s1 separately:

-U*R∙1T0-β∙t-Tg+T∞=U*R∙1-T0+β∙t+Tg-T∞	(s3)

If we define T1 as T1=-T0+Tg-T∞, then Eq. s3 can be simplified as:

U*R∙1β∙t+T1	(s4)

The same procedure can be applied on the second exponential part of Eq. s1.

κG∙Tm02∙Tm0+T0-β∙t2∙T0-β∙t2∙Tm0-T0-β∙t	(s5)

If we introduce the following expressions like T2=Tm0+T0 and T3=Tm0-T0 as well as 
A1=κG∙Tm022, the following simplified formula is obtained.

A1∙T2-β∙tT3-β∙t∙T02-2∙T0∙β∙t+β2∙t2	(s6)

Eq. s6 can be simplified further if we introduce the following combined parameters:

A2=T3∙T02

A3=β∙T02-2∙T3∙T0	(s7)

A4=β2∙T3-2∙T0

Using the combined parameters given in Eq. s7, the second exponential part can be written as:

A1∙-β∙t+T2β3∙t3+A4∙t2+A3∙t+A2	(s8)

The H-L equation as a function of time in the case of constant cooling rate can be given now by combining Eq. s4 and s8:

Gt=G0∙eU*R∙1β∙t+T1∙eA1∙-β∙t+T2β3∙t3+A4∙t2+A3∙t+A2	(s9)

Consequently, the first derivative can be written as:

G'(t)=G0∙-β∙A1β3∙t3+A4∙t2+A3∙t+A2-A1∙T2-β∙t∙3∙β3∙t2+2∙A4∙t+A3β3∙t3+A4∙t2+A3∙t+A22-U*R∙ββ∙t+T12∙eA1∙T2-β∙tβ3∙t3+A4∙t2+A3∙t+A2+U*Rβ∙t+T1	(s10)

Eq. s10 can be simplified if the repeating expressions are substituted by the following combined functions:

f1t=T2-β∙t      thus       f10=T2

f2t=β3∙t3+A4∙t2+A3∙t+A2      thus       f20=A2	(s11)

f2't=3∙β3∙t2+2∙A4∙t+A3         thus       f2'0=A3

f3t=β∙t+T1      thus       f30=T1

Accordingly, the final version of the first derivative can be written as:

G't=G0∙-β∙A1f2t-A1∙f1t∙f2'tf2t2-U*R∙βf3t2∙eA1∙f1tf2t+U*Rf3t	(s12)

The second derivative can be written as:

G''t=G0∙-β∙A1f2t-A1∙f1t∙f2'tf2t2-U*R∙βf3t22∙eA1∙f1tf2t+U*Rf3t+G0∙-2∙β∙A1∙f2'tf2t2-A1∙f1t∙6∙β∙t+2∙A4f2t2-2∙U*R∙β2f3t3∙eA1∙f1tf2t+U*Rf3t	(s13)

Accordingly, Eq. s12 and s13 are used in our simulation software in order to describe the growing distance under non-isothermal condition at a constant cooling rate.

The detailed data of all PCs, which were used in the study in order to simulate the crystalline structure of iPP.

Table S1 The characteristics of the computers used in the simulation

|  | Computer 1 | Computer 2 |
| --- | --- | --- |
| CPU | AMD Ryzen 7 5800H with Radeon Graphics, 3.20 GHz | 11th Gen Intel Core i3-1115G4, 3.00GHz |
| RAM | 64,0 GB | 8,0 GB |
| GPU | NVIDIA GeForce RTX 3060 Laptop | Intel® UHD Graphics |
| Storage device | 512 GB SSD | 256 GB SSD |