Appendix

Appendix 1. The results of the study using hourly precipitation data.

Fig. 8 a Scatter plot  b  chi plot c chi plot of first (lower-left) quadrant  d  chi plot of third (upper-right) quadrant of pair of hourly precipitation values of Monza and Lambro SUM.

The calculated value of tail dependence coefficient (0.22) for bivariate distribution of 6-hour precipitation of “Monza” and “Lambro SUM” is regarded as the value of Spearman's rank correlation coefficient (ρ) between depth of riverine and pluvial floodwater.

Table 2. Joint return periods for riverine and pluvial floods in Monza considering different dependence structure between the two hazards

| OR Case: Joe Copula (parameter = 1.30) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Return Period (years) | 10 | 20 | 50 | 100 | 1000 |
| 10 | 5.98 | 7.77 | 9.18 | 9.65 | 9.98 |
| 20 | ---- | 11.82 | 16.35 | 18.31 | 19.91 |
| 50 | ---- | ---- | 29.38 | 38.49 | 49.23 |
| 100 | ---- | ---- | ---- | 58.67 | 96.29 |
| 1000 | ---- | ---- | ---- | ---- | 586.13 |
| OR Case: Flipped Clayton Copula (parameter = 0.34) |  |  |  |  |  |
| 10 | 5.81 | 7.54 | 9.00 | 9.54 | 9.97 |
| 20 | ---- | 11.32 | 15.73 | 17.84 | 19.85 |
| 50 | ---- | ---- | 27.72 | 36.47 | 48.77 |
| 100 1000 | ---- ---- | ---- ---- | ---- ---- | 54.91 ---- | 94.34 540.19 |
| OR Case: Gumbel Copula (parameter = 0.17) |  |  |  |  |  |
| 10 | 5.76 | 7.50 | 8.96 | 9.51 | 9.97 |
| 20 | ---- | 11.28 | 15.72 | 17.82 | 19.83 |
| 50 | ---- | ---- | 27.86 | 36.69 | 48.77 |
| 100 | ---- | ---- | ---- | 55.49 | 94.60 |
| 1000 | ---- | ---- | ---- | ---- | 552.89 |