Natural Hazards

Supplementary Information

Landslide susceptibility mapping using automatically constructed CNN architectures with pre-slide topographic DEM of deep-seated catastrophic landslides caused by Typhoon Talas

Teruyuki Kikuchi1* · Koki Sakita2 · Satoshi Nishiyama3 · Kenichi Takahashi4

1 J-POWER Design Co., Ltd., Geoengineering Department, Sotokanda 2-16-2, Chiyoda-ku, Tokyo 101-0021, Japan

2 Shimizu Corporation Institute of Technology, Echujima 3-4-17, Koto-ku, Tokyo, 135-0044, Japan

3 Graduate School of Environmental and Life Science, Okayama University, Tsushimanaka 3-1-1, Kita-ku, Okayama 160-0004, Japan

4 Electric Power Development Co., Ltd., Ginza 6-15-1, Chuou-ku, Tokyo 104-8165, Japan

*Corresponding author: Teruyuki Kikuchi

Tel: +838050774207

E-mail: kikuchi-t@mbj.nifty.com

Online Resources 1 and 2 show the landslides organized according to latitude, longitude, landslide area, height difference of slide, aspect, slope angle (average), topographic features, and geological formation. To focus on DSGSD, ID numbers are assigned to topographic features and areas in that order.

Online Resource 1 Landslide data (N = 38)

| ID | Latitude | Longitude | Area (m2) | H (m) | Aspect | Slope Angle (°) | Topographic Features | Geologic Formation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 34.1914 | 135.8928 | 7,500 | 112 | NNE | 35.3 | 6 | Obadanigawa |
| 2 | 34.1230 | 135.7478 | 5,100 | 96 | E | 37.1 | 6 | Miyama |
| 3 | 34.0782 | 135.6136 | 48,000 | 56 | N | 45.8 | 4 and 6 | Miyama |
| 4 | 34.0638 | 135.6468 | 28,700 | 176 | WSW | 37.3 | 4 and 6 | Miyama |
| 5 | 34.2248 | 135.8360 | 26,400 | 124 | SSE | 30.8 | 4 and 6 | Yukawa |
| 6 | 34.1448 | 135.7723 | 6,600 | 108 | NE | 45.2 | 4 and 6 | Miyama |
| 7 | 34.1448 | 135.6443 | 35,900 | 176 | SW | 39.3 | 2, 5, and 6 | Yukawa |
| 8 | 34.0556 | 135.6388 | 29,900 | 254 | NE | 32.5 | 4 and 6 | Miyama |
| 9 | 34.1637 | 135.7611 | 9,200 | 128 | E | 34.7 | 2, 3, and 6 | Miyama |
| 10 | 34.1696 | 135.7626 | 9,400 | 92 | SW | 39.3 | 3, 4, and 6 | Miyama |
| 11 | 34.1265 | 135.7375 | 14,000 | 114 | NW | 29.8 | 1, 4, and 6 | Miyama |
| 12 | 34.0663 | 135.7005 | 4,500 | 94 | NNE | 32.7 | 1, 2, 4, and 6 | Miyama |
| 13 | 34.0834 | 135.7651 | 20,600 | 154 | N | 35.9 | 1, 4, and 6 | Miyama |
| 14 | 34.0145 | 135.6930 | 8,500 | 165 | NW | 28.2 | 3, 4, 5, and 6 | Ryujin |
| 15 | 34.1821 | 135.6968 | 42,100 | 174 | N | 31.3 | 2, 4, 5, and 6 | Hanazono |
| 16 | 34.1183 | 135.6765 | 48,100 | 308 | NW | 38.5 | 4, 5, and 6 | Miyama |
| 17 | 34.1358 | 135.5992 | 5,044 | 103 | W | 34.1 | 4, 5, and 6 | Yukawa |
| 18 | 34.0423 | 135.7303 | 14,300 | 154 | NNW | 35.9 | 1, 2, 4, 5, and 6 | Miyama |
| 19 | 34.0307 | 135.7018 | 34,100 | 234 | NE | 40.1 | 1, 3, 4, 5, and 6 | Ryujin |
| 20 | 34.1401 | 135.6660 | 13,500 | 136 | ENE | 37.8 | 1, 2, 3, 5, and 6 | Yukawa |
| 21 | 34.0627 | 135.6828 | 7,400 | 64 | W | 37.7 | 1, 3, 4, 5, and 6 | Miyama |
| 22 | 34.0985 | 135.8048 | 14,700 | 126 | NNW | 36.4 | 2, 3, 4, and 5 | Miyama |
| 23 | 34.0570 | 135.6281 | 42,300 | 218 | WSW | 35.1 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 24 | 34.1204 | 135.7735 | 5,100 | 158 | NW | 38.4 | 2, 3, 5, and 6 | Miyama |
| 25 | 34.1281 | 135.7810 | 48,000 | 226 | WSW | 32.2 | 3, 4, 5, and 6 | Miyama |
| 26 | 34.1937 | 135.8514 | 18,200 | 134 | NNW | 34.4 | 3, 4, and 6 | Hanazono |
| 27 | 34.1367 | 135.7467 | 1,400 | 32 | NE | 40.9 | 2, 3, 4, and 5 | Miyama |
| 28 | 34.0154 | 135.6951 | 51,700 | 227 | NNW | 36.4 | 1, 2, 4, and 6 | Ryujin |
| 29 | 34.0159 | 135.6980 | 22,500 | 92 | NNW | 28.2 | 2, 3, 4, and 6 | Ryujin |
| 30 | 34.2195 | 135.8346 | 77,200 | 124 | ESE | 28.9 | 2, 4, 5, and 6 | Hanazono |
| 31 | 34.2195 | 135.8434 | 38,600 | 120 | NNW | 32.5 | 1, 2, 4, 5, and 6 | Hanazono |
| 32 | 34.1544 | 135.7413 | 59,000 | 254 | ENE | 39.8 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 33 | 34.0414 | 135.7608 | 86,300 | 396 | NNE | 43.5 | 1, 3, 4, 5, and 6 | Miyama |
| 34 | 34.1398 | 135.7698 | 10,300 | 144 | SW | 40.7 | 1, 3, 4, 5, and 6 | Miyama |
| 35 | 34.1256 | 135.7263 | 254,900 | 592 | NW | 35.5 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 36 | 34.1120 | 135.7346 | 19,200 | 120 | NNE | 25.0 | 1, 3, 4, 5, and 6 | Miyama |
| 37 | 34.1294 | 135.7538 | 182,700 | 400 | NW | 37.5 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 38 | 34.1347 | 135.7412 | 195,100 | 446 | NNW | 30.8 | 1, 2, 3, 4, 5, and 6 | Miyama |

H: Elevation difference between the top and the end of the slide area, Angle Strike: maximum slope direction, Slope angle: angle between the top and end of the slide area (°). Topographic features, 1: Main scarp, 2: Minor scarp, 3: Frank, 4: Terminal Cliff, 5: Gully, 6: Irregular Undulation.

Online Resource 2 DSGSD data (N = 63)

| ID | Latitude | Longitude | A (m2) | H (m) | Aspect | Slope Angle (°) | Topographic Features | Geologic Formation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 34.0300 | 135.6469 | 58,800 | 236 | NNW | 36.0 | 4 and 6 | Ryujin |
| 40 | 34.2161 | 135.8354 | 59,100 | 164 | E | 31.6 | 1, 4, and 6 | Obadanigawa |
| 41 | 34.1382 | 135.6999 | 29,700 | 209 | E | 39.3 | 1, 4, and 5 | Miyama |
| 42 | 34.0513 | 135.7320 | 155,300 | 322 | NNW | 33.7 | 4, 5, and 6 | Miyama |
| 43 | 34.0957 | 135.7881 | 126,200 | 249 | NNE | 38.4 | 1, 3, 4, 5, and 6 | Miyama |
| 44 | 34.0517 | 135.7498 | 76,100 | 323 | NNW | 34.9 | 1, 4, and 6 | Miyama |
| 45 | 34.0535 | 135.7545 | 12,900 | 139 | N | 32.2 | 1, 4, and 6 | Miyama |
| 46 | 34.1417 | 135.6787 | 254,900 | 592 | W | 37.3 | 1, 3, and 4 | Yukawa |
| 47 | 34.1446 | 135.7085 | 13,900 | 91 | NNW | 33.9 | 1, 4, and 5 | Miyama |
| 48 | 34.1379 | 135.7694 | 44,600 | 230 | NW | 40.0 | 3, 5, and 6 | Miyama |
| 49 | 34.1130 | 135.6796 | 39,400 | 241 | SSW | 39.9 | 1, 4, and 5 | Miyama |
| 50 | 34.0392 | 135.6473 | 178,000 | 315 | NNE | 38.8 | 1, 2, 3, and 6 | Miyama |
| 51 | 34.1852 | 135.7018 | 9,200 | 128 | NW | 37.8 | 1, 3, 4, and 6 | Hanazono |
| 52 | 34.1451 | 135.6863 | 48,100 | 308 | N | 33.8 | 1, 2, 3, and 4 | Yukawa |
| 53 | 34.1377 | 135.6860 | 21,900 | 128 | NE | 36.7 | 1, 2, 4, and 5 | Miyama |
| 54 | 34.1591 | 135.6983 | 48,000 | 226 | WSW | 37.5 | 1, 2, 3, and 4 | Miyama |
| 55 | 34.1266 | 135.7321 | 18,500 | 182 | SSW | 36.3 | 1 and 3 | Miyama |
| 56 | 34.0967 | 135.7821 | 250,600 | 251 | NNW | 38.2 | 1, 2, 3, 4, and 6 | Miyama |
| 57 | 34.1588 | 135.6880 | 28,000 | 136 | NE | 37.2 | 1, 3, 4, and 5 | Yukawa |
| 58 | 34.1543 | 135.6868 | 10,300 | 144 | SW | 38.1 | 1, 3, 4, 5, and 6 | Yukawa |
| 59 | 34.1488 | 135.6838 | 6,600 | 108 | N | 37.1 | 1, 4, 5, and 6 | Yukawa |
| 60 | 34.1392 | 135.6685 | 39,400 | 173 | NE | 36.7 | 1, 3, 4, 5, and 6 | Yukawa |
| 61 | 34.1603 | 135.7133 | 5,100 | 158 | W | 33.2 | 1, 2, 5, and 6 | Miyama |
| 62 | 34.1309 | 135.7351 | 204,800 | 443 | WNW | 37.3 | 1, 4, 5, and 6 | Miyama |
| 63 | 34.0311 | 135.6526 | 113,100 | 424 | NNW | 37.8 | 1, 2, 3, 4, 5, and 6 | Ryujin |
| 64 | 34.0344 | 135.7223 | 8,500 | 165 | NNW | 39.3 | 1, 2, 3, 4, 5, and 6 | Ryujin |
| 65 | 34.1844 | 135.7050 | 80,600 | 124 | NNE | 36.9 | 1, 3, 4, 5, and 6 | Hanazono |
| 66 | 34.1617 | 135.7550 | 9,400 | 92 | WNW | 35.2 | 1, 4, 5, and 6 | Hanazono |
| 67 | 34.1341 | 135.6446 | 195,100 | 446 | WNW | 39.1 | 2, 3, 4, 5, and 6 | Yukawa |
| 68 | 34.1448 | 135.6816 | 25,400 | 139 | N | 34.0 | 1, 3, 4, 5, and 6 | Yukawa |
| 69 | 34.1590 | 135.7121 | 5,100 | 96 | WNW | 35.1 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 70 | 34.1528 | 135.7768 | 282,800 | 449 | SSE | 37.1 | 1, 4, 5, and 6 | Miyama |
| 71 | 34.1929 | 135.8534 | 93,400 | 278 | NW | 36.9 | 1, 3, 4, 5, and 6 | Hanazono |
| 72 | 34.1792 | 135.7094 | 121,100 | 258 | NNW | 35.9 | 1, 2, 3, 4, 5, and 6 | Hanazono |
| 73 | 34.1398 | 135.6768 | 14,000 | 114 | W | 37.2 | 1, 3, 4, 5, and 6 | Yukawa |
| 74 | 34.1525 | 135.7151 | 93,900 | 315 | SW | 39.0 | 1, 3, 4, 5, and 6 | Miyama |
| 75 | 34.1498 | 135.7153 | 19,200 | 120 | WNW | 36.6 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 76 | 34.1563 | 135.7152 | 182,700 | 400 | SW | 39.2 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 77 | 34.1132 | 135.7434 | 27,500 | 185 | NW | 36.8 | 1, 3, 4, 6 | Miyama |
| 78 | 34.0408 | 135.7263 | 57,200 | 247 | NNW | 35.8 | 1, 3, 4, 5, and 6 | Miyama |
| 79 | 34.1763 | 135.7025 | 78,500 | 195 | N | 36.2 | 1, 3, 4, 5, and 6 | Hanazono |
| 80 | 34.1097 | 135.7868 | 251,200 | 460 | NNE | 37.0 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 81 | 34.2225 | 135.8278 | 7,500 | 181 | SW | 38.5 | 1, 2, 3, 4, 5, and 6 | Hanazono |
| 82 | 34.0985 | 135.7997 | 14,500 | 151 | NNE | 32.0 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 83 | 34.0310 | 135.6535 | 14,300 | 154 | NW | 39.2 | 1, 2, 3, 4, 5, and 6 | Ryujin |
| 84 | 34.1537 | 135.6826 | 1,400 | 32 | S | 37.8 | 1, 2, 3, 4, 5, and 6 | Yukawa |
| 85 | 34.1416 | 135.6950 | 187,200 | 207 | SSE | 38.3 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 86 | 34.1553 | 135.7650 | 120,000 | 520 | SSE | 37.0 | 1, 3, 4, 5, and 6 | Miyama |
| 87 | 34.1375 | 135.7978 | 43,500 | 154 | SSE | 37.6 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 88 | 34.1571 | 135.7848 | 171,700 | 344 | WSW | 35.4 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 89 | 34.1530 | 135.7895 | 185,700 | 431 | SE | 37.6 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 90 | 34.1431 | 135.7114 | 157,400 | 198 | NE | 34.9 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 91 | 34.1143 | 135.7076 | 193,600 | 382 | ENE | 40.7 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 92 | 34.1651 | 135.7432 | 13,500 | 136 | SE | 33.7 | 1, 3, 4, 5, and 6 | Miyama |
| 93 | 34.0710 | 135.7122 | 24,000 | 188 | WSW | 32.7 | 1, 3, 4, 5, and 6 | Miyama |
| 94 | 34.0270 | 135.6374 | 124,300 | 146 | NE | 41.0 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 95 | 34.1643 | 135.7407 | 413,700 | 435 | SE | 30.4 | 1, 2, 3, 4, 5, and 6 | Miyama |
| 96 | 34.1410 | 135.7143 | 19,700 | 148 | NE | 40.3 | 1, 3, 4, 5, and 6 | Miyama |
| 97 | 34.0294 | 135.7239 | 51,700 | 227 | NNW | 35.9 | 1, 2, 3, 4, 5, and 6 | Ryujin |
| 98 | 34.0890 | 135.7631 | 39,500 | 94 | NNW | 34.0 | 1, 3, 4, 5, and 6 | Miyama |
| 99 | 34.0141 | 135.6871 | 77,200 | 124 | ENE | 42.3 | 1, 2, 3, 4, 5, and 6 | Ryujin |
| 100 | 34.0211 | 135.6855 | 38,600 | 120 | E | 39.0 | 1, 2, 3, 4, 5, and 6 | Ryujin |
| 101 | 34.1270 | 135.6779 | 51,200 | 196 | NE | 40.6 | 1, 2, 3, 4, 5, and 6 | Yukawa |

H: Elevation difference between the top and the end of the slide area, Angle Strike: maximum slope direction, slope angle: angle between the top and end of the slide area (°). Topographic features, 1: Main scarp, 2: Minor scarp, 3: Frank, 4: Terminal Cliff, 5: Gully, 6: Irregular Undulation.