Supplementary Information

Influence of gaseous oxygen species on liquid-phase, fixed nitrogen products in aqueous plasma-based electrochemical processes

Brandon Kamiyama, Mohammad Ali Eslamisaray, Emily Gillmore, and R. Mohan Sankaran*

Department of Nuclear, Plasma, and Radiological Engineering, The Grainger College

of Engineering, University of Illinois Urbana-Champaign, Champaign, IL, USA

*Corresponding email: rmohan@illinois.edu

Nitrogen product fraction in main and trap solutions calculation

The molar fraction of nitrogen products in the main solution and trap solution were calculated as the following:

Molar fraction of species i in main solution =Mi,mainMmain+Mtrap

where Mi,main is the molar concentration of NH4+, NO3-, and NO2-, respectively, in the main solution, and Mmain and Mtrapare the molar concentrations of all nitrogen products in the main and trap solutions, respectively.

Molar fraction of species i in trap solution =Mi,trap Mmain+Mtrap

where Mi,trap is the molar concentration of NH4+, NO3-, and NO2-, respectively, in the trap solution, and Mmain and Mtrapare the molar concentrations of all nitrogen products in the main and trap solutions, respectively.

Selectivity calculation

The molar selectivity of NH4+ with respect to NOx- was defined as the following:

SNH4+/NOx- =MNH4+MNO3-+MNO2-

where MNH4+ is the molar concentration of NH4+, MNO3- is the molar concentration of NO3-, and MNO2- is the molar concentration of NO2-.

Table S1. Summary of NH4+, NO3-, and NO2- concentrations measured in main and trap solutions of DC plasma electrochemical process as a function of oxygen content in mixed gas feed of UHP N2 and air.

O2 concentration ~ 0% (UHP N2)

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 4.84 | 0.21 | 0.16 | 13.16 | 0.48 | 0.01 | 0.00 |
| Trial 2 | 5.31 | 0.17 | 0.17 | 15.52 | 0.90 | 0.00 | 0.00 |
| Trial 3 | 5.06 | 0.25 | 0.15 | 12.73 | 0.76 | 0.00 | 0.00 |
| Average | 5.07±0.20 | 0.21±0.03 | 0.16±0.01 | 13.74±1.30 | 0.71±0.17 | 0.00±0.00 | 0.00±0.00 |

O2 concentration = 0.1%

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 3.36 | 0.83 | 0.36 | 2.83 | 0.33 | 0.01 | 0.00 |
| Trial 2 | 3.34 | 1.27 | 0.20 | 2.27 | 0.72 | 0.01 | 0.00 |
| Trial 3 | 2.91 | 0.71 | 0.42 | 2.58 | 0.42 | 0.01 | 0.00 |
| Average | 3.20±0.21 | 0.94±0.24 | 0.32±0.09 | 2.54±0.55 | 0.49±0.17 | 0.01±0.00 | 0.00±0.00 |

O2 concentration = 0.2%

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 2.88 | 1.34 | 0.25 | 1.81 | 0.32 | 0.01 | 0.01 |
| Trial 2 | 3.28 | 0.30 | 0.31 | 5.43 | 0.57 | 0.00 | 0.00 |
| Trial 3 | 2.06 | 1.29 | 0.26 | 1.32 | 0.35 | 0.01 | 0.02 |
| Average | 2.74±0.51 | 0.98±0.48 | 0.27±0.02 | 2.19±0.94 | 0.41±0.11 | 0.01±0.00 | 0.01±0.01 |

O2 concentration = 0.4%

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 2.16 | 0.94 | 0.44 | 1.56 | 0.29 | 0.02 | 0.02 |
| Trial 2 | 2.57 | 0.57 | 0.34 | 2.81 | 0.42 | 0.02 | 0.01 |
| Trial 3 | 1.88 | 0.25 | 0.54 | 2.39 | 0.23 | 0.04 | 0.01 |
| Average | 2.20±0.28 | 0.59±0.28 | 0.44±0.08 | 2.15±0.68 | 0.31±0.08 | 0.02±0.01 | 0.01±0.00 |

O2 concentration = 1.0%

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 1.86 | 0.95 | 0.30 | 1.49 | 0.06 | 0.02 | 0.02 |
| Trial 2 | 1.14 | 1.61 | 0.40 | 0.57 | 0.04 | 0.14 | 0.04 |
| Trial 3 | 1.60 | 1.11 | 0.36 | 1.09 | 0.07 | 0.06 | 0.04 |
| Average | 1.53±0.30 | 1.23±0.28 | 0.35±0.04 | 0.97±0.26 | 0.06±0.01 | 0.07±0.05 | 0.03±0.01 |

O2 concentration = 2.1%

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 0.46 | 3.20 | 0.36 | 0.13 | 0.01 | 0.59 | 0.06 |
| Trial 2 | 0.50 | 2.58 | 0.37 | 0.17 | 0.00 | 0.63 | 0.06 |
| Trial 3 | 0.55 | 3.53 | 0.48 | 0.14 | 0.01 | 0.76 | 0.06 |
| Average | 0.50±0.03 | 3.11±0.39 | 0.40±0.05 | 0.14±0.02 | 0.01±0.01 | 0.66±0.07 | 0.06±0.00 |

O2 concentration = 10.5%

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 0.00 | 5.94 | 0.47 | 0.00 | 0.00 | 2.10 | 0.06 |
| Trial 2 | 0.00 | 5.60 | 0.51 | 0.00 | 0.00 | 2.13 | 0.07 |
| Trial 3 | 0.00 | 5.34 | 0.38 | 0.00 | 0.00 | 2.72 | 0.08 |
| Average | 0.00±0.00 | 5.63±0.24 | 0.45±0.06 | 0.00±0.00 | 0.00±0.00 | 2.31±0.29 | 0.07±0.01 |

O2 concentration = 21% (Air)

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) | NH4+/NOx- ratio in main | NH4+ in trap (mM) | NO3- in trap (mM) | NO2- in trap (mM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trial 1 | 0.00 | 9.91 | 0.59 | 0.00 | 0.00 | 5.54 | 0.08 |
| Trial 2 | 0.00 | 9.50 | 0.74 | 0.00 | 0.00 | 5.80 | 0.08 |
| Trial 3 | 0.00 | 9.19 | 0.82 | 0.00 | 0.00 | 3.50 | 0.06 |
| Average | 0.00±0.00 | 9.54±0.29 | 0.72±0.10 | 0.00±0.00 | 0.00±0.00 | 4.95±1.03 | 0.07±0.01 |

Table S2a. Summary of NH4+, NO3-, and NO2- concentrations measured in main solution of DC plasma electrochemical process as a function of total flow rate of UHP N2 gas feed.

UHP N2 Flow = 10 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 6.81 | 0.10 | 0.16 |
| Trial 2 | 5.30 | 0.23 | 0.20 |
| Trial 3 | 5.34 | 0.19 | 0.36 |
| Trial 4 | 5.39 | 0.23 | 0.42 |
| Average | 5.71±0.64 | 0.19±0.14 | 0.28±0.10 |

UHP N2 Flow = 100 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 4.84 | 0.21 | 0.16 |
| Trial 2 | 5.31 | 0.17 | 0.17 |
| Trial 3 | 5.06 | 0.25 | 0.15 |
| Average | 5.07±0.20 | 0.21±0.03 | 0.16±0.01 |

UHP N2 Flow = 200 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 1.58 | 0.31 | 0.23 |
| Trial 2 | 1.43 | 0.59 | 0.31 |
| Trial 3 | 1.97 | 0.25 | 0.32 |
| Average | 1.66±0.23 | 0.38±0.16 | 0.29±0.04 |

UHP N2 Flow = 500 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 1.01 | 0.60 | 0.24 |
| Trial 2 | 1.14 | 0.33 | 0.23 |
| Trial 3 | 0.72 | 0.63 | 0.19 |
| Average | 0.96±0.18 | 0.87±0.06 | 0.22±0.02 |

Table S2b. Summary of NH4+, NO3-, and NO2- concentrations measured in main solution of DC plasma electrochemical process as a function of total flow rate of air gas feed.

Air Flow = 10 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 0.12 | 18.07 | 1.91 |
| Trial 2 | 0.01 | 19.08 | 2.21 |
| Trial 3 | 0.14 | 15.09 | 2.06 |
| Average | 0.09±0.06 | 17.41±1.70 | 2.06±0.12 |

Air Flow = 100 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 0.00 | 9.91 | 0.59 |
| Trial 2 | 0.00 | 9.50 | 0.74 |
| Trial 3 | 0.00 | 9.19 | 0.82 |
| Average | 0.00±0.00 | 9.54±0.29 | 0.72±0.10 |

Air Flow = 200 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 0.00 | 3.92 | 0.30 |
| Trial 2 | 0.00 | 3.64 | 0.21 |
| Trial 3 | 0.02 | 3.11 | 0.04 |
| Average | 0.01±0.01 | 3.56±0.34 | 0.18±0.11 |

Air Flow = 500 sccm

|  | NH4+ in main (mM) | NO3- in main (mM) | NO2- in main (mM) |
| --- | --- | --- | --- |
| Trial 1 | 0.00 | 1.87 | 0.00 |
| Trial 2 | 0.01 | 1.97 | 0.00 |
| Trial 3 | 0.01 | 1.54 | 0.00 |
| Average | 0.01±0.00 | 1.79±0.18 | 0.00±0.00 |

Fig. S1. Ratios of intensities of two OH emission lines within the A2Σ+ X2Π band to the Ar 750.39 nm line as a function of total flow rate in plasma-liquid system. The gas feed consisted of 95% UHP N2 and 5% Ar. The shading represents one standard deviation based on three independent measurements.