Supplementary Material 2

Detailed procedures of extraction and quantification of phytohormones, proline, betaines and carbohydrates

Extraction and quantification of phytohormones:

Extraction was repeated twice and combined extracts were evaporated under nitrogen stream. The residue was re-dissolved in 1ml of 5% MeOH in 1 M HCOOH and cleaned-up on SPE cartridges (BondElutPlexa PCX, 30 mg, 1mm, Agilent, USA). The fraction of neutral and acidic hormones (ABA, auxins, GAs, JA, SA) was eluted with methanol : acetonitrile (MeOH:CAN 1:1 v/v) and the basic compounds (CKs) were eluted with 5% ammonia in MeOH:CAN (1:1 v/v). Fractions were combined, then evaporated to dryness under N2, reconstituted in 100 µl of CAN, filtered (0,22 µm nylon membrane) and used for UHPLC analyses. The system consisted of UHPLC (Agilent Infinity 1260, Agilent, Germany) and triple quadruple mass spectrometer (MS/MS Agilent 6410, Agilent, USA) with electro-spray ionization (ESI). Separation was achieved on AscentisExpress RP-Amide analytical column (2.7 µm, 2.1 mm x 75mm; Supelco, USA) at linear gradient of water vs. CAN both with 0.01 % of HCOOH. Quantitation was based on calibration curves obtained with pure standards of all compounds taking account of the recovery rates of ISTDs.

Extraction and determination of 1-aminocyclopropanecarboxylic acid (ACC):

Lyophilized samples spiked with D-ACC were extracted and cleaned up like described above for other phytohormones. UHPLC separation of 100% CAN dissolved samples was performed by hydrophilic interaction liquid chromatography (HILIC) on an Agilent 1260 Infinity system coupled to MS/MS (Agilent 6410). Separation was achieved on a AsecentisExpress HILIC (3.0×100 mm, 2,7µm, Supelco, USA) at 35 oC. The mobile phase was A) 5 mM ammonium formate in H2O and B) CAN with 1% 5mM ammonium formate. Gradient elution at flow rate of 0,5 ml/min was used. Initial conditions were 100 % B, then in 4 min 50% B, then to 4,5 min 25% B (flushing column) and to 5,5  min return to 100 % B. The injection volume was 1 µl. An 6410 triple quadruple mass spectrometer equipped with electrospray ionization (ESI) in the positive ionization mode was used. The following conditions were found to be optimal for analysis: capillary voltage 3 kV, gas temperature 350 ◦C, drying gas flow 12 l/min and nebulizer pressure of 35 psi. Samples were analyzed by dynamic multiple reactions monitoring (MRM). Both amino acids formed [M+29]+ adduct ions. The ion transition at m/z 130→56 was used for ACC wears transition at m/z 134→60 was used for D-ACC. For both transitions fragmentor 60 V and collision energy 10 V were used. Mass hunter software was used to control the LC–MS/MS system and for data analysis.

Extraction and determination of betaines and free proline:

Betaine (glicyne betaine, Gly-bet), stachydrine (proline betaine Pro-bet) and free proline were estimated in the same samples simultaneously with ACC, and D-ACC was used as internal standard. Betaines and proline formed [M+H]+ adduct ions. The ion transition at m/z 118→58 (fragmentor 100 V and collision energy 25 V) was used for Gly-bet, the transition at m/z 144→84 (fragmentor 120 V and collision energy 21 V)was used for Pro-bet, whereas the transition at m/z 116→70 (fragmentor 60 V and collision energy 9 V) was used for proline quantitation. Mass hunter software was used to control the LC–MS/MS system and for data analysis. Quantification was done based on calibration curves taking account recovery of ISTD.

Extraction and determination of carbohydrates:

About 5 mg of lyophilized and homogenized samples was extracted in 1ml of ultra-pure water (Option R, Elga, UK) by shaking for 15 min at 30 Hz (MM 400, Retsch, Germany). Then the samples were centrifuged for 5 min at 21000 x g (Universal 32R, Hettich, Germany). Supernatant was collected, diluted with acetonitrile 1:1 (v/v), filtered (0.22 µm nylon membrane, Costar Spin-X, Corning, USA), and analyzed on HPLC for soluble sugar content. High performance liquid chromatography was performed using an Agilent 1200 chromatograph equipped with a degasser, binary pump, automated liquid sampler and thermostated column component (Agilent, Germany) and ESA Coulochem II electrochemical detector with 5040 Analytical Cell (ESA, USA) with analogue-to-digital converter. Separation of soluble sugars (glucose, fructose, sucrose and fructans) and starch hydrolyzate (glucose) was achieved on RCX-10; 7 µm; 250 x 4.1 mm column (Hamilton, USA), at a flow rate of 1.5 ml min-1 in gradient mode. Mobile phase A: 75 mM aqueous NaOH solution, B: 500 mM sodium acetate in 75 mM aqueous NaOH solution, 0% A (0-3.5 min), 0-35% B (3.5-19 min), then 0% B (19-20 min). Injection volume was 0.01 cm3, column temperature 40ºC; integration time 20 min. Pulse amperometric detection was employed (analytical potential 200 mV; oxidizing potential 700 mV, reducing potential – 900 mV with reference to a palladium electrode) on a gold electrode. The measurements for each treatment were taken in 3 replicates.