Impact of storage time on characteristics of synthetic greywater with two different pollutant strengths to be treated or recycled

International Journal of Environmental Science and Technology

S. N. Abed1	• S. A. Almuktar1, 2	• M. Scholz1, 3, 4

Corresponding author:

E‒mail address: miklas.scholz@tvrl.lth.se (M. Scholz)

Telephone: +46(0)462228920

Fax: +46(0)462224435

1 Civil Engineering Research Group, School of Computing, Science and Engineering, The University of Salford, Newton Building, Salford M5 4WT, England, United Kingdom

2 Department of Architectural Engineering, Faculty of Engineering, The University of Basrah, Al‒Basrah, Iraq

3 Division of Water Resources Engineering, Department of Building and Environmental Technology, Faculty of Engineering, Lund University, P.O. Box 118, 221 00 Lund, Sweden

4 Department of Civil Engineering Science, School of Civil Engineering and the Built Environment, University of Johannesburg, Kingsway Campus, PO Box 524, Aukland Park 2006, Johannesburg, South Africa

Online Resource 2 Physiochemical properties of fresh synthetic greywater

| Parameter | Unit | Low Concentration |  |  |  |  | High Concentration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n | Mean | SD | Min | Max | n | Mean | SD | Min | Max |
| pH | ‒ | 81 | 6.9 | 0.48 | 5.3 | 7.9 | 81 | 8.4 | 1.61 | 5.4 | 11.5 |
| Redox potential | mV | 81 | 34.1 | 21.23 | -18.1 | 111.2 | 81 | -36.6 | 74.22 | -182.1 | 97.9 |
| Turbidity | NTU | 81 | 22.9 | 7.14 | 9.8 | 41.6 | 81 | 188.9 | 47.22 | 18.3 | 308.0 |
| Total suspended solids | mg/L | 81 | 39.9 | 15.94 | 10.0 | 87.0 | 81 | 317.0 | 58.35 | 173.0 | 473.0 |
| Electrical conductivity | µS/cm | 81 | 164.6 | 63.24 | 98.7 | 452.0 | 81 | 988.5 | 196.09 | 612.0 | 1677.0 |
| Dissolved oxygen | mg/L | 81 | 10.4 | 1.24 | 7.7 | 12.3 | 81 | 10.5 | 1.39 | 6.9 | 12.6 |
| Colour | Pa/Co | 81 | 214.5 | 64.07 | 26.0 | 340.0 | 81 | 1587.8 | 379.89 | 787.0 | 2499.0 |
| Temperature | °C | 81 | 17.7 | 4.58 | 6.7 | 27.0 | 81 | 16.9 | 5.40 | 6.5 | 27.8 |
| Biochemical oxygen demand | mg/L | 81 | 17.6 | 8.00 | 2.0 | 40.0 | 81 | 34.7 | 12.99 | 10.0 | 60.0 |
| Chemical oxygen demand | mg/L | 81 | 28.9 | 14.47 | 8.2 | 86.7 | 81 | 129.2 | 34.68 | 63.9 | 221.0 |
| Ammonia‒nitrogen | mg/L | 81 | 0.2 | 0.22 | 0.0 | 1.1 | 81 | 0.4 | 0.19 | 0.1 | 1.1 |
| Nitrate‒nitrogen | mg/L | 81 | 1.3 | 1.21 | 0.1 | 7.6 | 81 | 8.9 | 6.38 | 0.2 | 29.8 |
| Ortho‒phosphate‒phosphorus | mg/L | 81 | 8.4 | 4.36 | 3.3 | 27.4 | 81 | 59.1 | 14.16 | 30.6 | 94.2 |
| Element |  |  |  |  |  |  |  |  |  |  |  |
| Aluminium | mg/L | 45 | 0.52 | 0.528 | 0.09 | 1.56 | 45 | 2.13 | 0.869 | 0.76 | 4.77 |
| Boron | mg/L | 33 | 0.14 | 0.067 | 0.10 | 0.36 | 33 | 0.57 | 0.068 | 0.49 | 0.72 |
| Calcium | mg/L | 55 | 10.54 | 0.853 | 8.51 | 11.81 | 45 | 36.08 | 8.750 | 22.38 | 50.72 |
| Cadmium | mg/L | 42 | 0.09 | 0.056 | 0.04 | 0.23 | 39 | 7.36 | 2.981 | 3.67 | 11.58 |
| Chromium | mg/L | 54 | 0.04 | 0.063 | 0.01 | 0.23 | 54 | 3.20 | 0.918 | 1.67 | 5.70 |
| Copper | mg/L | 63 | 0.16 | 0.058 | 0.10 | 0.30 | 63 | 1.44 | 0.435 | 0.95 | 2.03 |
| Iron | mg/L | 48 | 0.21 | 0.102 | 0.11 | 0.43 | 48 | 6.41 | 2.476 | 2.48 | 10.37 |
| Potassium | mg/L | 12 | 4.04 | 0.448 | 3.41 | 4.65 | 12 | 60.16 | 1.684 | 58.31 | 62.53 |
| Magnesium | mg/L | 48 | 1.45 | 0.191 | 1.19 | 1.78 | 48 | 17.16 | 2.119 | 12.68 | 20.33 |
| Manganese | mg/L | 63 | 0.17 | 0.084 | 0.07 | 0.26 | 63 | 0.98 | 0.257 | 0.64 | 1.29 |
| Sodium | mg/L | 12 | 14.32 | 1.662 | 12.01 | 16.00 | 12 | 62.68 | 14.538 | 45.50 | 76.92 |
| Nickel | mg/L | 51 | 0.04 | 0.065 | 0.01 | 0.18 | 51 | 0.05 | 0.065 | 0.01 | 0.20 |
| Zinc | mg/L | 39 | 0.21 | 0.159 | 0.01 | 0.39 | 39 | 4.25 | 1.500 | 2.22 | 6.34 |

n number of tested samples, SD standard deviation, Min minimum value, Max maximum value, NTU nephelometric turbidity unit