Impact of storage time on characteristics of synthetic greywater with two different pollutant strengths to be treated or recycled

International Journal of Environmental Science and Technology

S. N. Abed1 • S. A. Almuktar1, 2 • M. Scholz1, 3, 4

Corresponding author:

E‒mail address: miklas.scholz@tvrl.lth.se (M. Scholz)

Telephone: +46(0)462228920

Fax: +46(0)462224435

1 Civil Engineering Research Group, School of Computing, Science and Engineering, The University of Salford, Newton Building, Salford M5 4WT, England, United Kingdom

2 Department of Architectural Engineering, Faculty of Engineering, The University of Basrah, Al‒Basrah, Iraq

3 Division of Water Resources Engineering, Department of Building and Environmental Technology, Faculty of Engineering, Lund University, P.O. Box 118, 221 00 Lund, Sweden

4 Department of Civil Engineering Science, School of Civil Engineering and the Built Environment, University of Johannesburg, Kingsway Campus, PO Box 524, Aukland Park 2006, Johannesburg, South Africa

Online Resource 1 Chemical recipes for low and high concentration strengths of synthetic greywater

| Item | Chemical name | Chemical formula | Molar mass (g/mol) | Low concentration (mg/L) | High concentration (mg/L) | Composition percentages (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Kaolin | Al2Si2O5(OH)4 | 258.16 | 15 | 100 | Al (20.90%), H (1.56%), O (55.78%) and Si (21.76%) |
| 2 | Cellulose | (C6H10O5) n | 162.14 | 15 | 100 | C (44.45%), H (6.22%) and O (49.34%) |
| 3 | Humic acid | C187H186O89N9S1 | 4015.55 | 5 | 20 | C (55.90%), H (4.67%), O (35.46%), N (4.67%) and S (0.80%) |
| 4 | Sodium chloride | NaCl | 58.44 | 10 | 120 | Cl (60.66%) and Na (39.34%) |
| 5 | Sodium hydrogen carbonate | NaHCO3 | 84.01 | 10 | 85 | C (14.30%), H (1.20%), Na (27.37%) and O (57.14%) |
| 6 | Calcium chloride | CaCl2 | 147.02 | 10 | 55 | Ca (36.11%) and Cl (63.89%) |
| 7 | Potassium nitrate | KNO3 | 101.10 | 0 | 90 | K (38.67%), N (13.85%) and O (47.48%) |
| 8 | Calcium nitrate | Ca(NO3)2 | 164.09 | 0 | 150 | Ca (24.43%), N (17.07%) and O (58.50%) |
| 9 | Magnesium sulphate | MgSO4 | 120.37 | 2 | 240 | Mg (20.19%), S (26.64%) and O (53.17%) |
| 10 | Monopotassium phosphate | KH2PO4 | 136.09 | 13 | 85 | H (1.48%), K (28.73%), O (47.03%) and P (22.76%) |
| 11 | Iron(III) chloride | FeCl3 | 162.20 | 0.3 | 50.0 | Fe (34.43%) and Cl (65.57%) |
| 12 | Boric acid | H3BO3 | 61.83 | 0.6 | 3.0 | H (4.89%), B (17.48%) and O (77.63%) |
| 13 | Manganese(II) chloride | MnCl2 | 125.84 | 0.03 | 3.20 | Cl (56.34%) and Mn (43.66%) |
| 14 | Zinc sulphate | ZnSO4 | 161.44 | 0.25 | 15.00 | O (39.64%), S (19.86%) and Zn (40.50%) |
| 15 | Copper sulphate | CuSO4 | 159.61 | 0.025 | 7.00 | Cu (39.81%), O (40.10%) and S (20.09%) |
| 16 | Ammonium molybdate tetrahydrate | (NH4)6Mo7O24 | 1163.94 | 0.35 | 0.35 | H (2.08%), Mo (57.71%), N (7.22%) and O (32.99%) |
| 17 | Cadmium oxide | CdO | 128.41 | 0.02 | 12.50 | Cd (87.54%) and O (12.46%) |
| 18 | Nickel oxide | NiO | 74.69 | 0.02 | 0.06 | Ni (78.58%) and O (21.42%) |
| 19 | Chromium(III) nitrate | CrN3O9 | 99.99 | 0.045 | 70.00 | Cr (21.85%), N (17.65%) and O (60.50%) |
| 20 | Sodium sulphate | Na2SO4 | 142.04 | 2.60 | 25.00 | Na (32.37%), O (45.06%) and S (22.57%) |
| 21 | Sodium phosphate monobasic | H2NaPO4 | 119.98 | 0.00 | 250.00 | H (1.68%), Na (19.16%), O (53.34%) and P (25.82%) |
| 22 | Lead(II) oxide | Pb3O4 | 685.60 | 0.16 | 1.40 | Pb (90.67%) and O (9.33%) |
| 23 | Secondary treatment effluent with microbial content (mL/L) | – | – | 20.00 | 100.00 | – |

Al aluminium, H hydrogen, O oxygen, Si silicon, C carbon, N nitrogen, S sulphur, Cl chlorine, Na sodium, Ca calcium, K potassium, Mg magnesium, P phosphorus, Fe iron, B boron, Mn manganese, Zn zinc, Cu copper, Mo molybdenum, Cd Cadmium, Ni nickel, Cr chromium, Pb lead. Note that item 23 was not considered in this study